| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Marginal Island Width of NTMs in NSTX |  |  |  |
| OP-XP-739 | Revision: REVISION | Effective Date: 5/2/07 Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: R.J. La Haye, E.J. Strait |  |  | Date |
| ATI – ET Group Leader: S.A. Sabbagh |  |  | Date |
| RLM - Run Coordinator: D. Gates |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

TITLE: Marginal Island Width of NTMs in NSTX	No.  OP-XP-739

AUTHOR: R.J. La Haye, E.J. Strait	DATE:  5/2/07

# 1.	Overview of planned experiment

Small island stabilizing effects (transport, polarization current, etc.) set the marginal beta above which neoclassical tearing modes (NTMs) can be excited. The marginal full island width wmarg is the value of w at which growth, dw/dt, is largest. This island size plays an important role in determining the electron cyclotron current drive (ECCD) requirements for stabilization in ITER. In “high” aspect ratio devices with R/a»3 (ASDEX-Upgrade, DIII-D and JET), the marginal island width is found to be about twice the ion banana width, 2e1/2ρqi. The purpose of this experiment, in NSTX, is to investigate if this scaling occurs at reduced aspect ratio.

# 2.	Theoretical/ empirical justification

An NTM can be excited when there is a region of parameter space with dw/dt>0 between w=wcrit dw/dtº0 and w=wsat dw/dtº0. In general dw/dt is a maximum at wmarg. When sufficiently seeded, wseed³wcrit, the island grows to the saturated size wsat. If beta is then slowly reduced, the island shrinks as the destabilizing bootstrap current jbsµβq is reduced. When w~wmarg, dw/dt~0 for all w and stabilization occurs. An example from DIII-D is shown in Fig. 1.

Figure 1   NTM island width vs. poloidal beta in DIII-D and theoretical island growth rate vs. island width

Data from ASDEX Upgrade, DIII-D and JET is plotted in Fig. 2; this is the normalized electron beta poloidal, the dominant destabilizing perturbed bootstrap term, versus the normalized poloidal ion gyroradius at q=32 for the spontaneous stabilization of the mn=32 NTM. This is equivalent to a scaling of wmarg»2e1/2ρqi. However, e12 is essentially the same in all cases. While wmarg»2e1/2ρqi is most consistent with the polarization threshold, this is not a proof. The empirical scaling predicts ITER will be very susceptible to destabilization, as also shown in Fig. 2.

Figure 2   Normalized electron poloidal beta vs. normalized poloidal ion gyroradius at q = 1.5 for several devices

NSTX, with a much larger aR has a larger e12 and can extend tests of small island threshold physics. With Ra»54, q(0)»1, and q95=6.8, one expects e12 to be 3/5412»1.6 times larger in NSTX.

3.	Experimental run plan

To best add to the existing database, an m/n=2/1 NTM needs to be excited and allowed to saturate. Desirable is q95=6.8 and q0»1 so that the ρ value of q=2 will be similar to large aspect ratio devices for the q=3/2 cases already run. This should follow XP740 in which such modes are excited and piggyback on that to develop beta ramp down after excitation. Once excited and saturated at near constant heating power, the power should be lowered in steps to get beta to decrease “slowly”. Steps could be -40% every 3tE for about 4 steps taking βq down to »13 of the saturated value. H-mode with ELMs should be retained. One could first just try turning off selected beams in series and adjust further if necessary with modulation pre-programmed. A IP&BT scan with beta ramp down is desirable, keeping Ip/BT fixed, if time permits. The reference discharge is shape of 123487. An ELMy H-mode, ELMs or n=1 kinks excite the m/n=2/1 mode; the method of excitation is not an issue.  This will previously have been developed in XP740.

This is a ½ day experiment.

XP739 Shot Plan:

1. Establish the plasma with a reproducible m/n=2/1 mode that saturates as beta stops rising using results of XP740 and step down beams sequentially (2.0 to 1.5 to 1.0 to 0.5 assuming only 2 beams are available with 10msec on, 10msec off to get a 50% beam.) Does n=1  stabilize before H to L? If not, some development is needed to try to stay in H mode.  Increase kappa and/or drsep.                                                                        (4 shots)

2.	 Repeat with heating power “ramped” down more gradually before the mode stabilization in 1. by using adjusted modulation of beams that are stepped down. Stay in ELMing H-mode and look for the decrease in mode amplitude (by n=1 Mirnov) until spontaneous stabilization occurs. Check that even if no H to L transition, the n=1 mode does not lock. If desired result is obtained, stagger the 2 Thomson lasers from the usual 16 msec spacing to fire at 180 degrees difference in phase of rotating mode; for example 1 kHz would be 0.5 msec apart every 32 msec.

(4 shots)

3.	 Repeat 2 with different IP&BT at fixed q95 to vary the plasma conditions for stabilization. A reduction of both in proportion to 0.75MA and 3.4T would be a modest mini-scan. Desired is a variation in rhoitheta during the ramp down.

(4 shots)

4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Toroidal Field: BT=4.5 kG

for q95=6.8

Plasma current:  Ip=1.0 MA

Shape:  NSTX 123487

Beams: Enough power to raise beta high enough to excite the m/n=2/1 mode. 2 needed but 3 is desirable.

Essential Diagnostics :          Magnetics (fast and slow), RWM detectors

CHERS for core Ti and rotation

Thomson

SXR for island width

5.	Planned analysis

EFIT is needed to locate q=2 and determine Bq. Ti profile measurement is then needed to get ρqi. Lacking ECE radiometer, wmarg will have to be computed from magnetics using calibration by SXR inversion if possible.  Data for the marginal point, as shown as C in Fig. 1, for wmarg, ρqi/rs, βq and βpers/Lp will be added to the existing date set in Fig. 2.  CHERS has high radial resolution and Thomson bursts may also help get the island width by “flat spots”.

Note that 2/1 experiments are planned in DIII-D, AUG and JET so that the 2/1 NSTX data fits there. ECCD beta removal experiments for the m/n=2/1 mode in DIII-D suggest the same wmarg scaling holds but no database at high aspect ratio was gathered due to going out of H-mode before the 2/1 mode stabilized. Higher q95 is expected to resolve this so the q95~6.8 in NSTX would be matched.

6.	Planned publication of results

Results could be published as a letter in Nuclear Fusion to follow up previous publication of results in multiple high aspect ratio devices.

PHYSICS OPERATIONS REQUEST

Title	OP-XP-739

Machine conditions (specify ranges as appropriate) SAME AS 740 WHICH PRECEDES THIS. LI EVAPORATOR REQUIRED. HE GLOW BETWEEN SHOTS.

ITF (kA): 4.5 kG field	Flattop start/stop (s):

IP (MA): 0.7 – 1.1 MA	Flattop start/stop (s):

Configuration: see shot 123487

Outer gap (m): 		Inner gap (m):

Elongation :		Triangularity :

Z position (m):

Gas Species: D	Injector(s):

NBI - Species: D	Sources: A,B (C optional)Voltage (kV): 90	Duration (s):

ICRF – Power (MW): 	Phasing: 	Duration (s):

CHI:

Either:	List previous shot numbers for setup: 123487, or suitable shot from XP740, use shot 123184 for n = 3 braking if these waveforms not available in setup shot.

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

Title	OP-XP- 739

| Diagnostic | Need | Desire | Instructions |
| --- | --- | --- | --- |
| Bolometer – tangential array |  | x |  |
| Bolometer – divertor |  | x |  |
| CHERS – toroidal | x |  |  |
| CHERS – poloidal |  | x |  |
| Divertor fast camera |  | x |  |
| Dust detector |  | x |  |
| EBW radiometers |  | x |  |
| Edge deposition monitors |  | x |  |
| Edge pressure gauges |  | x |  |
| Edge rotation diagnostic |  | x |  |
| Fast ion D_alpha - FIDA |  | x |  |
| Fast lost ion probes - IFLIP |  | x |  |
| Fast lost ion probes - SFLIP |  | x |  |
| Filterscopes | x |  |  |
| FIReTIP |  | x |  |
| Gas puff imaging |  | x |  |
| H camera - 1D |  | x |  |
| High-k scattering |  | x |  |
| Infrared cameras |  | x |  |
| Interferometer - 1 mm |  | x |  |
| Langmuir probes - divertor |  | x |  |
| Langmuir probes – RF antenna |  | x |  |
| Magnetics – Diamagnetism | x |  |  |
| Magnetics - Flux loops | x |  |  |
| Magnetics - Locked modes | x |  |  |
| Magnetics - Pickup coils | x |  |  |
| Magnetics - Rogowski coils | x |  |  |
| Magnetics - RWM sensors | x |  |  |
| Mirnov coils – high frequency | x |  |  |
| Mirnov coils – poloidal array | x |  |  |
| Mirnov coils – toroidal array | x |  |  |
| MSE | x |  |  |
| NPA – ExB scanning |  | x |  |
| NPA – solid state |  | x |  |
| Neutron measurements |  | x |  |
| Plasma TV |  | x |  |
| Reciprocating probe |  | x |  |
| Reflectometer – 65GHz |  | x |  |
| Reflectometer – correlation |  | x |  |
| Reflectometer – FM/CW |  | x |  |
| Reflectometer – fixed f |  | x |  |
| Reflectometer – SOL |  | x |  |
| RF edge probes |  | x |  |
| Spectrometer – SPRED |  | x |  |
| Spectrometer – VIPS |  | x |  |
| SWIFT – 2D flow |  | x |  |
| Thomson scattering | x |  |  |
| Ultrasoft X-ray arrays | x |  |  |
| Ultrasoft X-ray arrays – bicolor |  | x |  |
| Ultrasoft X-rays – TG spectr. |  | x |  |
| Visible bremsstrahlung det. |  | x |  |
| X-ray crystal spectrometer - H |  | x |  |
| X-ray crystal spectrometer - V |  | x |  |
| X-ray fast pinhole camera |  | x |  |