| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Edge Electrode Biasing for SOL Control |  |  |  |
| OP-XP-744 | Revision: v. 3 | Effective Date: 5/31/07 Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: S. Zweben |  |  | Date 531/07 |
| ATI – ET Group Leader: V. Soukhanovskii |  |  | Date |
| RLM - Run Coordinator: D. Gates |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

TITLE:  Edge Electrode Biasing for SOL Control	No.  OP-XP-744

AUTHOR:  S. Zweben, C. Bush, R. Maqueda, L. Roquemore	DATE:  5/25/07

# 1.	Overview of planned experiment

This XP will use the newly installed edge electrodes to test the theory of Cohen and Ryutov concerning the effect of a local poloidal electric field on the SOL transport.  The electrode hardware has been successfully tested with XMP-051 (completed May 10, 2007).   This XP will extend these test results to higher electrode voltages and currents, will vary the plasma conditions from the single case tried in XMP-051, will try a positive biased electrode, and will (if there is any extra time available) try electrode biasing in an Ohmic H-mode.

# 2.	Theoretical/ empirical justification

The main justification for this XP is to test one of the very few existing theoretical ideas for modifying the SOL in magnetically confined plasmas.  If successful, this could lead to methods to control and reduce the peak particle and heat flux in any magnetic fusion device.

The theory and its previous application to the MAST experiment has been described in: R.H. Cohen et al, “Plasma Convection Induced by Toroidal Asymmetries of the Divertor Plates and Gas Puffing”, Nucl. Fusion 37 (1997) 621 ;  D.D. Ryutov et al, “On the Possibility of Inducing Strong Plasma Convection in the Divertor of MAST”,  Plasma Phys. Cont. Fusion 43 (2001) 1388 ;  G.F. Counsell et al, “Reduction of Divertor Power Loading in MAST”, 30th EPS Conference (2003) paper P-3.202 ; and R.H. Cohen et al, “Current and Potential Distribtion in a Divertor with Toroidally-Asymmetric Biasing of the Divertor Plate”, Plasma Phys. Cont. Fusion 49 (2007) 1.  The basic idea is very simple:  the application of a local poloidal (DC) electric field will cause a local Vr = Epol x B drift in the radial direction, which can in principle move and/or broaden the SOL.  The electric field needed to do this in NSTX should be quite low (~ 10 V/cm) since the B field is quite low.  This field level can be created by the small electrode set installed in the shadow of the RF antenna for this run.  The main physics question is how far the potential induced by the electrodes penetrates along and across the B field.  Some success was obtained by using a (somewhat differently designed) electrode set in MAST.  A secondary physics question is to determine whether the application of this local bias affects the edge turbulence locally, which could cause additional changes in the SOL width.

# 3.	Experimental run plan

The run plan is based on repeating the plasma condition used in XMP-051 (Checkout of Biased Probe and Electrodes), e.g. shot #124062 (B= 4.5 kG, I=0.8 MA, Ohmic, outer gap slowly reduced from ~ 10 cm to ~ 1 cm over the duration of the discharge).  This condition had the proper B field line angle for connecting the biased electrodes to the GPI diagnostic.

shot 1:  set electrode #1 and #2 voltages to -100 V / -50 volts (compared with -70 volts/ -35 volts used previously in XMP-051), and increase the density with extra gas puffing to <ne>~ 4x1013 cm-3.  Electrodes #3 is at zero volts and electrode #4 is hard grounded.

shot 2:  repeat shot #1 with electrodes #1 and 2 to -50 V/ -100 V to change sign of local B. Increase density further if possible.  Keep electrodes #3 and at zero volts and #4 hard grounded.

shot 3:  if a clear effect of electrode bias is seen in shot #1 or #2, as determined by the GPI or probe measurements, then repeat that shot.

shot 4:  using the same shot above, keeping electrodes #1 and 2 at -50 V / -100 V, now set electrode #3 at +10 volts (if hardware allows), to see effect of electron collection in electrode.

shot 5:  same as shot 4, but increase electrode #3 voltage to +70 volts in electron saturation.

shot 6:   if either shots #5 or #6 make a clear change in the local SOL, as determined by the GPI or probe measurements, then repeat that shot.

shot 7:  change to B=3.5 kG, I=620 kA to see effect of increased Vr due to lower B;  Ohmic plasma, moderate density, same outer gap as part (1).  Set electrode #1 and #2 voltages to -100 V / -50 volts. Set electrode #3 as guided by shots 5,6, and electrode #4 is hard grounded.

shot 8:  using the same B=3.5 kG shot above, switch electrodes #1 and 2 to  -50 V/ -100 V to change sign of local B.  Keep electrodes #3 and 4 at zero volts and hard grounded.

only if extra time is available:

shots 9-12:  repeat shot #117260 to create Ohmic H-mode, but with outer gap slowly reduced from ~ 5 cm to ~ 1 cm, and try various electrode configurations (as above) to modify the SOL.

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Ohmic plasmas, no NBI or RF or CHI

Necessary Diagnostics:  Langmuir probes mounted in the biased electrode holder (Bush)                                                          	GPI with gas puff (Maqueda), fast camera to view electrode region from the other side of the 	machine (Roquemore).

5.	Planned analysis

The effect of the electrodes on the local SOL can be determined by the GPI and by the 	Langmuir probes mounted in the biased electrode holder.  If there is a fast radial convection 	induced by the electrode bias, the GPI gas cloud should move outward radially (as seen in the 	MAST experiment).  The turbulence radial speed can also be measured by the GPI images.  	The edge turbulence fluctuation levels, correlation lengths, and autocorrelation times between 	the electrodes can be measured by the local probes.

The results will be directly compared with the analytical models mentioned above.  	Comparisons with the XGC code of CS Chang will be attempted.  Comparison with the MAST 	results will also be made.

# 6.	Planned publication of results

The results could be published as a full-length paper in Phys. Plasmas, NF, or PPCF.

PHYSICS OPERATIONS REQUEST

Title:  Edge Electrode Biasing for SOL Control	OP-XP-744

Machine conditions (specify ranges as appropriate)

ITF (kA):  -53,  -42		Flattop start/stop (s):

IP (MA): 0.8, 0.6	Flattop start/stop (s):

Configuration:  Shots #1-6 exactly same as #124062 , except for higher density

Shots #7-8 same as #1-6 but with ITF=-42kA, Ip=620 kA

Shots #9-12:  exactly like #117260, but with outer gap to 1 cm     Outer gap (m): down to ~ 1 cm	Inner gap (m):

Elongation :			Triangularity :

Z position (m):

Gas Species: D	Injector(s):

NBI – off

ICRF – off

CHI – off

Shot numbers for setup:   124062 (shots #1-6); scale down Ip, ITF or use 121356 with modified outer gap (shots #7, 8);  117260 (#9-12)

DIAGNOSTIC CHECKLIST

OP-XP-744

| Diagnostic | Need | Desire | Instructions |
| --- | --- | --- | --- |
| Bolometer – tangential array |  | X |  |
| Bolometer – divertor |  | X |  |
| CHERS – toroidal |  |  |  |
| CHERS – poloidal |  |  |  |
| Divertor fast camera |  | X |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitors |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation diagnostic |  | X |  |
| Fast ion D_alpha - FIDA |  | X |  |
| Fast lost ion probes - IFLIP |  |  |  |
| Fast lost ion probes - SFLIP |  |  |  |
| Filterscopes |  | X |  |
| FIReTIP |  | X |  |
| Gas puff imaging | X |  |  |
| Ha camera - 1D |  | X |  |
| High-k scattering |  |  |  |
| Infrared cameras |  | X |  |
| Interferometer - 1 mm |  | X |  |
| Langmuir probes - divertor |  | X |  |
| Langmuir probes – RF antenna |  | X |  |
| Magnetics – Diamagnetism |  | X |  |
| Magnetics - Flux loops |  | X |  |
| Magnetics - Locked modes |  | X |  |
| Magnetics - Pickup coils |  | X |  |
| Magnetics - Rogowski coils |  | X |  |
| Magnetics - RWM sensors |  | X |  |
| Mirnov coils – high frequency |  | X |  |
| Mirnov coils – poloidal array |  | X |  |
| Mirnov coils – toroidal array |  | X |  |
| MSE |  |  |  |
| NPA – ExB scanning |  |  |  |
| NPA – solid state |  |  |  |
| Neutron measurements |  |  |  |
| Plasma TV |  | X |  |
| Reciprocating probe |  | X |  |
| Reflectometer – 65GHz |  | X |  |
| Reflectometer – correlation |  | X |  |
| Reflectometer – FM/CW |  | X |  |
| Reflectometer – fixed f |  | X |  |
| Reflectometer – SOL |  | X |  |
| RF edge probes |  | X |  |
| Spectrometer – SPRED |  | X |  |
| Spectrometer – VIPS |  | X |  |
| SWIFT – 2D flow |  | X |  |
| Thomson scattering | X |  |  |
| Ultrasoft X-ray arrays |  | X |  |
| Ultrasoft X-ray arrays – bicolor |  | X |  |
| Ultrasoft X-rays – TG spectr. |  | X |  |
| Visible bremsstrahlung det. |  | X |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray fast pinhole camera |  |  |  |