| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: ELM characterization in NSTX |  |  |  |
| OP-XP-746 | Revision: | Effective Date: Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: R. Maqueda |  |  | Date 06/04/07 |
| ATI – ET Group Leader: V. Soukhanovskii |  |  | Date |
| RLM - Run Coordinator: D. Gates |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

TITLE:  ELM characterization in NSTX	No.  OP-XP-746

AUTHOR: R. Maqueda, et al.	DATE: 06/04/07

# 1.	Overview of planned experiment

Edge Localized Modes (ELMs) are present during routine operation of NSTX with 3 different types observed: Type I, Type III and Type V.  These modes are born at the H-mode pedestal, grow through linear and non-linear phases, evolve into filamentary structures and, finally, propagate both poloidally and radially in the SOL until they reach the plasma facing components.  The ELMs are also characterized by a short period of increased turbulent and blob activity following the initial mode growth.  The goal of this XP is to measure all phases of the ELM, as briefly described above, for all 3 types of ELMs.  All relevant diagnostics available in NSTX will be used in this effort.

# 2.	Theoretical/ empirical justification

The ELMs present during H-mode operation of tokamak devices evolve into filamentary structures that propagate radially out in the SOL and result on short-time, localized deposition of energy and particles on plasma facing components.  This is an ITER-relevant concern and is, consequently, of great interest in the community (see Refs. 1-9 and references within).  NSTX diagnostics, including visible fast-imaging, are very well suited to substantially contribute in this subject.

This experiment will reproduce all 3 ELM types routinely seen in NSTX and diagnose their evolution, from the initial growth phase, through the non-linear coalescence into filaments, and into filament propagation in the SOL.

In addition, the ELM is followed by a short period of increased turbulent and blob activity.  A potential candidate for the origin of this activity is the perturbed field lines associated with the ELM and a redution (or loss) of the electric field well,10 a characteristic of the H-mode.  This leads to additional particles and energy losses that in the MAST device have been measured to account for 50%-70% of the ELM losses.  (The filaments themselves account for the rest.)  This transient increase in turbulence will be also studied.

For these purposes the following diagnostics will be used:

- Visible fast-imaging (wide angle, gas puff imaging and divertor).

- USXR arrays with different filters and thicknesses.

- FIReTIP chords.

- Fast scanning probe.

- Divertor probes, filterscopes and 1-D cameras.

- 30-point Thomson scattering (for stability analysis).

- UCLA reflectometer, possibly ORNL’s edge reflectometer.

- Fast Mirnov arrays.

_____________

1.	W. Fundamenski, et al., Plasma Phys. Control. Fusion 46 (2004) 233.

2.	J. Boedo, et al., Phys. Plasmas 12 (2005) #072516.

3.	B. Kurzan, et al., Phys. Rev. Lett. (2005) #145001.

4.	A. Kirk, et al., Phys. Rev .Lett. 96 (2006) #185001.

5.	G. Antar, et al., Phys. Plasmas 13 (2006) #082503.

6.	A. Kirk, et al., Plasma Phys. Control. Fusion 48 (2006) B433.

7.	J. Terry, et al., J. Nuclear Mater. 363-365 (2007) 994.

8.	R. Maqueda, et al., J. Nuclear Mater. 363-365 (2007) 1000.

9.	B. Kock, et al., J. Nuclear Mater. 363-365 (2007) 1056

10.	M. Wade, et al., Phys. Rev. Lett. 94 (2005) #225001.

# 3.	Experimental run plan

This XP will reproduce shots taken in the very recent past (post-Liter) and repeat them to allow diagnostic settings to be changed:

1)	Type I ELM: Reproduce shot 124364 (5 shots).

2)	Type III ELM: Reproduce shot 124374 (5 shots).

3)	Type V ELM: Reproduce shot 124635 (5 shots).

Total: 15 shots.

Note: In case that the desired ELMs are not obtained, multiple variations can be tried: fuelling, beam power, elongation, use of SPAs, etc.  Options will be consulted with machine operators and colleagues having intimate knowledge of the options available.

For each of the 5 shots reproduced for each ELM Type the filters on the USXR arrays will be changed as follows:

| Shot | Hup | Hdown |
| --- | --- | --- |
| 1 | Be 100m | Be 10m |
| 2 | Be 10m | Be 10m |
| 3 | Be 5m | Be 10m |
| 4 | Be 5m | Be 5m |
| 5 | Pinhole | Be 5m |

The fast scanning probe will be inserted to increasing depths during these 5 shots until problems are encountered and then fixed “one step back” for the rest of the shots.  Probe timing will be kept fixed for each particular shot (i.e., ELM Type) to match the timing of the GPI puff (see below).

The Gas Puff Imaging D2 puff will be triggered at the following times:

Type I ELMs, shot 124365: 350 ms.

Type III ELMs, shot 124373: 290 ms.

Type IV ELMs, shot 124635: 300 ms

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

All 3 sources of NBI.  See diagnostics table for diagnostics required.

# 5.	Planned analysis

This XP will gather data from multiple diagnostics to characterize the mode structure of the ELM, its evolution into edge/SOL filaments and the propagation of such filaments in the SOL.

EFIT/LRDFIT needed.

Data will also be provided for stability evaluation.

# 6.	Planned publication of results

If successful, results will be submitted to appropriate journal.

PHYSICS OPERATIONS REQUEST

Title: ELM characterization in NSTX	OP-XP-746

Machine conditions (specify ranges as appropriate)

ITF (kA): 4.5 kG	Flattop start/stop (s): *

IP (MA): 0.8 MA	Flattop start/stop (s): *

Configuration:

Outer gap (m): *		Inner gap (m): *

Elongation : *		Triangularity : *

Z position (m): *

Gas Species: D2	Injector(s): HFS

NBI - Species: D	Sources: all	Voltage (kV): 2 MW ea	Duration (s): *

ICRF – Power (MW): no	Phasing: n.a.	Duration (s): n.a.

CHI: no

Either:	List previous shot numbers for setup: (*) 124364, 124374, 124635

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

Title: ELM characterization in NSTX	OP-XP-746

| Diagnostic | Need | Desire | Instructions |
| --- | --- | --- | --- |
| Bolometer – tangential array |  | X |  |
| Bolometer – divertor |  | X |  |
| CHERS – toroidal |  | X |  |
| CHERS – poloidal |  | X |  |
| Divertor fast camera |  | X |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitors |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation diagnostic | X |  |  |
| Fast ion D_alpha - FIDA |  | X |  |
| Fast lost ion probes - IFLIP |  |  |  |
| Fast lost ion probes - SFLIP |  |  |  |
| Filterscopes | X |  |  |
| FIReTIP | X |  |  |
| Gas puff imaging | X |  |  |
| H camera - 1D |  | X |  |
| High-k scattering |  |  |  |
| Infrared cameras |  | X |  |
| Interferometer - 1 mm |  |  |  |
| Langmuir probes - divertor | X |  |  |
| Langmuir probes – RF antenna |  |  |  |
| Magnetics – Diamagnetism | X |  |  |
| Magnetics - Flux loops | X |  |  |
| Magnetics - Locked modes | X |  |  |
| Magnetics - Pickup coils | X |  |  |
| Magnetics - Rogowski coils | X |  |  |
| Magnetics - RWM sensors |  | X |  |
| Mirnov coils – high frequency | X |  |  |
| Mirnov coils – poloidal array | X |  |  |
| Mirnov coils – toroidal array | X |  |  |
| MSE |  | X |  |
| NPA – ExB scanning |  |  |  |
| NPA – solid state |  |  |  |
| Neutron measurements |  |  |  |
| Plasma TV | X |  | With Phantom 7 camera |
| Reciprocating probe | X |  |  |
| Reflectometer – 65GHz | X |  |  |
| Reflectometer – correlation |  | X |  |
| Reflectometer – FM/CW |  |  |  |
| Reflectometer – fixed f |  |  |  |
| Reflectometer – SOL |  | X |  |
| RF edge probes |  |  |  |
| Spectrometer – SPRED |  | X |  |
| Spectrometer – VIPS |  | X |  |
| SWIFT – 2D flow |  |  |  |
| Thomson scattering | X |  |  |
| Ultrasoft X-ray arrays | X |  |  |
| Ultrasoft X-ray arrays – bicolor |  |  |  |
| Ultrasoft X-rays – TG spectr. |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray fast pinhole camera |  |  |  |