| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Joint NSTX/DIII-D poloidal rotation experiment |  |  |  |
| OP-XP-1041 | Revision: | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: Ronald Bell |  |  | Date |
| ATI – ET Group Leader: Howard Yuh |  |  | Date |
| RLM - Run Coordinator: Eric Fredrickson |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| RESTRICTIONS or MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Joint NSTX/DIII-D poloidal rotation experiment | No. OP-XP-1041 |
| --- | --- |
| AUTHORS: R. E. Bell, K. Burrell, M. Podestá, W. Solomon | DATE: 6/9/10 |

# 1.	Overview of planned experiment

The goal of this experiment is to investigate the carbon poloidal rotation in matched NSTX and D III-D plasmas to determine whether the differences between the measured poloidal rotation and the neoclassical predictions are affected by aspect ratio.

# 2.	Theoretical/ empirical justification

Measured poloidal velocities on various machines is inconsistent with respect to expectations for poloidal velocity from neoclassical theory.

DIII-D:  QH-mode plasmas had sign and magnitude of measured poloidal velocity differing from neoclassical theory.

NSTX: H-mode plasmas with measured poloidal velocity at or below neoclassical expectation.

JET: Measured poloidal velocity in ITB an order of magnitude larger than neoclassical.

JT-60:  Measured poloidal flow is consistent with neoclassical in ITB.

MAST:  Measured poloidal flow is consistent with neoclassical in L and H mode plasmas.

A comparison between DIII-D and NSTX for similar H-mode at low magnetic field would provide a unique comparison at distinctly different aspect ratios. Comparisons with modest ion temperature and low magnetic field would eliminate a potentially serious systematic error due to gyro orbit and finite lifetime effects.

# 3.	Experimental run plan.

This joint NSTX/DIII-D experiment uses target plasmas established in 2006 to match plasma shape. Poloidal velocity measurements were available on DIII-D, but not NSTX for these original plasmas. The current experiment seeks to reproduce these target plasmas to document the poloidal velocity. The first half of the experiment was carried out at D III-D in January 2010. The goal of the second half of the experiment on NSTX is to reproduce the original target discharge and then to adjust the parameters to more closely match the D III-D discharges obtained in 2010. The more recent D III-D discharges used slightly higher Bt and lower Ip than the 2006 target. The toroidal rotation on NSTX for the target discharges was 3-4 times higher than toroidal rotation on D III-D. A reduction in toroidal velocity using magnetic braking will be done to try to obtain a better match.

The NSTX run plan:

1. Establish the DN target plasma, using reference shot 120200, matching shape, Ip=0.73 MA, Bt=0.495 T.

2. Increase Bt from 0.495 T to 0.55 T to match D III-D discharges.

3. Decrease Ip from 0.73 MA to 0.6-0.64 MA range. Steps 2 and 3 can be done simultaneously.

4. Add n=3 error field in steps to reduce toroidal velocity to about 50-60  km/s. This should require EFC currents from 0, 400, 800 kA.

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Poloidal and toroidal CHERS are required. It would be highly desirable to document the turbulence in these plasmas, since turbulence is offered as a reason for the disparity between measured and neoclassical values of poloidal velocity. The pitch angle (Ip/Bt = 1.1 MA/T) for these plasmas is far from optimal for the new BES diagnostic, which is aligned with field lines when Ip/Bt ~ 2 MA/T. Even with the non-optimal viewing geometry, BES is desired. High-k scattering is also desired; it should be configured with a radial location around 140 cm radius. The UCLA interferometer is also desired. See the Diagnostic checklist for other diagnostic needs.

# 5.	Planned analysis

EFIT, TRANSP, NCLASS, GTC-Neo, NEO

# 6.	Planned publication of results

TBD

PHYSICS OPERATIONS REQUEST

| TITLE: Joint NSTX/DIII-D poloidal rotation experiment | No. OP-XP-1041 |
| --- | --- |
| AUTHORS: R. E. Bell, K. Burrell, M. Podestá, W. Solomon | DATE: 6/9/10 |

(use additional sheets and attach waveform diagrams if necessary)

Brief description of the most important operational plasma conditions required: (1) Reproduce shot 120200 discharge parameters and shape. (2) Modify Bt (increase) and Ip (decrease) to more closely match DIII-D target plasma. (3) Add n=3 EFC in steps to reduce toroidal velocity to match DIII-D target.

Previous shot(s) which can be repeated: 120200 Previous shot(s) which can be modified: 120200

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): 59.2-64.5 (0.495-0.55 T) Flattop start/stop (s): -0.035/0.9 IP (MA): 0.73 – 0.6 Flattop start/stop (s): ): 0.15/0.8 Configuration: DN Equilibrium Control: Isoflux (rtEFIT) Outer gap (m): see reference Inner gap (m): Z position (m): Elongation: 1.9 Triangularity (U/L): OSP radius (m): Gas Species: D Injector(s): see reference NBI Species: D Voltage (kV) A: 90 B: 90 C: Duration (s): CHI: Off Bank capacitance (mF): LITERs: On Total deposition rate (mg/min): LLD: OFF EFC coils: On Configuration: Odd (optimum EF correction, later n=3 DC, < 1 kA).

DIAGNOSTIC CHECKLIST

| TITLE: Joint NSTX/DIII-D poloidal rotation experiment | No. OP-XP- |
| --- | --- |
| AUTHORS: R. E. Bell, K. Burrell, M. Podestá, W. Solomon | DATE: |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Beam Emission Spectroscopy | √ |  |
| Bolometer – divertor |  |  |
| Bolometer – midplane array | √ |  |
| CHERS – poloidal | √ |  |
| CHERS – toroidal | √ |  |
| Dust detector |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  |  |
| Edge pressure gauges |  |  |
| Edge rotation diagnostic | √ |  |
| Fast cameras – divertor/LLD |  |  |
| Fast ion D_alpha - FIDA |  | √ |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes |  |  |
| FIReTIP |  |  |
| Gas puff imaging – divertor |  |  |
| Gas puff imaging – midplane |  |  |
| H camera - 1D |  |  |
| High-k scattering | √ |  |
| Infrared cameras |  |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  |  |
| Langmuir probes – LLD |  |  |
| Langmuir probes – bias tile |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – B coils | √ |  |
| Magnetics – Diamagnetism | √ |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents |  |  |
| Magnetics – RWM sensors | √ |  |
| Mirnov coils – high f. | √ |  |
| Mirnov coils – poloidal array | √ |  |
| Mirnov coils – toroidal array | √ |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE |  |  |
| NPA – E¦¦B scanning |  |  |
| NPA – solid state |  |  |
| Neutron detectors |  |  |
| Plasma TV |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f | √ |  |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – divertor |  |  |
| Spectrometer – SPRED |  | √ |
| Spectrometer – VIPS |  |  |
| Spectrometer – LOWEUS |  | √ |
| Spectrometer – XEUS |  | √ |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray – pol. arrays |  | √ |
| Ultrasoft X-rays – bicolor |  | √ |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. |  | √ |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray tang. pinhole camera |  |  |