| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Search for the “I-mode” |  |  |  |
| OP-XP-10?? | Revision: | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: R. Maingi |  |  | Date Oct. 20, 2010 |
| ATI – ET Group Leader: V.A. Soukhanovskii |  |  | Date |
| RLM - Run Coordinator: E.D. Fredrickson |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| RESTRICTIONS or MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Search for the “I-mode” | No. OP-XP-10?? |
| --- | --- |
| AUTHORS: R. Maingi, A.E. Hubbard, J.W. Hughes, D.G. Whyte | DATE: Oct. 20, 2010 |

# 1.	Overview of planned experiment

The primary goal of this XP is look for the Intermediate of “I-mode” confinement regime recently reported from the Alcator C-Mod group. This experiment is part of the NSTX contribution to the FY2011 Joint Research Target on pedestal physics structure.

# 2.	Theoretical/ empirical justification

Fig. 1: Comparison of edge profiles in L-mode, I-mode, and H-mode [McDermott, PoP 16 (2009) 056103].

Recent experiments on Alcator C-mod have identified an operational regime that sits between L-mode and H-mode, i.e. an intermediate or “I-mode” scenario [McDermott, PoP 2009; Whyte NF 2010]. I-mode is observed with the “unfavorable” grad-B drift direction, just below the L-H transition power threshold. I-mode is characterized by a thermal barrier but with a weak particle barrier as seen in H-mode. Specifically the edge profiles show a pedestal-like Te profile with a tanh shape (Figure 1). This scenario is devoid of ELMs but achieves an H98y2 energy confinement multiplier ~ 1, comparable to EDA H-mode. An example of the evolution of an I-mode discharge is shown in Figure 2.

An important change in turbulence is measured in I-mode from L-mode, that differs from EDA H-mode: the QC mode does not form. Rather there is a general decrease in the broadband L-mode turbulence in I-mode, with a few bands of turbulence persisting (some times referred to as a “quasi-quasi coherent mode”, e.g. Figure 2).

Access to clear, sharp I-mode transitions is facilitated at low q95 (or more specifically high Ip) and low target density, as shown in Figure 3. Note that low q95 does require a higher L-I power threshold. High dis also favorable for access, also produces more stable discharges in NSTX, and will facilitate this search. There is no known k dependence, and drsep is typically strongly single-null in C-Mod.

# 3.	Experimental run plan (1/2 day)

Fig. 2: Example of upper single-null BT = 5.6T, q95 ∼ 3.2 I-mode discharge with divertor cryopumping with L-mode, I-mode and ELM-free H-mode phases indicated. (a) Ip and ICRF heating power, (b) Dα recycling and core radiated power, (c) D2 gas fuelling rate and line-averaged density (d) Te,95 pedestal Te at r/a ∼ 0.95 from ECE, (e) Te,0 central electron temperature, (f ) stored thermal energy Wth and confinement quality H98,y2, (g) pedestal Te gradient evaluated from edge TS, (h) local density at pedestal (r/a ∼ 0.95) and separatrix (r/a ∼ 1.0) from TS, (i) poloidal magnetic field fluctuation amplitude | ˜Bθ | averaged over the frequency range 100–150 kHz (from Bθ coils), density fluctuation amplitude spectrum from outer midplane reflectometery at 88 GHz [Whyte, NF 50 (2010) 105005].

I-mode access and sharp transitions are optimum at high Ip (more so than low Bt); high d is also favorable. We request all fluctuation diagnostics to be available. We also propose to use 150 mg of lithium between discharges for access to low L-mode target density, if possible, or a heavy liter coating in morning if that’s the only option.

- Start with a fiducial plasma: 0.9 MA, 0.45 T, but with drsep <~ 0 cm, slightly biased down, with 4 MW NBI (2)

- Perform slow drsep ramp: 0->+10mm from 100-300ms, and then back down to 0 from 300-500ms (2). The idea is to prevent the L-H (at 180 msec nominally) to study the L-mode operating window to look for I-mode. The ramp back down later is to confirm the L-H transition power.

- Adjust the HFS gas from the target discharge, probably downward, to enable acess to I-Mode. The best guess is to drop the fueling in 200 torr increments. (2)

- Implement bN feedback < 2-3, depending on best guess for stable L-mode (2)

- Increase Ip up to between 1.1 and 1.2 MA to facilitate I-mode access at low q95. If unsuccessful, drop Ip to 0.7 MA for high q95 for completeness. (4)

- Do fixed drsep values at most stable Ip: nominal +5, +10 mm (4)

- Optimize PNBI for L-mode again (2)

- Compare with reference H-modes with drsep ~ 0 with same NBI timing (2)

Note: if I-mode like profiles observed, stop and document profiles and try to obtain long-pulse quasi-steady I-mode.

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

NBI up to 6 MW, no CHI or rf.

# 5.	Planned analysis

Profile analysis and edge transport analysis will be done.

# 6.	Planned publication of results

The results will contribute to the Joule milestone report and APS papers.

Fig. 3: I-mode operational window [open squares] in C-Mod [Whyte, NF 50 (2010) 105005].

PHYSICS OPERATIONS REQUEST

| TITLE: Search for the “I-mode” | No. OP-XP-10?? |
| --- | --- |
| AUTHORS: R. Maingi, A.E. Hubbard, J.W. Hughes, D.G. Whyte | DATE: Oct. 20, 2010 |

(use additional sheets and attach waveform diagrams if necessary)

Brief description of the most important operational plasma conditions required: Relatively constant OSP radius; ability to scan Ip and Bt over wide range

Previous shot(s) which can be repeated: Previous shot(s) which can be modified: 141445, 141454

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): 0.45 T Flattop start/stop (s): IP (MA): 0.7-1.2 MA Flattop start/stop (s): Configuration: Limiter / DN / LSN / USN Equilibrium Control: Outer gap / Isoflux (rtEFIT) / Strike-point control (rtEFIT) Outer gap (m): 10cm Inner gap (m): varies Z position (m): varies Elongation: 2.2 Triangularity (U/L): 0.8 OSP radius (m): 35cm Gas Species: D2 Injector(s): NBI Species: D Voltage (kV) A: 90 B: 1 MW C: 0.5 MW Duration (s): ICRF Power (MW): Phase between straps (°): Duration (s): CHI: Off / On Bank capacitance (mF): LITERs: Off / On Total deposition rate (mg/min): 200 mg desired LLD: Temperature (°C): unheated EFC coils: Off/On Configuration: Odd / Even / Other (attach detailed sheet)

DIAGNOSTIC CHECKLIST

| TITLE: Search for the “I-mode” | No. OP-XP-10?? |
| --- | --- |
| AUTHORS: R. Maingi, et al. | DATE: Oct. 20, 2010 |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Beam Emission Spectroscopy |  | √ |
| Bolometer – divertor |  | √ |
| Bolometer – midplane array |  | √ |
| CHERS – poloidal |  | √ |
| CHERS – toroidal |  | √ |
| Dust detector |  | √ |
| Edge deposition monitors |  | √ |
| Edge neutral density diag. |  | √ |
| Edge pressure gauges |  | √ |
| Edge rotation diagnostic |  | √ |
| Fast cameras – divertor/LLD |  | √ |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes | √ |  |
| FIReTIP |  | √ |
| Gas puff imaging – divertor |  | √ |
| Gas puff imaging – midplane |  | √ |
| H camera - 1D |  | √ |
| High-k scattering |  | √ |
| Infrared cameras |  | √ |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  | √ |
| Langmuir probes – LLD |  | √ |
| Langmuir probes – bias tile |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – B coils | √ |  |
| Magnetics – Diamagnetism | √ |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes |  | √ |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents |  | √ |
| Magnetics – RWM sensors |  | √ |
| Mirnov coils – high f. |  | √ |
| Mirnov coils – poloidal array |  |  |
| Mirnov coils – toroidal array |  | √ |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE |  | √ |
| NPA – E¦¦B scanning |  |  |
| NPA – solid state |  |  |
| Neutron detectors |  | √ |
| Plasma TV |  | √ |
| Reflectometer – 65GHz |  | √ |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL |  | √ |
| RF edge probes |  |  |
| Spectrometer – divertor |  | √ |
| Spectrometer – SPRED |  | √ |
| Spectrometer – VIPS |  | √ |
| Spectrometer – LOWEUS |  | √ |
| Spectrometer – XEUS |  | √ |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray – pol. arrays |  | √ |
| Ultrasoft X-rays – bicolor |  |  |
| Ultrasoft X-rays – TG spectr. |  | √ |
| Visible bremsstrahlung det. |  | √ |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray tang. pinhole camera |  | √ |