| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Dimensionless parameter scans in H-mode plasmas |  |  |  |
| OP-XP-433 | Revision: | Effective Date: (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: S.M. Kaye, J.D. Strachan |  |  | Date |
| ATI – ET Group Leader: |  |  | Date |
| RLM - Run Coordinator: S.M. Kaye |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

Dimensionless parameter scans in H-mode plasmas	OP-XP-433

# 1.	Overview of planned experiment

The aim of this experiment is to study the dependence of the normalized confinement, B, and local transport primarily on t, holding other dimensionless variables fixed as much as possible. Knowledge of this is important for understanding the basic physics processes governing plasma transport, and for gaining confidence in making predictions for large scale devices such as ITER.

# 2.	Theoretical/ empirical justification

At present, there is much uncertainty regarding the -scaling of confinement. While scans from individual experiments exhibit a weak dependence on t, a statistical analysis of data taken from the international confinement database from a variety of devices shows a strong dependence on this parameter. The difference in scaling dependences of course results in large uncertainty in scaling confinement to larger devices, assuming  values are different. Furthermore, the difference in scaling leads to uncertainty in understanding the basic physics causing transport in toroidal geometries, whether it is electrostatic or electromagnetic. NSTX can help contribute to resolving this difference in part because of the more extended range of  that is accessible.

In a dimensionless scaling experiment, the approach is to vary one dimensionless parameter while keeping the others as fixed as possible. In the  scan, for instance, other variables such as q,  and  must be kept constant. According to the main dependences of these parameters, this means that for fixed geometry (R, a, k),

q=constant  BT/Ip = constant

=constant T  B2

=constant n  B4

With a  scan,  is held constant, which means T  B2. Ideally, rotation and Ti/Te must also be held constant in these scans.

# 3.	Experimental run plan

Because collisionality may change during the  scan, it is important to determine the scaling on this parameter as well, and, therefore, a scan to determine this dependence will be performed.  will be changed by varying either density through beam fueling (e.g., varying the number of beam sources), or toroidal field, assuming a concomitant change in Te with BT. From previous studies, either from dedicated scans or from statistical analysis, it was found that the  dependence of confinement was not a strong one.  will be varied by changing both Ip and BT at fixed BT/Ip at constant beam power (two or three sources, depending on MHD activity).  How well the other important parameters, , rotation and Ti/Te are kept fixed through these scans will be determined during the course of the experiment. It may be necessary to adjust the scans by varying gas valve fueling and beam timing and power in order to keep these parameters constant. While this initial study is expected to take approximately one day, it is likely that additional run time, with modified experimental approaches, will be required at a later date.

While most of the data on this scaling in the international database is in a DND configuration, in order to maximize the chances for results in as short a time as possible, LSN H-mode plasmas will be used as a target. It is necessary that reproducible H-modes can be obtained in these conditions (~1.9, ~0.6) with one beam source. If additional information becomes available making it possible to run this XP in DND (with H-mode occurring with only one source), we will do this. This XP cannot be executed until operations with toroidal fields up to 5.5 kG is available.

Because of the expected discharge development time during the course of the run day, the scans are limited. Below are the target discharge parameters.

BT (T)		Ip (MA)	Relative beam power		Condition

I)	0.55		1.1		High				High , low 

II)	0.35		0.7 		Low				Low , low 

III)	0.35		0.7 		High				High 

IV)	0.45		0.9 		High				Medium 

I & II determine  scaling

I, III and IV determine  scaling (when  dependence taken into account)

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

None

# 5.	Planned analysis

EFIT, TRANSP, etc.

# 6.	Planned publication of results

PRL if successful, presentation at APS, IAEA, ITPA.

PHYSICS OPERATIONS REQUEST

Dimensionless parameter scans in H-mode plasmas	OP-XP-433

Machine conditions (specify ranges as appropriate)

ITF (kA): 66 kA	Flattop start/stop (s):  _____/_____

IP (MA): 1.1 MA	Flattop start/stop (s):  _____/_____

Configuration:  Lower Single Null \l

Outer gap (m):	_____,	Inner gap (m):	_____

Elongation :	~2,	Triangularity :	~0.7 (lower)

Z position (m):	0.00

Gas Species:  D ,	Injector:  Midplane

NBI - Species: D,	Sources: A/B/C,	Voltage (kV): 90 keV,	Duration (s): _____

ICRF – Power (MW):      ,	Phasing: Heating / CD,	Duration (s): _____

CHI:   Off

Either:	List previous shot numbers for setup: 112152

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

Dimensionless parameter scans in H-mode plasmas	OP-XP-433

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer – tangential array | x |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS | x |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy | x |  |  |
| Fast lost ion probes - IFLIP |  |  |  |
| Fast lost ion probes - SFLIP |  |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes | x |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  | x |  |
| Infrared cameras |  |  |  |
| Interferometer - 1 mm |  |  |  |
| Langmuir probe array |  | x |  |
| Magnetics - Diamagnetism | x |  |  |
| Magnetics - Flux loops | x |  |  |
| Magnetics - Locked modes | x |  |  |
| Magnetics - Pickup coils | x |  |  |
| Magnetics - Rogowski coils | x |  |  |
| Magnetics - RWM sensors |  | x |  |
| Mirnov coils – high frequency |  | x |  |
| Mirnov coils – poloidal array |  | x |  |
| Mirnov coils – toroidal array | x |  |  |
| MSE | x |  |  |
| Neutral particle analyzer |  |  |  |
| Neutron measurements | x |  |  |
| Plasma TV |  | x |  |
| Reciprocating probe |  |  |  |
| Reflectometer – core |  | x |  |
| Reflectometer - SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| SPRED |  | x |  |
| Thomson scattering | x |  |  |
| Ultrasoft X-ray arrays | x |  |  |
| Visible bremsstrahlung det. |  | x |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray PIXCS (GEM) camera |  |  |  |
| X-ray pinhole camera |  |  |  |
| X-ray TG spectrometer |  |  |  |