| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Stability limits vs. normalized current at high delta with new PF1A |  |  |  |
| OP-XP-502 | Revision: | Effective Date: (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: D. Gates |  |  | Date 1/27/05 |
| ATI – ET Group Leader: S. A. Sabbagh |  |  | Date |
| RLM - Run Coordinator: J. Menard, S. Sabbagh (Deputy) |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

Error: Reference source not found Stability limits vs. normalized current at high delta with new PF1A	OP-XP-502

# 1.	Overview of planned experiment

The goal is to investigate the  limit in high triangularity double null discharges at high elongation using the recently modified PF1A coil. In particular to investigate the impact of increased edge shear and the anticipated higher operating current. The plasma internal inductance will be minimized in an attempt to avoid/delay the onset of the n=1,m=1 internal kink at high current (taking advantage of the increased edge shear.

# 2.	Theoretical/ empirical justification

It has been well documented that shaping can affect the b limit on NSTX (and other machines).  A large body of evidence supports this experiments (see e.g. NSTX 5 year plan, D. Gates and the NSTX Research Team, Phys. Plasmas 10 (2003), J. E. Menard, et al. Nuclear Fusion (2002), S. A. Sabbagh, et al., Nucl. Fusion to be published (2005) and others.

# 3.	Experimental run plan

The experiment will benefit greatly from an XMP that investigates the formation of high elongation double null high triangularity plasmas with early divertor formation. The experiment will also attempt to investigate the impact of early H-mode formation on the achievable .

1. Starting with an existing scenario ramp plasma current to 1.5MA (or as high as possible at a rate of 5.0MA/s. Adjust plasma shaping to avoid contact with plasma facing components. (3-7shots)

2. Vary the current ramp rate between 5.0 and 7.0MA/s in 0.5MA/s steps. Stop if edge MHD activity disrupts the plasma during the ramp. Adjust the plasma shape control to maintain relatively constant shape. Adjust beam timing to heat as early as possible for each ramp rate (10-20 shots, if a promising scenario is observed this section could expand).

3.  Add flatspot to Ip in attempt to induce early H-mode. Vary secondary current ramp rate to avoid deleterious MHD. If necessary try an imbalance in the PF1A coil to bias the plasma towards lower single null. Vary beam timing to heat as early as possible in each ramp rate (20 shots, if a promising scenario is observed this section could expand).

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Reliable high power NBI is a prerequisite. Thomson is required. CHERS and MSE are highly desirable. A recent boronization is required. It would be best to have investigated the use of the movable glow probe ahead of time. A vessel conditioning regime that is compatible with the high beta scenario is desirable.

# 5.	Planned analysis

EFIT is required. TRANSP will be run on select shots according to data availability. DCON analysis will be useful, particularly if q(0) > 1 in some regimes. Eventually, if n=1 internal kinks are observed as in the past, MARS analysis is possible.

# 6.	Planned publication of results

If the anticipated increase in  is realized, the results will be submitted to PRL. There will also be a more detailed Nuclear Fusion and/or a PoP paper as well.

PHYSICS OPERATIONS REQUEST

Error: Reference source not found Stability limits vs. normalized current at high delta with new PF1A	OP-XP-502

Machine conditions (specify ranges as appropriate)

ITF (kA): 36kA	Flattop start/stop (s):  N/A

IP (MA): ~1.5MA	Flattop start/stop (s):  _____/_____

Configuration: Inner Wall / Lower Single Null / Upper SN / Double Null

Outer gap (m):	10cm,	Inner gap (m):	2-4cm

Elongation :	2.5,	Triangularity :	0.8

Z position (m):	0.00

Gas Species:  D,	Injector:  Midplane

NBI - Species: D,	Sources: A/B/C,	Voltage (kV): 90kV,	Duration (s): 400ms

ICRF – Power (MW): None,	Phasing: N/A,	Duration (s): N/A

CHI:  Off

Either:	List previous shot numbers for setup: This is a new experiment.

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

Error: Reference source not found Stability limits vs. normalized current at high delta with new PF1A	OP-XP-502

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer – tangential array |  |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS |  |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  |  |  |
| Fast lost ion probes - IFLIP |  |  |  |
| Fast lost ion probes - SFLIP |  |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes |  |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer - 1 mm |  |  |  |
| Langmuir probe array |  |  |  |
| Magnetics - Diamagnetism |  |  |  |
| Magnetics - Flux loops |  |  |  |
| Magnetics - Locked modes |  |  |  |
| Magnetics - Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency |  |  |  |
| Mirnov coils – poloidal array |  |  |  |
| Mirnov coils – toroidal array |  |  |  |
| MSE |  |  |  |
| Neutral particle analyzer |  |  |  |
| Neutron measurements |  |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer – core |  |  |  |
| Reflectometer - SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| SPRED |  |  |  |
| Thomson scattering |  |  |  |
| Ultrasoft X-ray arrays |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray PIXCS (GEM) camera |  |  |  |
| X-ray pinhole camera |  |  |  |
| X-ray TG spectrometer |  |  |  |