| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Early divertor and H-mode development for long pulse in LSN |  |  |  |
| OP-XP-507 | Revision: | Effective Date: 3/08/05 (Ref. OP-AD-97) Expiration Date: 3/08/07 (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: J. Menard |  |  | Date March 8, 2005 |
| ATI – ET Group Leader: R. Maingi |  |  | Date |
| RLM - Run Coordinator: J. Menard (S. Sabbagh) |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

Early divertor and H-mode development for long pulse in LSN	OP-XP-507

# 1.	Overview of planned experiment

This experiment will attempt to further increase the pulse-length at high- in long-pulse NSTX plasmas.  This work is a continuation of previous proposals XP432 (long-pulse development), XP440 (develop early H-mode startup), and XP451 (very early LSN).  We will attempt to divert the plasma by t=50ms with good gap control.  An IP ramp-rate scan will be utilized to maximize the stable IP at t=50ms.  A pause in the current ramp will then be used to attempt to induce H-mode.  NBI timing and X-point position will be scanned if early H-mode is not easily obtained.   The subsequent IP ramp-rate and plasma lower squareness will then be scanned to try to reduce tearing and ELM activity.   Scans of TF and/or IP will then be performed to find the maximum achievable flat top duration.

# 2.	Theoretical/ empirical justification

High elongation, stronger shaping (via PF1B), and early H-mode have resulted in NSTX record pulse-length discharges at high current (IP  1MA) in a lower-single-null divertor configuration.  However, these discharges suffer from increased tearing activity in the flat-top and larger ELMs than obtained previously in lower =2-2.1 LSN plasmas.  The experiment aims to further reduce flux consumption and minimize deleterious MHD activity at high .

# 3.	Experimental run plan

Day 1 – 30 shots

  - Re-obtain FY04 long-pulse discharges at 4.5kG and document q profile   		 (10 shots)

    - Reproduce 0.8MA (112546), 0.9 (112570), 1.0 (112581) and 1.2 MA (112596)

    - Document q(R,t) of existing early H-mode scenarios with MSE

  - Modify early discharge evolution to divert ASAP and reduce flat-top IP		   (5 shots)

    - Reproduce 113460 with 1.2MA flat-top and diverted boundary at 50ms

    - Reduce flat-top current to 1MA at t=230ms

      - Remove source C, move source A as early as possible without flat-top disruption

  - Find highest stable IP ramp-rate before t=50ms in 1MA target from 3.b.ii  		(15 shots)

    - Increase IP request at t=50ms from 350kA in 100kA increments

    - Increase current linearly from IP value at t=50ms to 1MA at t=230ms

    - Control inner gap = 6-8cm, outer gap = 8-10cm during ramp and flat-top

Day 2 – 30 shots

  - Attempt to induce H-mode at t=50ms at highest stable current from 3.c 		(10 shots)

    - Add current pause after t=45ms for 15ms	 - reduce/scan pause time if H-mode is obtained

    - Move source A start-time to t=35ms if it is not already that early

    - If H-mode transition is not obtained, add source B at t=40ms

    - If transition still not obtained, reduce bottom X-point height

    - If transition still not obtained, increase bottom X-point major radius with PF2

  - Scan IP ramp-rate after t=50ms to minimize tearing (w/ or w/o early H-mode)  	(5 shots)

  - Scan lower squareness during ramp and flat-top to determine impact on MHD	(5 shots)

  - Maximize plasma flat-top duration 							(8 shots)

    - If pulse-length is limited by TF coil, reduce TF in 0.25kG steps at fixed IP

    - If pulse-length is limited by OH coil or MHD, reduce IP in 0.50kA steps at fixed TF

  - Increase IP to 1.2MA to document OH, TF, and/or MHD limits at high current	(2 shots)

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

The usual diagnostic capabilities are required, NBI voltages are A, B, C = 90, 90, 80kV

# 5.	Planned analysis

EFIT, TRANSP, MPTS, CHERS, and internal magnetic sensor analysis will be performed.

# 6.	Planned publication of results

Results will be published in conference proceedings and/or journal such as Nuclear Fusion or Physics of Plasmas within one year of experiment.

PHYSICS OPERATIONS REQUEST

Early divertor and H-mode development for long pulse in LSN	OP-XP-507

Machine conditions (specify ranges as appropriate)

ITF (kA): 42-53kA		Flattop start/stop (s):  _-0.02s_/_1-1.5s____

IP (MA): 0.8-1.2		Flattop start/stop (s):  0.12-0.18 / 0.4

Configuration: LSN

Outer gap (m):	5-10cm,	Inner gap (m):	4-6cm

Elongation :	2.1-2.5,	Triangularity :	0.5-0.7

Z position (m):	0.00

Gas Species:  D,	Injector:  CS Midplane, Outer Midplane

NBI - Species: D,	Sources: A,B,C Voltage (kV): 90,90,80kV,	Duration (s): 1s

ICRF – Power (MW): ____,	Phasing: N/A,	Duration (s): _____

CHI:  Off

Either:	Previous shot numbers for setup: 112546, 112570, 112581, 112596, 113460

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

Early divertor and H-mode development for long pulse in LSN	OP-XP-507

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer - tangential array |  |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS |  |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  |  |  |
| Fast lost ion probes – IFLIP |  |  |  |
| Fast lost ion probes – SFLIP |  |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes |  |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  |  |
| High-k scattering |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer – 1 mm |  |  |  |
| Langmuir probes - PFC tiles |  |  |  |
| Langmuir probes - RF antenna |  |  |  |
| Magnetics – Diamagnetism |  |  |  |
| Magnetics – Flux loops |  |  |  |
| Magnetics – Locked modes |  |  |  |
| Magnetics – Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency |  |  |  |
| Mirnov coils – poloidal array |  |  |  |
| Mirnov coils – toroidal array |  |  |  |
| MSE |  |  |  |
| Neutral particle analyzer |  |  |  |
| Neutron Rate (2 fission, 4 scint) |  |  |  |
| Neutron collimator |  |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer - FM/CW |  |  |  |
| Reflectometer - fixed frequency homodyne quadrature |  |  |  |
| Reflectometer - homodyne correlation |  |  |  |
| Reflectometer - HHFW/SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| Solid State NPA |  |  |  |
| SPRED |  |  |  |
| Thomson scattering - 20 channel |  |  |  |
| Thomson scattering - 30 channel |  |  |  |
| Ultrasoft X-ray arrays |  |  |  |
| Ultrasoft X-ray arrays - 2 color |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray pinhole camera |  |  |  |
| X-ray TG spectrometer |  |  |  |