| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Non-solenoidal Ip Rampup with HHFW |  |  |  |
| XP-521 | Revision: | Effective Date: 03/03/05 (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: C. E. Kessel, O. Mitarai, R. Maingi, RF Group |  |  | Date 03/03/05 |
| ATI – ET Group Leader: R. Maingi and C. E. Kessel (deputy) |  |  | Date |
| RLM - Run Coordinator: J. Menard (S. Sabbagh) |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

Error: Reference source not found	Error: Reference source not found

# 1.	Overview of planned experiment

Use the solenoid to ramp up the plasma current to various values, then flattop the OH coil current, and inject HHFW power to observe the heating/CD effectiveness in sustaining the plasma current.  Starting from 400 kA at tflattop = 80 ms, the plasma current values will be progressively reduced.   A few NBI heating cases will be done first to setup various turn on times for rtEFIT/isoflux, OH current clamping, and heating.

# 2.	Theoretical/ empirical justification

The goal of non-solenoidal current startup and rampup can be broken into 3 phases; the breakdown and early startup, the RF plasma current rampup, and the RF+NBI plasma current rampup.  These can be separated and developed independently.  At low plasma currents which we expect CHI or PF coil startup to provide, say 50-200 kA the NBI confinement is poor, and an RF method for heating and driving current is preferable.  The HHFW system is suited to this purpose, and efforts to develop this capability should be pursued.  In the 2004 run campaign, routine heating was established at 7 m-1 and 14 m-1 in deuterium plasmas, along with transition into H-mode.  This requires the plasma to be close to the antenna for sufficient coupling.  The challenge here is to establish this coupling (location of R+a relative to the antenna) at earlier times, at lower plasma currents and lower electron temperatures.  The approach taken here is to start with a successful HHFW H-mode discharge from the 2004 run campaign, and terminate the plasma current rampup at lower currents, but otherwise apply a similar timing for position control and heating/CD.  Simulations showed under various conditions that heating or heating with CD could sustain the plasma current at 400 kA, and drive the plasma current up from 200 kA, with 3 MW of HHFW power, and reasonable transport assumptions.

# 3.	Experimental run plan

Reference discharge for HHFW is 112283, an rtEFIT/isoflux controlled HHFW experiment, with IP = 700 kA, BT = 0.45 T, k|| = 14 m-1, HOWEVER, we will be cutting the IP rampup short.

1)  reference shot:  rampup to 500 kA, t = 80 ms, flattop IP at 500 kA, apply rtEFIT/isoflux at 110 ms, NO OH clamping, and NO NBI, record IOH at 140 ms  (2 shots)

2)  use solenoid to rampup plasma current up to 500 kA, at t = 80 ms, flattop IP = 500 kA, apply rtEFIT/isoflux at 110 ms, clamp OH current at its value at 140 ms, and apply 2 (A at 90 keV, B at 70 keV), and 3 (A at 90 keV, B at 70 keV, C at 70 keV) NBI sources at 150 ms, respectively.  Maintain 10 cm outer gap.  (3 shots)

3) reference shot:  rampup to 400 kA, t = 80 ms, flattop IP at 400 kA, apply rtEFIT/isoflux at 110 ms, NO OH clamping and NO HHFW, record IOH at 140 ms.  (2 shots)

4) Use solenoid to ramp plasma current up to 400 kA, at t = 80 ms. Flattop  IP at 400 kA, apply rtEFIT/isoflux at 110 ms, clamp OH current at its value at 140 ms, apply 2-4 MW of HHFW with 14 m-1 heating phasing at 150 ms.  Maintain <  5 cm outer gap.  (5 shots)

5) reference shot:  rampup to 300 kA, t = 80 ms, flattop IP at 300 kA, apply rtEFIT/isoflux at 110 ms, NO OH clamping and NO HHFW, record IOH at 140 ms.  (2 shots)

6)  same as #4 but ramp up to IP = 300 kA  (5 shots)

7) reference shot:  rampup to 200 kA, t = 70 ms, flattop IP at 200 kA, apply rtEFIT/isoflux at 110 ms, NO OH clamping and NO HHFW, record IOH at 140 ms.  (2 shots)

8)  same as #4 but ramp up to IP = 200 kA  (5 shots)

9) choose one or 2 shots and do references:  apply HHFW with no OH current clamping, and clamp OH current with no HHFW.  (4 shots)

Total shots:  30

10) if difficulties, shift entire timing sequence out by 20 ms

11) attempt some cases with 7 m-1 heating phasing, and 7 m-1 co-CD phasing

12) ramp up to 300 kA at t = 60 ms, and apply same time sequence as #1 and #2

13) ramp up to 200 kA at t = 40 ms, and apply same time sequence as #1 and #2

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

HHFW system, maximum power available, 7 m-1 and 14 m-1, heating phasing and possibly co-CD phasing

NBI blips of source A 90 keV and B at 70 keV, 10 ms duration, separated by 40 ms, starting at 100 ms

MSE diagnostic, NBI blips, appears to require 100 ms blips, which is too long, due to beam voltage transient.

BT = 0.45 T

IP ramp, HHFW with rtEFIT/isoflux  control, 14 m-1 reference is 112283 (700 kA), traamp = 175 ms, tisoflux = 200 ms, tHHFW = 250 ms.

rtEFIT/isoflux controller

NBI 3 sources, A at 90 keV, B at 70 keV, C at 70 keV

# 5.	Planned analysis

EFIT

CURRAY

TRANSP

TSC

# 6.	Planned publication of results

Results would be presented at the APS meeting and subsequently submitted to Nuc. Fusion or Phys. of Plasmas

PHYSICS OPERATIONS REQUEST

Error: Reference source not found	Error: Reference source not found

Machine conditions (specify ranges as appropriate)

ITF (kA): 52	Flattop start/stop (s):  0/1.5

IP (MA): 0.2-0.5	Flattop start/stop (s):  0.08/0.6

Configuration: Double Null, diverted

Outer gap (m):	<0.05,	Inner gap (m):

Elongation :	≈2.0,	Triangularity :	0.5

Z position (m):	0.00

Gas Species:  D,	Injector:  Midplane / Inner wall / Lower Dome

NBI - Species:D,	Sources: A/B/C,	Voltage (kV): 90/70/70,	Duration (s): 0.8

ICRF – Power (MW): 4.0,	Phasing: Heating/CD,	Duration (s): 0.5

CHI:  OFF

Either:	List previous shot numbers for setup: _____________________________

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

Error: Reference source not found	Error: Reference source not found

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer - tangential array |  |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS | X |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  |  |  |
| Fast lost ion probes – IFLIP | X |  |  |
| Fast lost ion probes – SFLIP | X |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes |  |  |  |
| FIReTIP | X |  |  |
| Gas puff imaging |  |  |  |
| High-k scattering |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer – 1 mm | X |  |  |
| Langmuir probes - PFC tiles |  |  |  |
| Langmuir probes - RF antenna |  |  |  |
| Magnetics – Diamagnetism | X |  |  |
| Magnetics – Flux loops |  |  |  |
| Magnetics – Locked modes | X |  |  |
| Magnetics – Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors | X |  |  |
| Mirnov coils – high frequency | X |  |  |
| Mirnov coils – poloidal array | X |  |  |
| Mirnov coils – toroidal array | X |  |  |
| MSE | X |  |  |
| Neutral particle analyzer | X |  |  |
| Neutron Rate (2 fission, 4 scint) |  |  |  |
| Neutron collimator |  |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer - FM/CW |  |  |  |
| Reflectometer - fixed frequency homodyne quadrature |  |  |  |
| Reflectometer - homodyne correlation |  |  |  |
| Reflectometer - HHFW/SOL | X |  |  |
| RF antenna camera | X |  |  |
| RF antenna probe | X |  |  |
| Solid State NPA | X |  |  |
| SPRED |  |  |  |
| Thomson scattering - 20 channel |  |  |  |
| Thomson scattering - 30 channel | X |  |  |
| Ultrasoft X-ray arrays |  |  |  |
| Ultrasoft X-ray arrays - 2 color |  |  |  |
| Visible bremsstrahlung det. | X |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray PIXCS (GEM) camera |  |  |  |
| X-ray pinhole camera |  |  |  |
| X-ray TG spectrometer |  |  |  |