| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Study of Transport with Reversed Shear in NSTX |  |  |  |
| OP-XP-522 | Revision: | Effective Date: (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: F. M. Levinton, D. Stutman, Kaye, J. Menard, E. Synakowski |  |  | Date 3-25-2005 |
| ATI – ET Group Leader: S. Kaye |  |  | Date |
| RLM - Run Coordinator: J. Menard (S. Sabbagh) |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL	OP-XP-522

# 1.	Overview of planned experiment

The plan of this experiment is to develop a robust reversed shear q-profile and study the thermal and particle transport properties of the ions and electrons.  We will have available the MSE diagnostic and EFIT for between shot analysis.  This will provide the needed guidance to develop the appropriate q-profile.  When the reversed shear scenario was developed on TFTR the guidance from MSE was essential.  The presence of MHD of various forms was the major impediment to achieving a high q(0) and strongly reversed shear q-profile.  It was by systematically eliminating the deleterious MHD modes that a high q(0) was achieved.  To obtain a reversed shear q-profile the plan proposed for NSTX, similar to TFTR, is to start with a large fully grown plasma as early as possible in the discharge with a high Te and then ramp up the plasma current.  The large, hot plasma, before the current has ramped up will maximize the current penetration time leading to a hollow current profile and reversed shear.  The main elements in the plan for reversed shear development will be;

- Document q-profile evolution in low density L-mode shots from XP-411:

Varying ramp rate and beam timing in L-mode discharges created in XP-411 documented changes in the Te and Ti profiles and thermal transport. We propose documenting three conditions with the MSE diagnostic, reproducing the conditions from that XP.  The indication from the USXR diagnostic and TRANSP modeling is that the barrier is associated with a region of negative magnetic shear.

- Develop reversed shear with H-mode:

Develop an early H-mode using NBI injection and fast current ramp, avoiding MHD with pauses in ramp when necessary.  Utilize helium conditioning and perhaps lithium pellets and/or RWM coils depending on results from XP’s 501(RWM) and 515(lithium pellet injection).  If HHFW is successfully coupled into the startup phase of a discharge to increase the plasma temperature, then this technique could be incorporated into the plan.  This will depend on the results of XMP –030 and XP-510.

3.  Develop reversed shear with low density L-mode edge:

Using similar techniques as above, but with low density L-mode.

# 2.	Theoretical/ empirical justification

Many theoretical based models for stability and transport depend on the q-profile and in particular on the magnetic shear.  Experimental observations of the effect of magnetic shear on transport and stability have been documented in several tokamaks, such as TFTR and DIII-D.  In this XP we propose to explore the stability and transport effects in the ST with reversed shear.

# 3.	Experimental run plan

The conditions for this XP should be run at 4.5 kG with source A and B at 90 kV.  If possible some conditions at higher magnetic field would be repeated.  In order to slow the current penetration to the plasma core we want to elevate the electron temperature as much as possible and produce as large a plasma as possible.

- Document q-profile evolution in low density L-mode shots from XP-411:

We plan on documenting three conditions from XP411 with the MSE diagnostic. Shots 112996, 112988, 112989 have different ramp rates and very different electron temperature profiles.  The discharge with the fastest current ramp and earlier neutral beam start time has the broader electron temperature profile.   The original XP had source B injecting early.  We will substitute source A in order to get MSE data.  If higher field than 4.5 kG is possible we would like to try increasing the magnetic field for conditions 112996 and 112989.  We would maintain the plasma current at 1.0 MA, but increase the beam power.  Good machine condition was essential for XP411.  Helium discharge conditioning was not used in XP411, and may have compromised the performance during the second half of the day when XP411 was run.  We plan on trying to maintain good machine conditions during the course of the day.

- Develop reversed shear with H-mode:

We propose using a fast current ramp, similar to 112989, but grow the plasma more rapidly to full size with high kappa and DND. Increase beam power, if necessary, to go into H-mode.  Avoid MHD using the following techniques;

  - Vary current ramp rate (6 MA/s, 5MA/s, 4MA/s).

  - Vary NB timing (40-100 ms start time).

  - Pauses in the current ramp to allow some current penetration.  This reduces the edge current density if external MHD modes are the problem.

Utilize helium conditioning and perhaps lithium pellets and/or RWM coils depending on results from XP’s 501(RWM) and 515(lithium pellet injection).  If HHFW is successfully coupled into the startup phase of a discharge to increase the plasma temperature, then this technique could be used here.  This will depend on the results of XMP –030 and XP-510.

3.   Develop reversed shear with low density L-mode edge:

In contrast to H-mode plasmas,  lower density L-mode offers a regime with a different collisionality and profiles to test the effects of reversed shear in.  The approach for development would be similar to section #2  above, but with the aim of preventing an H-mode from developing.   The strategy to prevent H-mode from occurring would be;

-Gap adjustment.

-Limiter operation.

-Reduced NBI power.

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Well conditioned operation at 1.0 MA and 4.5 kG. Source A at 90 kV, and source B and C available.  For parts 2 and 3 of this XP it would be best to have done XP-501 and –515, as well as XMP-30/XP-510 for RF development.

# 5.	Planned analysis

LRDFIT and EFIT for equilibrium reconstruction.  Transp for transport analysis.  Possible analysis of microstability with GS2 or other codes.

# 6.	Planned publication of results

APS meeting and refereed journals.

PHYSICS OPERATIONS REQUEST

OP-XP-522

Machine conditions (specify ranges as appropriate)

ITF (kA): 53	Flattop start/stop (s):  _____/_____

IP (MA): 1.0	Flattop start/stop (s):  .14-.33/ > .5

Configuration: Inner Wall / Lower Single Null / Upper SN / Double Null

Outer gap (m):	_____,	Inner gap (m):	_____

Elongation :	_____,	Triangularity :	_____

Z position (m):	0.00

Gas Species:  D / He,	Injector:  Midplane / Inner wall / Lower Dome

NBI - Species: D,	Sources: A/B/C,	Voltage (kV): 90,	Duration (s): .6

ICRF – Power (MW): ____,	Phasing: Heating / CD,	Duration (s): _____

CHI:  Off

Either:	List previous shot numbers for setup: _____________________________

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

OP-XP-522

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer - tangential array |  |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS |  |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  |  |  |
| Fast lost ion probes – IFLIP |  |  |  |
| Fast lost ion probes – SFLIP |  |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes |  |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  |  |
| High-k scattering |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer – 1 mm |  |  |  |
| Langmuir probes - PFC tiles |  |  |  |
| Langmuir probes - RF antenna |  |  |  |
| Magnetics – Diamagnetism |  |  |  |
| Magnetics – Flux loops |  |  |  |
| Magnetics – Locked modes |  |  |  |
| Magnetics – Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency |  |  |  |
| Mirnov coils – poloidal array |  |  |  |
| Mirnov coils – toroidal array |  |  |  |
| MSE |  |  |  |
| Neutral particle analyzer |  |  |  |
| Neutron Rate (2 fission, 4 scint) |  |  |  |
| Neutron collimator |  |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer - FM/CW |  |  |  |
| Reflectometer - fixed frequency homodyne quadrature |  |  |  |
| Reflectometer - homodyne correlation |  |  |  |
| Reflectometer - HHFW/SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| Solid State NPA |  |  |  |
| SPRED |  |  |  |
| Thomson scattering - 20 channel |  |  |  |
| Thomson scattering - 30 channel |  |  |  |
| Ultrasoft X-ray arrays |  |  |  |
| Ultrasoft X-ray arrays - 2 color |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray PIXCS (GEM) camera |  |  |  |
| X-ray pinhole camera |  |  |  |
| X-ray TG spectrometer |  |  |  |