| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Dependence of ELM Severity and Confinement on Boundary Shape |  |  |  |
| OP-XP-526 | Revision: | Effective Date: (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: S.M. Kaye, R. Maingi, D. Gates |  |  | Date 6/10/05 |
| ATI – ET Group Leader: R. Maingi |  |  | Date |
| RLM - Run Coordinator: J. Menard (S. Sabbagh) |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

Boundary Shape Dependences	OP-XP-526

# 1.	Overview of planned experiment

The goal of this experiment is to examine the severity of ELMs and dependence of energy confinement on plasma shape, most notably elongation and triangularity. Because of the scans involved in this XP, both topics can be addressed. Scans in both triangularity at fixed elongation, and the converse, will be performed. The experiment is planned to complete in 1 run day.

# 2.	Theoretical/ empirical justification

ELM severity has been found to depend on plasma elongation. In experiments conducted in FY04, the plasma elongation and X-point height were varied simultaneously, but then independently, to determine that plasma elongation was the controlling factor, with ELM severity getting worse, and plasma stored energy decreasing, with increasing elongation (Fig. 1). In this set of discharges, neither triangularity or squareness was held fixed. Plasma confinement was also shown to be dependent on plasma boundary shape, with the confinement worsening with increased triangularity (Fig. 2). The source of this is not entirely ELMS, as this trend is observed even in ELM-free discharges, as can be seen in the figure.

Fig. 1

Fig. 2

Fig. 2

# 3.	Experimental run plan

In this experiment we will separate the effect of elongation and triangularity to address how ELM severity and plasma confinement varies with these parameters. Several scans at fixed elongation or triangularity are planned. The scan will start with a baseline shot of 115308/310 at an elongation of 2.1 and triangularity of 0.6 (800 kA). The first scan will be to increase kappa to the max (2.7) while keeping delta fixed. Then, delta will be scanned at fixed kappa, and finally, another kappa scan will be performed at lower delta. These scans can be done within the operating limits of the new PF1a coil. The run plan, in tabular form, is given below along with a schematic showing the shots in kappa-delta space.

| CASE | Kappa | delta |
| --- | --- | --- |
| 1 | 2.1 | 0.6 |
| 2 | 2.3 | 0.6 |
| 3 | 2.5 | 0.6 |
| 4 | 2.7 | 0.6 |
| 5 | 2.5 | 0.7 |
| 6* | 2.5 | 0.8 |
| 7 | 2.5 | 0.5 |
| 8* | 2.5 | 0.4 |
| 9 | 2.3 | 0.4 |
| 10 | 2.1 | 0.4 |
| 11 | 1.9 | 0.4 |

# * Power scan to be performed  - First three sources, then 1 source (A or B, whichever is on earlier)

# Several shots may need to be taken at given conditions in order to obtain enough data for good stability and transport analysis.

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

No special capabilities are needed. The edge stability analysis to be done in support of this XP would benefit from high resolution MPTS implementation, but it may be more important to get a first cut through the space before going back to diagnose specific conditions with this high resolution/edge capability.

# 5.	Planned analysis

EFIT, TRANSP, Edge stability (ELITE)

# 6.	Planned publication of results

Presented at meetings, and published in a refereed journal.

PHYSICS OPERATIONS REQUEST

Boundary Shape Dependences	OP-XP-526

Machine conditions (specify ranges as appropriate)

ITF (kA): 56	Flattop start/stop (s):  usual

IP (MA): 0.8	Flattop start/stop (s):  usual

Configuration:  Double Null

Outer gap (m):	_____,	Inner gap (m):	_____

Elongation :	_____,	Triangularity :	_____

Z position (m):	0.00

Gas Species:  D,	Injector:  Midplane

NBI - Species: D,	Sources: A/B/C,	Voltage (kV): 90,	Duration (s): _____

ICRF – Power (MW): ____,	Phasing: Heating / CD,	Duration (s): _____

CHI:  Off

Either:	List previous shot numbers for setup: 115308 with outer gap reduced by 2-3 cm for outer MPTS channels

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

Boundary Dependences	OP-XP-526

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer - tangential array | ♫ |  |  |
| Bolometer array - divertor |  | ♪ |  |
| CHERS | ♫ |  |  |
| Divertor fast camera | ♫ |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  | ♪ | Only available by special request of T. Biewer @ MIT |
| Fast lost ion probes – IFLIP |  |  |  |
| Fast lost ion probes – SFLIP |  |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes | ♫ |  |  |
| FIReTIP |  | ♪ |  |
| Gas puff imaging |  | ♪ |  |
| High-k scattering |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer – 1 mm |  |  |  |
| Langmuir probes - PFC tiles |  |  |  |
| Langmuir probes - RF antenna |  |  |  |
| Magnetics – Diamagnetism | ♫ |  |  |
| Magnetics – Flux loops | ♫ |  |  |
| Magnetics – Locked modes |  | ♪ |  |
| Magnetics – Pickup coils | ♫ |  |  |
| Magnetics - Rogowski coils | ♫ |  |  |
| Magnetics - RWM sensors |  | ♪ |  |
| Mirnov coils – high frequency | ♫ |  |  |
| Mirnov coils – poloidal array | ♫ |  |  |
| Mirnov coils – toroidal array | ♫ |  |  |
| MSE | ♫ |  |  |
| Neutral particle analyzer |  |  |  |
| Neutron Rate (2 fission, 4 scint) |  |  |  |
| Neutron collimator |  |  |  |
| Plasma TV | ♫ |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer - FM/CW |  |  |  |
| Reflectometer - fixed frequency homodyne quadrature |  |  |  |
| Reflectometer - homodyne correlation |  |  |  |
| Reflectometer - HHFW/SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| Solid State NPA |  |  |  |
| SPRED |  |  |  |
| Thomson scattering - 20 channel | ♫ |  |  |
| Thomson scattering - 30 channel | ♫ |  |  |
| Ultrasoft X-ray arrays | ♫ |  |  |
| Ultrasoft X-ray arrays - 2 color |  | ♪ |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray pinhole camera |  |  |  |