| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: HHFW Heating into Reversed-shear Plasmas |  |  |  |
| OP-XP-538 | Revision: 0.1 | Effective Date: (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: B. LeBlanc, R.J. Wilson, J. Menard, F. Levinton, D. Stutman, R. Bell, D. Gates, J.C. Hosea |  |  | Date: 17-Aug-05 |
| ATI – ET Group Leader: J.R. Wilson |  |  | Date |
| RLM - Run Coordinator: J. Menard (S. Sabbagh) |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

Title: HHFW Heating into Reversed-shear Plasmas	OP-XP-538

# 1.	Overview of planned experiment

The goal of this experiment is to apply HHFW electron heating, some of it directed, to reversed-shear (RS) L-mode plasmas.

# 2.	Theoretical/ empirical justification

The justification of this exercise resides in the experimental observation and theoretical expectation of better electron thermal confinement in RS plasmas. For example, analysis of HHFW heated plasma with very high Te (i.e. 4 keV) suggested that q(R) was reversed. Also last year’s work by Stutman indicated that RS improved electron thermal confinement.

# 3.	Experimental run plan

(a) Utilize RS scenario developed by Stutman and Levinton to establish a plasma target suitable for HHFW operation.

(b) Attempt to separate affects of fast-ion HHFW power absorption by delaying HHFW onset after NBI turnoff.

| Shot Id | Description | NBI A(90 kV) | HHFW ( 2 MW) | N |
| --- | --- | --- | --- | --- |
| Apply HHFW in presence of fast-ion population |  |  |  |  |
| 0 | Target | 0.2-0.5 s | No | 1 |
| 1 | Add HHFW heating | 0.2-0.5 s | 0.2-0.5 s, k = 14 m-1 | 1 |
| 2 | Add HHFW ctr-CD | 0.2-0.5 s | 0.2-0.5 s, k = 7 m-1 | 1 |
| 3 | Add HHFW co-CD | 0.2-0.5 s | 0.2-0.5 s, k = -7 m-1 | 1 |
| Apply HFFW in “absence” of fast ions, i.e. wait 50 ms after NBI turnoff |  |  |  |  |
| 4 | Target, but | 0.2-0.3, 0.4-0.5 s | No | 1 |
| 5 | Add HHFW heating | 0.2-0.3, 0.4-0.5 s | 0.35-0.5 s, k = 14 m-1 | 1 |
| 6 | Add HHFW ctr-CD | 0.2-0.3, 0.4-0.5 s | 0.35-0.5 s, k = -7 m-1 | 1 |
| 7 | Add HHFW co-CD | 0.2-0.3, 0.4-0.5 s | 0. 35-0.5 s, k = 7 m-1 | 1 |

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

This XP will need NBI and RF system. 
	See attached Physics Operations Request and Diagnostic Checklist pages for details.

# 5.	Planned analysis

EFIT, TRANSP, and specialized RF codes.

# 6.	Planned publication of results

The data will analyzed by the lead author and depending on experimental success, publication will be made to PRL or POP, or equivalent.

PHYSICS OPERATIONS REQUEST

Title: HHFW Heating into Reversed-shear Plasmas	OP-XP-538

Machine conditions (specify ranges as appropriate)

ITF (kA): 4.5 kG	Flattop start/stop (s):  _____/_____

IP (MA): ___1.0___	Flattop start/stop (s):  _____/_____

Configuration:  Double Null

Outer gap (m):	0.04 m,	Inner gap (m):	_____

Elongation :	_____,	Triangularity :	_____

Z position (m):	0.00

Gas Species:  D ,	Injector:  Midplane / Inner wall / Lower Dome

NBI - Species: D,	Sources: A,	Voltage (kV): 90,	Duration (s): _____

ICRF – Power (MW): > 2.0,	Phasing: Heating / CD,	Duration (s): _____

CHI:  Off

Either:	List previous shot numbers for setup: 116978 <<<<<<<<<<<<<<<<

Desired Heating patterns

Shots 0-3

Shots 4-7

DIAGNOSTIC CHECKLIST

Title: HHFW Heating into Reversed-shear Plasmas	OP-XP-538

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer - tangential array |  |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS |  |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  |  | Only available by special request of T. Biewer @ MIT |
| Fast lost ion probes – IFLIP |  |  |  |
| Fast lost ion probes – SFLIP |  |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes |  |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  |  |
| High-k scattering |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer – 1 mm |  |  |  |
| Langmuir probes - PFC tiles |  |  |  |
| Langmuir probes - RF antenna |  |  |  |
| Magnetics – Diamagnetism |  |  |  |
| Magnetics – Flux loops |  |  |  |
| Magnetics – Locked modes |  |  |  |
| Magnetics – Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency |  |  |  |
| Mirnov coils – poloidal array |  |  |  |
| Mirnov coils – toroidal array |  |  |  |
| MSE |  |  |  |
| Neutral particle analyzer |  |  |  |
| Neutron Rate (2 fission, 4 scint) |  |  |  |
| Neutron collimator |  |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer - FM/CW |  |  |  |
| Reflectometer - fixed frequency homodyne quadrature |  |  |  |
| Reflectometer - homodyne correlation |  |  |  |
| Reflectometer - HHFW/SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| Solid State NPA |  |  |  |
| SPRED |  |  |  |
| Thomson scattering - 20 channel |  |  |  |
| Thomson scattering - 30 channel |  |  |  |
| Ultrasoft X-ray arrays |  |  |  |
| Ultrasoft X-ray arrays - 2 color |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray pinhole camera |  |  |  |