| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Study of Transport with Reversed Shear in NSTX |  |  |  |
| OP-XP-610 | Revision: 0 | Effective Date: (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: F. M. Levinton |  |  | Date 2-6-2006 |
| ATI – ET Group Leader: M. Bell |  |  | Date |
| RLM - Run Coordinator: R. Raman |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL	OP-XP-610

# 1.	Overview of planned experiment

The plan of this experiment is to continue development of reversed shear q-profiles and study the thermal and particle transport properties of the ions and electrons.  We will have available the MSE diagnostic and EFIT for between shot analysis.  This will provide the needed guidance to develop the appropriate q-profile.  The presence of MHD of various forms is the major impediment to achieving a high q(0) and strongly reversed shear q-profile. Last year we ran XP522 in NSTX which successfully obtained a reversed shear q-profile.  The results indicated a correlation of improved ion and electron transport with a reversed shear q-profile in an L-mode plasma.  We plan to continue to pursue these studies with the aim of varying the shear and documenting the transport with L-mode.  We obtained some variation of shear last year, but would like to extend this to a monotonic q-profile to obtain a wider range of parameters and also operate experiment at higher magnetic field if possible.  The other part of the experimental plan is to repeat the L-mode case, but have the plasma transition into an H-mode by moving it off the center stack.  This was attempted last year, but very little time was spent on it.  We did manage to get an H-mode transition that lasted a short amount of time.  We would like to extend the period of the H-mode phase to determine the thermal and particle transport properties.

# 2.	Theoretical/ empirical justification

Many theoretical based models for stability and transport depend on the q-profile and in particular on the magnetic shear.  Experimental observations of the effect of magnetic shear on transport and stability have been documented in several tokamaks, such as TFTR and DIII-D.  In this XP we propose to explore the stability and transport effects in the ST with reversed shear.

# 3.	Experimental run plan

The conditions for this XP should be run at 4.5 kG, or higher if possible, with NB-A and NB-B at 90 kV.  In order to slow the current penetration to the plasma core we want to elevate the electron temperature as much as possible and produce as large a plasma as possible.

- Reversed shear with L-mode at 4.5 kG:

- Reproduce 116960.

- Repeat 115821.

- Develop monotonic q-profile:  Vary NB-A timing.  Start NB-A at 0.04, 0.25, and 0.275 seconds.

- If (c) does not work then add NB-B at 0.04, 0.08, .120  seconds for 50-100 ms to try to induce an early magnetic reconnection.

- Slow current ramp and delay NB-A to allow current penetration; Ip flattop at 0.275 seconds, NB-A at .295 seconds.

Utilize GDC between shots.

This scenario development will take about 15 shots.

- Reversed shear with L-mode at >4.5 kG:

  - Reproduce 116960 like conditions at higher field.  May require some retuning of NB-A timing to avoid MHD problems.

  - Repeat 1b above, but with retuning if necessary.

  - Repeat most successful approach of 1c-1e.

This development will take about 8 shots.

- Develop reversed shear with H-mode:

In contrast to L-mode plasmas,  higher density H-mode offers a regime with a different collisionality and high pressure gradient at the edge.  This will change the NBCD and bootstrap current profile.  The approach for development would be to take our condition in section 1 or 2 above and pull the plasma off the center stack(CS) after the reversed shear high Te mode has developed and add CS gas for a transition into H-mode.  This approach was begun last year, but was not completed.  One shot (116982) was successful in forming an H-mode but disrupted early.  The goal here is to continue with that approach and extend the length of the H-mode phase.

- H-mode scan: Move inner gap about 2 cm off CS at 0.2 seconds.  Use CS gas injector to induce H-mode(116982).

- Vary timing (0.2-0.25 seconds) of inner gap movement.

- Add NB-B at 0.3 s.

This section will take about 15 shots.

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Well conditioned operation at 1.0 MA and 4.5 kG or higher. NB-A at 90 kV.    NB-B and NB-C available.

# 5.	Planned analysis

LRDFIT and EFIT for equilibrium reconstruction.  Transp for transport analysis.  Possible analysis of microstability with GS2, GYRO,  or other codes.

# 6.	Planned publication of results

APS meeting and refereed journals.

PHYSICS OPERATIONS REQUEST

OP-XP-610

Machine conditions (specify ranges as appropriate)

ITF (kA): 53	Flattop start/stop (s):  _____/_____

IP (MA): 1.0	Flattop start/stop (s):  .14-.33/ > .5

Configuration: Inner Wall / Lower Single Null / Upper SN / Double Null

Outer gap (m):	_____,	Inner gap (m):	_____

Elongation :	_____,	Triangularity :	_____

Z position (m):	0.00

Gas Species: D	Injector:  Midplane / Inner wall / Lower Dome

NBI - Species: D,	Sources: A/B/C,	Voltage (kV): 90,	Duration (s): .7

ICRF – Power (MW): ____,	Phasing: Heating / CD,	Duration (s): _____

CHI:  Off

Either:	List previous shot numbers for setup: 116960, 115821

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

OP-XP-610

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer - tangential array |  |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS | x |  |  |
| Divertor fast camera |  | x |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  | x |  |
| Fast lost ion probes – IFLIP |  |  |  |
| Fast lost ion probes – SFLIP |  |  |  |
| Filtered 1D cameras |  |  |  |
| Filterscopes |  | x |  |
| FIReTIP |  | x |  |
| Gas puff imaging |  | x |  |
| High-k scattering | x |  |  |
| Infrared cameras |  |  |  |
| Interferometer – 1 mm |  |  |  |
| Langmuir probes - PFC tiles |  |  |  |
| Langmuir probes - RF antenna |  |  |  |
| Magnetics – Diamagnetism | x |  |  |
| Magnetics – Flux loops | x |  |  |
| Magnetics – Locked modes | x |  |  |
| Magnetics – Pickup coils | x |  |  |
| Magnetics - Rogowski coils | x |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency | x |  |  |
| Mirnov coils – poloidal array | x |  |  |
| Mirnov coils – toroidal array | x |  |  |
| MSE | x |  |  |
| Neutral particle analyzer |  | x |  |
| Neutron Rate (2 fission, 4 scint) |  |  |  |
| Neutron collimator |  |  |  |
| Plasma TV |  | x |  |
| Reciprocating probe |  |  |  |
| Reflectometer - FM/CW |  |  |  |
| Reflectometer - fixed frequency homodyne quadrature |  |  |  |
| Reflectometer - homodyne correlation |  |  |  |
| Reflectometer - HHFW/SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| Solid State NPA |  |  |  |
| SPRED |  |  |  |
| Thomson scattering - 20 channel | x |  |  |
| Thomson scattering - 30 channel |  | x |  |
| Ultrasoft X-ray arrays | x |  |  |
| Ultrasoft X-ray arrays - 2 color | x |  |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray PIXCS (GEM) camera |  |  |  |
| X-ray pinhole camera |  |  |  |
| X-ray TG spectrometer |  |  |  |