| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Divertor heat flux reduction and detachment in NBI-heated plasmas |  |  |  |
| OP-XP-605 | Revision: | Effective Date: (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Author: V. A. Soukhanovskii |  |  | Date |
| ATI – ET Group Leader: R. Maingi |  |  | Date |
| RLM - Run Coordinator: R. Raman (S. Sabbagh) |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

Divertor heat flux reduction and detachment in NBI-heated plasmas	OP-XP-605

# 1.	Overview of planned experiment

The goal of this experiment is to determine conditions of the outer divertor target heat flux reduction and detachment in the NBI-heated plasma operational space. The lower single null (LSN) plasma shape with elongation and triangularity =0.5 will be used. The highest peak outer divertor heat fluxes (q < 4-7 MW/m2) have been measured in this shape due to a low magnetic flux expansion (~ 3) at the outer strike point. Two techniques will be used to achieve the outer target heat flux reduction and detachment: raising density by means of D2 injections, and raising edge radiated power by injecting an extrinsic impurity. In the first part of the experiment deuterium will be injected in the lower divertor region. In the second part of the experiment deuterated methane will be injected in increased quantities to yield Prad/Pin up to 0.5 to obtain the outer target detachment. Divertor measurements, such as the D, D, D brightnesses, heat flux (from IR cameras), radiated power, divertor Langmuir probe Isat and neutral pressure will be monitored for signs of detachment.

# 2.	Theoretical/ empirical justification

Previous divertor experiments in NSTX [1,2] with deuterium and neon puffing demonstrated that the outer divertor peak heat flux can be reduced by 50-80 % and simultaneously H-mode confinement can be maintained. The experiments revealed a strong dependence of the plasma performance and divertor heat flux reduction on a gas puffing location. When deuterium is puffed in the midplane locations radiative divertor conditions are apparently attained, whereas if D2 is puffed in the lower divertor region a partially detached divertor state can be achieved. Midplane neon injections apparently lead to the radiative mantle type heat exhaust since NSTX divertor temperature in the heat flux limited regime was too low for neon to radiate effectively. This experiment will study divertor heat reduction and detachment further. The Branch 5 injector will be used to puff deuterium in the lower divertor region in a steady-state manner to further study the partially detached regime. Midplane CD4 injections will be used to increase divertor radiated power and achieve OSP detachment through divertor radiation simultaneously maintaining an H-mode confinement.

# 3.	Experimental run plan

- Setup an LSN (with PF2L coil) HFS-fueled plasma with elongation and triangularity =0.5 and perform gas injection rate scan. Obtain several conditions at low, intermediate and high plasma density, with emphasis on the high-density part (15 - 20 shots)

  - Setup a discharge with 2 NBI sources at full energy, adjust the X-point height so that the inner strike point remains at 1-2 cm from the inner wall throughout discharge.

  - Example shots: 116484, 116485, 116488 (0.7 MA, 0.4 T RtEFIT-controlled shot)

  - Use Branch 5 gas injector in FLO mode at several plenum pressures (1300 - 1800 T) to obtain injection rates 100 - 250 Torr l /s

  - Add LDGIS injectors in series to obtain one LGDIS gas puff (fill pressure 100 - 200 Torr) s to directly compare to Branch 5 results

  - In one high density discharge, turn off NBI at the time when ne is high (> 6 x 1019 m-3) to obtain high density low input power condition for about 50 ms

- Greenwald density for Ip=0.8 MA is nG = 5.5 x 1019 m-3

- (Conditional) In one discharge, turn off gas feed at 0.25-0.30 s

- (Conditional) Run a helium discharge to de-saturate walls if necessary

(Note: if the limit of choking the plasma with gas is reached, proceed to CD4 injections with configuration as in 1)

- Perform CD4 injections in increasing quantities (0.05 – 0.2 s duration pulses at a rate from 1 to 20 Torr l / s ) into intermediate density two NBI source shot from 1. Monitor radiated power (10-15 shots).

  - Use Injector 3 for CD4. Methane pulse start time 0.30 -0.35 s

  - Start with a CD4 pulse at the rate of 1 Torr l / s, pulse duration 50 ms

  - Increase CD4 injection rate to 20 Torr l / s in steps of 1-2 Torr l / s, increase the pulse duration from 50 ms to 200 ms, in accordance with plasma behavior and the flat-top length obtained (expect shortening due to increased plasma resistivity (Zeff) and higher ohmic flux consumption).  Monitor plasma radiated power

  - A 10 liter bottle of CD4 has been reserved for this experiment. The quantity of methane in the bottle is 10 liter x 350 psi x 51.71 Torr = 181000 Torr l. A modification of gas handling setup may be developed to reduce the loss of methane to pipe and plenum cycling and pumpout.

- Conditional, time permitting - perform CD4 injections at established in 2 rates into high density two NBI source shot from 1. Monitor radiated power.

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Physics Operations Request and Diagnostic Checklist are attached.

Diagnostic capabilities: Tile Langmuir probes, IR cameras, main plasma and divertor bolometers, and the D, D cameras should be operational. Lower divertor Langmuir probe locations are (major radii, m): 0.2775, 0.4952, 0.7970, 0.9110, 1.0170.

Branch 5 Injector in FLO mode must be tested and calibrated prior to the execution of this experiment.

# 5.	Planned analysis

The following numerical tools will be used for data analysis: EFIT04, LRDFIT, UEDGE, ADAS, DEGAS 2, TRANSP, analytic two point divertor model.

# 6.	Planned publication of results

Results will be presented at the upcoming PSI, ITPA and/or IAEA meetings, and will be published in a refereed journal as appropriate.

References

[1] V. A. Soukhanovskii;  R. Maingi;  C. J. Lasnier, et al. "Particle and power exhaust in NBI-heated plasmas in NSTX", 32nd EPS Conference on Plasma Phys. (Tarragona, Spain, June 2005), Europhys. Conf. Abstr., 29C, pp P-4.016 (2005).

[2] V. A. Soukhanovskii et. al., GO3.00009: Dissipative divertor experiments in NSTX, 2005 47th Annual Meeting of the Division of Plasma Physics, October 24–28, 2005; Denver, Colorado

PHYSICS OPERATIONS REQUEST

Divertor heat flux reduction and detachment in NBI-heated plasmas	OP-XP-605

Machine conditions (specify ranges as appropriate)

ITF (kA): -52.5	Flattop start/stop (s):  -0.02/1.0

IP (MA): 0.7-0.8	Flattop start/stop (s):  0.12/0.6

Configuration: Lower Single Null

Outer gap (m):	0.1,	Inner gap (m):	0.05-0.1

Elongation :	1.9-2.0,	Triangularity :	0.5

Z position (m):	0.00

Gas Species:  D / He / CD4,

Gas Injector:  Midplane / Inner wall / Lower Dome/Branch 5

NBI - Species: D,	Sources: A/B/C,	Voltage (kV): 80,	Duration (s): 0.6

ICRF – Power (MW): 0,	Phasing: Heating / CD,	Duration (s): _____

CHI:  Off

Either:	List previous shot numbers for setup: 116485, 116488

Gas setup: CS Injector – D2, LDGIS - D2, Injector 1 – He, Injector 2 – D2, Injector 3 - CD4,

Injector Bay B High Flow  - D2 or He for GPI

DIAGNOSTIC CHECKLIST

Divertor heat flux reduction and detachment in NBI-heated plasmas	OP-XP-605

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer - tangential array |  |  |  |
| Bolometer array - divertor |  |  |  |
| CHERS |  |  | Need in both deuterium and neon shots |
| Divertor fast camera |  |  | View the X-point region with Da filter |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitor |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation spectroscopy |  |  |  |
| Fast lost ion probes – IFLIP |  |  |  |
| Fast lost ion probes – SFLIP |  |  |  |
| Filtered 1D cameras |  |  | CAM1, CAM2: Da, CAM3: Db, CAM4: Dg |
| Filterscopes |  |  | Use neon filter |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  | Measure turbulence in neon shots |
| High-k scattering |  |  |  |
| Infrared cameras |  |  | Divertor lower div., CS |
| Interferometer – 1 mm |  |  |  |
| Langmuir probes - PFC tiles |  |  |  |
| Langmuir probes - RF antenna |  |  |  |
| Magnetics – Diamagnetism |  |  |  |
| Magnetics – Flux loops |  |  |  |
| Magnetics – Locked modes |  |  |  |
| Magnetics – Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency |  |  |  |
| Mirnov coils – poloidal array |  |  |  |
| Mirnov coils – toroidal array |  |  |  |
| MSE |  |  |  |
| Neutral particle analyzer |  |  |  |
| Neutron Rate (2 fission, 4 scint) |  |  |  |
| Neutron collimator |  |  |  |
| Plasma TV |  |  | Da filter when requested, no filter otherwise |
| Reciprocating probe |  |  | Operate on several shots when requested |
| Reflectometer - FM/CW |  |  |  |
| Reflectometer - fixed frequency homodyne quadrature |  |  |  |
| Reflectometer - homodyne correlation |  |  |  |
| Reflectometer - HHFW/SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| Solid State NPA |  |  |  |
| SPRED |  |  |  |
| Thomson scattering - 20 channel |  |  |  |
| Thomson scattering - 30 channel |  |  |  |
| Ultrasoft X-ray arrays |  |  |  |
| Ultrasoft X-ray arrays - 2 color |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| Visible spectrometers (VIPS) |  |  | Chords: inner, outer divertor and CS |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray PIXCS (GEM) camera |  |  |  |
| X-ray pinhole camera |  |  |  |
| X-ray TG spectrometer |  |  |  |