| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: X-point height and H-mode Power threshold |  |  |  |
| OP-XP- | Revision: | Effective Date: Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: K. C. Lee |  |  | Date 06/06/07 |
| ATI – ET Group Leader: V. Soukhanovskii |  |  | Date |
| RLM - Run Coordinator: D. Gates |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

TITLE:  X-point height and H-mode Power threshold	No.  OP-XP-

AUTHOR: K. C. Lee	DATE: 06/06/07

# 1.	Overview of planned experiment

It is found that when the x-point height is lower the H-mode access is easier on NSTX plasmas. Comparing #117747 (low x-point) with #123644 (high x-point), low x-point case generated L/H transition at 1 MW NBI heating power while high x-point case didn’t make H-mode, even with up to 6 MW NBI power (in a nearby discharge). This evidence appeared repeatedly in a series of shots. While reducing the X-point height has been known to reduce the power threshold in tokamaks, the role of the physical distance between the X-point (related presumably to neutral physics) and the connection length (related to electron/ion physics) needs clarification, particularly since reducing the X-point height also reduces the connection length at constant Ip and Bt.

One candidate of interpretation for this phenomenon is the gyrocenter shift theory (discussed below) in which a narrow gap between the x-point and diverter strike point increases the neutral density gradient so that the induced radial electric field becomes higher , leading to a reduced H-mode power threshold. The goal of this XP is to measure the power threshold at two different X-point heights and two different plasma currents (connection length variation) to document which is more important in setting the value of PLH. To test the gyro-center shift theory, the neutral density profile in the X-point region must be measured during the power threshold determination in the four conditions described above.

# Theoretical/ empirical justification

The gyrocenter shift theory appeared as one of the most important mechanisms of H-mode transition [K.C. Lee, Phys. of Plasmas, (2006)]. To verify the effect of gyrocenter shift the measurement of neutral density profile is required. On DIII-D the neutral density profile study was carried out using diverter D-alpha camera and x-point tangential D-alpha camera and diverter Thomson scattering with analysis of emission dependency on plasma parameters [Colchin, Maingi et al.,Nuclear Fusion (2000)]. Using this technique, it was found in DIII-D that the neutral density gradient in the low x-point case was higher than the high x-point case.

- NSTX has capability to measure the neutral density profiles for same study which was done on DIII-D, assuming that the midplane Thomson can be mapped to the X-point region (this is reasonably valid for the low density L-mode phase preceding the L-H transition).

- NSTX has mid plane D-beta camera and edge rotational diagnostics to measure the neutral density and edge rotation profiles just before the L-H transition.

- NSTX will be the first machine to investigate the H-mode power threshold in conjunction with the gyrocenter shift theory by measuring the neutral density gradient at variable X-point height.

- The run plan includes verification of x-point height and effect of connection length by scanning of plasma current.

# 3.	Experimental run plan

The standard shot will be #117747 (600 kA, 4.5 kG, with NBI power increase; 1MW, 2MW and 4MW, low x-point case). A more recent version of this shot with rtEFIT control is 124641 from 6/5/2007. However the NBI timing from 117747 will be used.

  - Start with low x-point height, Power threshold measurement with all diagnostics (4-5 shots). To localize the power threshold, we will need 1 src as low as it goes (historically source C at ~ 56 kV), but source B at 60 kV would be fine also. The second source (B or C) would be set to ~ 1.2 MW, and source A will be at full voltage for MSE, i.e. at 2 MW.

  - H-mode Power threshold measurement with high x-point and high Ip~ 0.8 MA i.e. 123644 at slightly lower Ip, except use Bt=0.45 T for consistency with step 1. From past experience, it is likely that this discharge will not go into H-mode at all with large drsep. This is the shortest connection length (2-3 shots)

  - Decrease Ip to 0.6 MA to increase connection length with high X-point, and measure PLH. From past experience, it is likely that this discharge will not go into H-mode at all with large drsep.

  - Repeat #1 with 0.8 MA (4-5 shots)

Below is a table with the conditions, and the anticipated H-mode access (O = yes, X = no).

|  | Low x-point | High x-point |
| --- | --- | --- |
| Low Ip ~ 0.6 MA (large lc) | O | X |
| High Ip ~ 0.8 MA (small lc) | O | X(?) |

Total: 14 shots

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

3 sources of NBI with B and C at reduced voltages, one for ~ 600-800 kW, and the other for 1.2 MW Source A at nominal voltage.  See diagnostics table for diagnostics required.

# 5.	Planned analysis

D-alpha emission analysis will need to be developed. This means implementing an algorithm similar to the one used for tangential TV analysis to convert the 3-D view to a poloidal cut, assuming toroidal symmetry. Alternately analysis of the emission profiles with DEGAS-2 tools may lead to a direct estimate of the neutral density gradient scale length as needed for the theory calculation.

The gyrocenter shift calculation will be done with the best profiles available.

# 6.	Planned publication of results

Results will be submitted to Plasma Physics and Controlled Fusion or similar Journals

PHYSICS OPERATIONS REQUEST

Title: X-point height and H-mode Power threshold	OP-XP-

Machine conditions (specify ranges as appropriate)

ITF (kA): 4.5 kG	Flattop start/stop (s): *

IP (MA): 0.6-0.8 MA	Flattop start/stop (s): *

Configuration:

Outer gap (m): *		Inner gap (m): *

Elongation : *		Triangularity : *

Z position (m): *

Gas Species: D2	Injector(s): *

NBI - Species: D	Sources: all	Voltage (kV): A at 90, B or C at 70, 60 respectively	Duration (s): *

ICRF – Power (MW): no	Phasing: n.a.	Duration (s): n.a

CHI: no

Either:	List previous shot numbers for setup: (*) 124641

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

Title: ELM characterization and the effect of Lithium	OP-XP-

| Diagnostic | Need | Desire | Instructions |
| --- | --- | --- | --- |
| Bolometer – tangential array | X |  |  |
| Bolometer – divertor |  | X |  |
| CHERS – toroidal | X |  |  |
| CHERS – poloidal |  | X |  |
| Divertor fast camera | X |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitors |  |  |  |
| Edge pressure gauges | X |  |  |
| Edge rotation diagnostic | X |  |  |
| Fast ion D_alpha - FIDA |  | X |  |
| Fast lost ion probes - IFLIP |  |  |  |
| Fast lost ion probes - SFLIP |  |  |  |
| Filterscopes | X |  |  |
| FIReTIP | X |  |  |
| Gas puff imaging | X |  |  |
| H camera - 1D | X |  |  |
| High-k scattering |  |  |  |
| Infrared cameras |  | X |  |
| Interferometer - 1 mm |  |  |  |
| Langmuir probes - divertor | X |  |  |
| Langmuir probes – RF antenna |  | X |  |
| Magnetics – Diamagnetism | X |  |  |
| Magnetics - Flux loops | X |  |  |
| Magnetics - Locked modes |  | X |  |
| Magnetics - Pickup coils | X |  |  |
| Magnetics - Rogowski coils | X |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency | X |  |  |
| Mirnov coils – poloidal array | X |  |  |
| Mirnov coils – toroidal array | X |  |  |
| MSE |  | X |  |
| NPA – ExB scanning |  |  |  |
| NPA – solid state |  |  |  |
| Neutron measurements |  |  |  |
| Plasma TV | X |  |  |
| Reciprocating probe |  | X |  |
| Reflectometer – 65GHz |  | X |  |
| Reflectometer – correlation |  |  |  |
| Reflectometer – FM/CW |  |  |  |
| Reflectometer – fixed f |  |  |  |
| Reflectometer – SOL |  |  |  |
| RF edge probes |  |  |  |
| Spectrometer – SPRED |  | X |  |
| Spectrometer – VIPS |  | X |  |
| SWIFT – 2D flow |  |  |  |
| Thomson scattering | X |  |  |
| Ultrasoft X-ray arrays |  | X |  |
| Ultrasoft X-ray arrays – bicolor |  |  |  |
| Ultrasoft X-rays – TG spectr. |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray fast pinhole camera |  |  |  |