| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Assess Affect of Alfvén Cascades on Fast ion transport |  |  |  |
| OP-XP-808 | Revision: | Effective Date: Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: Neal Crocker |  |  | Date |
| ATI – ET Group Leader: |  |  | Date |
| RLM - Run Coordinator: |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Assess Affect of Alfvén Cascades on Fast ion transport | No. OP-XP-808 |
| --- | --- |
| AUTHORS: Neal Crocker | DATE: Mar 14, 2008 |

# 1.	Overview of planned experiment

This XP will devote approximately one half the run time towards documenting fast ion redistribution by Alfvén Cascades (AC) modes and the second half towards using the AC modes to measure the effective specific heat of the thermal electrons, thermal ions and beam ions, identify the bfast threshold for AC excitation and separate b and b’ role in AC frequency sweep suppression.  Discharges with AC modes from 2006 (120106, 120107 and 120109) show evidence of enhanced fast ion losses during the frequency sweep period of n=3 an AC mode.  We propose to duplicate this shot and use the new FIDA diagnostic, as well as an NPA scan to document to the extent possible the pitch angles and energies of the lost/redistributed fast ions.  In parallel with the NPA scan, there will be a radial scan of the scattering volume for the high-k scattering diagnostic to look for coupling of the AC to Kinetic Alfvén Waves (KAW).

The minimum in the frequency sweep of the AC modes, c.f., the n=2 mode in 123096 at 300 ms, is believed to be the GAM frequency.  This frequency depends on the mass density and effective gammas of the thermal and fast ion populations.  By reducing the beam power, we can reduce the contribution of the fast ions to this frequency, and the scaling of the frequency should give a measurement of the effective fast ion gamma, and thus, also the effective gamma for the thermal ion population.  This power scan will incidentally also give the bfast threshold for AC excitation.  HHFW heating of the electrons, and triggering an H-mode, with flat density profile, will help separate wmin dependence on thermal and fast ion components as well as separate pressure from pressure gradient terms.

W. Lee will piggy back on these experiments to look for the mode conversion to Kinetic Alfvén waves (KAW) at the radii of the intersection of the AC modes with the continuum with the high-k scattering diagnostic (his thesis project).

If additional time remains,  we would explore one or more of the following important issues.  Can AC modes be excited in H-mode plasmas?  Does reduction of the fast ion contribution to beta, perhaps at higher field, allow for AC mode excitation at higher density operation (e.g., with better reflectometer data).  In such a plasma, can we document the spatial profile of the AC modes, perhaps demonstrating the conversion from AC to TAE?  What is the structure of the AC mode during the downward frequency sweep?  What additional information can we acquire regarding the 2nd Gap AC modes?

# 2.	Theoretical/ empirical justification

Alfvén Cascade or rsAE modes have been studied on many conventional tokamaks.  They are found to be an accurate indicator of the qmin(t).  Their presence in reverse-shear plasmas on DIII-D, in plasmas with neutron deficits, has also raised suspicion that they may enhance fast ion transport.  Until recently, AC modes were absent from reverse shear plasmas in STs, and a theoretical explanation was developed which predicted that AC modes would only be present at very low (by ST standards) beta.

Alfvén Cascade-like modes were discovered at very low beta in ultra-low density plasmas during XP607 in the ’06 campaign.  The predicted suppression at high electron beta was observed. Experiments in ’07 acquired MSE data and confirmed identification of modes as Alfvén Cascades.  A return to this experiment would allow for more complete documentation of the beta-threshold, as well as investigation of their role in fast ion redistribution.  The expanded array of reflectometer channels might also make possible the documentation of the expected change in mode structure as the frequency chirp saturates.

# 3.	Experimental run plan

Run starts with attempt to reproduce AC spectrum and sFLIP losses as seen on shots 120106, 120107 or 120109.  This most likely requires very precise replication of current profile evolution, density evolution and possibly He fraction, monitored by comparison of neutron rates.  All proposed shots will include early source A for TRANSP q-profile initialization, if it doesn’t adversely impact AC mode spectrum evolution.

The measurement of fast ion and thermal geff will be done in part 1.  In Part 2 the toroidal field is increased to 5.5 kG to increase the range of the Cascade frequency sweeps.

Part 3 will address measurement of the internal mode structure of AC and TAE modes by attempting to increase target density at higher field, lower fast ion beta.

Part 4 will extend AC operational regime to H-mode plasmas, or at least minimize uncertainties in interpretation of neutron data by reducing He concentration.  The flat density profiles in H-mode will reduce the b’, leaving only the frequency shift from temperature gradient and gradient of fast ion density. The substitution of Deuterium for Helium should increase the thermal ion beta (twice the ion density for the same electron density).

In Part 5, HHFW will be used to increase the electron temperature to determine the dependence of minimum frequency on electron b.

Part 6 will study effects of rotation/rotational shear on AC mode stability (Joule milestone).

The affect of AC modes on fast ion transport will be documented in the second half of the experiment.  This phase requires remotely steered high-k scattering, available in April?.  The NPA scan (part 2) will include High-k scattering radial scan of scattering volume and some scan of the spectrum for the fast FIDA system.  If sFLIP losses can’t be reproduced, then FIDA, NPA and High-k scans should be moved to Part 5 in the hope that we’ll have good mode amplitude and mode structure data to interpret fast ion redistribution and high-k scattering data.

0.	Reproduce density, current, beams, neutron rate evolution for shot 120106 (123096).

1.	DO I=1,7

Reproduce density, current, beams, neutron rate evolution for shot 120106.

IF(replication of AC spectrum, sFLIP measurement of losses) THEN GOTO 2.

c…Scan of bfast to determine scaling of AC minimum frequency (g).

ENDDO

GOTO 3.

2.	Introduce source A to 150 ms to provide early measurement of q-profile

IF (AC spectrum, fast ion losses NOT reproduced) THEN revert to no early source A shot.

DO I=1,7

Vertical NPA scan and High-k scattering radial scan.  FIDA scan.

ENDDO

GOTO 3

c…Document fast ion redistribution, FIDA spectral scan, look for KAW

c…Introduce HHFW later in shot to prepare for Part 7.’

c…total 7 – 15 shots

3.	DO I=1,3 (While AC modes present)

Reduce beam voltage in 10 kV increments (80 kV, 70 kV,  60 kV)

ENDDO

c…Scan of bfast to determine scaling of AC minimum frequency (g).

IF (AC modes become stabilized) THEN

Increase beam voltage by 5 kV

END

IF (AC modes become stabilized) THEN

DO I=1,3

Mini-NPA/FIDA documentation of quiescent plasma?

END

Increase beam voltage by 5 kV

END

4.	Increase toroidal field to 5.5 kG

IF (AC modes stable) THEN

Reduce beam voltage by 5 kV, look for AC return

ENDIF

c…Higher field, lower beam power may increase density threshold for suppression

c…≈5 shots for 3&4

5.	DO I=1,3 (at 5.5 kG)

Density scan at lowest beam voltage shot with good AC modes up to “b-suppressed” density.

c…Get better reflectometer data on AC mode structure

c…Determine b scaling of minimum frequency

ENDDO

DO I=1,2

Move source A ‘on’ time back to t1, t2 < 300ms (to be determined) to document q evolution.

ENDDO

c…couple of points to document q-profile evolution

6.  	DO I=1,3

Replace He prefill and puffing with D prefill and puffing, maintain low density

ENDDO

c…Attempt AC modes in H-mode plasma, measure fast ion redistribution with FIDA

c…Reduce b’ to determine role in minimum frequency

IF(L-mode) THEN

Add/extend source A

ENDIF

DO I=1,2

Move source A ‘on’ time back to t1, t2 < 300ms to be determined to document q evolution.

ENDDO

c…≈10 shots for 5&6

7.	Add as much HHFW to heat electrons from 150ms to 300ms.

DO I=1,2

Move source A ‘on’ time back to t1, t2 < 300ms to be determined to document q evolution.

ENDDO

c…Increase electron b relative to other terms, sort out electron gamma

8.	Set n=3 non-resonant braking to 800A

DO I=1,3

Choose moderate density AC shot, increase n=3 braking in 150A increments

ENDDO

c…Stop rotation to minimize rotational shear effects

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

FIDA/NPA/sFLIP/ssNPA required.  MSE is a must.  N=3 braking capability is desired. 5-channel reflectometers and two “correlation reflectometers” set to intermediate frequencies.  Fast Mirnov, sources A and C required.  Remotely steered high-k scattering req’d.

# 5.	Planned analysis

Apart from the initial and simple analysis of the frequency chirping, sufficient data should have been acquired for careful NOVA modeling of the Cascade mode chirps, comparison of Nova radial eigenfunctions to multipoint reflectometer data, and possibly Orbit estimates of the effect of the Cascades on fast ion confinemnt.

# 6.	Planned publication of results

I would envision one or more papers on AC impact on fast ion transport, experimental measurement of fast ion gamma and contributions to a thesis on coupling to KAW.

PHYSICS OPERATIONS REQUEST

| TITLE: Assess Affect of Alfvén Cascades on Fast ion transport | No. OP-XP-8-- |
| --- | --- |
| AUTHORS: Neal Crocker | DATE: Mar 14, 2008 |

Machine conditions (specify ranges as appropriate)

ITF (kA): 34-65	Flattop start/stop (s): 0.0/1.0

IP (MA): 0.8	Flattop start/stop (s): 0.2/0.9

Configuration: 123096

Outer gap (m): 		Inner gap (m):

Elongation :		Upper/lower triangularity :

Z position (m):

Gas Species: He,D		Injector(s):  Midplane/Inner wall

NBI Species: D	Sources: 	ABC	Voltage (kV): 90/60-90/60-90  Duration (s): 0.9s

ICRF Power (MW): 2-3 MW	Phasing: 	180º	Duration (s): 0.02

CHI:	Off	Bank capacitance (mF):

LITER: Off

Shot numbers for setup: 124764

DIAGNOSTIC CHECKLIST

| TITLE: Fast ion transport induced by Alfvén avalanches | No. OP-XP-8-- |
| --- | --- |
| AUTHORS: Neal Crocker | DATE: Mar 12, 2008 |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array |  |  |
| Bolometer – divertor |  |  |
| CHERS – toroidal |  |  |
| CHERS – poloidal |  |  |
| Divertor fast camera |  |  |
| Dust detector |  |  |
| EBW radiometers |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  |  |
| Edge pressure gauges |  |  |
| Edge rotation diagnostic |  |  |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes |  |  |
| FIReTIP |  |  |
| Gas puff imaging |  |  |
| Ha camera - 1D |  |  |
| High-k scattering |  |  |
| Infrared cameras |  |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  |  |
| Langmuir probes – BEaP |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – Diamagnetism |  |  |
| Magnetics – Flux loops |  |  |
| Magnetics – Locked modes |  |  |
| Magnetics – Pickup coils |  |  |
| Magnetics – Rogowski coils |  |  |
| Magnetics – Halo currents |  |  |
| Magnetics – RWM sensors |  |  |
| Mirnov coils – high f. |  |  |
| Mirnov coils – poloidal array |  |  |
| Mirnov coils – toroidal array |  |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE |  |  |
| NPA – ExB scanning |  |  |
| NPA – solid state |  |  |
| Neutron measurements |  |  |
| Plasma TV |  |  |
| Reciprocating probe |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – SPRED |  |  |
| Spectrometer – VIPS |  |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering |  |  |
| Ultrasoft X-ray arrays |  |  |
| Ultrasoft X-rays – bicolor |  |  |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. |  |  |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer - XEUS |  |  |