| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Effect of Rotation on Energy and Impurity Confinement |  |  |  |
| OP-XP-812 | Revision: | Effective Date: Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: S. Kaye, L. Delgado-Aparicio |  |  | Date |
| ATI – ET Group Leader: S. Kaye |  |  | Date |
| RLM - Run Coordinator: M. Bell |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Effect of rotation on energy and impurity confinement | No. OP-XP-812 |
| --- | --- |
| AUTHORS: Kaye & Delgado-Aparicio | DATE: 2/1/08 |

# 1.	Overview of planned experiment

# The goal of this experiment is to study the effect of varying rotation on energy and impurity confinement and transport in steady H-modes. Steady n=3 braking  fields will be used to vary the plasma rotation.

# 2.	Theoretical/ empirical justification

ExB shear could have profound effect on local turbulence and transport, and rotation and shear can be varied using the n=3 braking. There were attempts to do this last year in dedicated scans, but MHD activity led to plasma distress, and the changes in transport properties could not be assessed properly. A subsequent NTM experiment used the n=3 braking fields without destabilizing MHD immediately, and these discharges will be used as a basis for this year’s experiment. This XP addresses one of the elements of the 2008 Joule milestone.

# 3.	Experimental run plan

# Attempt to attain MHD quiescent H-mode condition with no Lithium, shot 123848 (2-4 shots); no n=3 braking (decrease Ip to 0.9 MA, increase BT to 5.5 kG)

# If this fails, attempt MHD quiescent condition with 121154 (2-4 shots); no n=3 braking

# If failure in ½ day: Cut losses and move on

# Return later in no-Li portion of run, or with Li

# If successful in either condition

# Vary n=3 fields in 4 to 5 steps

# Do binary search in n=3 coil current starting at 600 A (increase to 1.2 kA, etc)

# Ensure no large islands at any n=3 current (SXR, profiles)

# 3 shots/condition: no impurity injection, impurity injection @ 400 ms, no impurity injection

-	Additional impurity injection shot in one or two (best) cases to check reproducibility

# 15-21 shots

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Shot development needed to attain MHD quiescent H-mode condition

# 5.	Planned analysis

LRDFIT, TRANSP, NTV analysis (IPEC), specialized codes

# 6.	Planned publication of results

Joule milestone, TTF, IAEA, PRL (?)

PHYSICS OPERATIONS REQUEST

| TITLE: Effect of rotation on energy and impurity confinement | No. OP-XP-812 |
| --- | --- |
| AUTHORS: Kaye and Delgado-Aparicio | DATE: 2/1/08 |

Machine conditions (specify ranges as appropriate)

ITF (kA): 64 kA (5.5 kG)	Flattop start/stop (s):

IP (MA): 0.9 MA	Flattop start/stop (s):

Configuration: LSN

Outer gap (m): 		Inner gap (m):

Elongation : 2.3		Upper/lower triangularity : 0.8

Z position (m):	0

Gas Species: D, Neon	Injector(s):

NBI Species: D	Sources: 	3	Voltage (kV): 	80, 90	Duration (s): full shot

ICRF Power (MW):	0	Phasing: 		Duration (s):

CHI:	Off	Bank capacitance (mF):

LITER:  Off (during initial attempt at XP)

Either:	List previous shot numbers for setup:	123848, 121154

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

| TITLE: Effect of rotation on energy and impurity confinement | No. OP-XP-812 |
| --- | --- |
| AUTHORS: Kaye and Delgado-Aparicio | DATE: 2/1/08 |

| Diagnostic | Need | Want | Conditions |
| --- | --- | --- | --- |
| Bolometer – tangential array | x |  |  |
| Bolometer – divertor |  |  |  |
| CHERS – toroidal | x |  |  |
| CHERS – poloidal | x |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitors |  |  |  |
| Edge neutral density diag. |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation diagnostic |  | x |  |
| Fast ion D_alpha - FIDA | x |  |  |
| Fast lost ion probes - IFLIP |  |  |  |
| Fast lost ion probes - SFLIP |  | x |  |
| Filterscopes | x |  |  |
| FIReTIP |  | x |  |
| Gas puff imaging |  | x |  |
| H camera - 1D |  |  |  |
| High-k scattering |  | x |  |
| Infrared cameras |  |  |  |
| Interferometer - 1 mm |  |  |  |
| Langmuir probes - divertor |  |  |  |
| Langmuir probes – RF ant. |  |  |  |
| Magnetics – Diamagnetism | x |  |  |
| Magnetics - Flux loops | x |  |  |
| Magnetics - Locked modes | x |  |  |
| Magnetics - Pickup coils | x |  |  |
| Magnetics - Rogowski coils | x |  |  |
| Magnetics - RWM sensors | x |  |  |

| Diagnostic | Need | Want | Conditions |
| --- | --- | --- | --- |
| Mirnov coils – high f. | x |  |  |
| Mirnov coils – poloidal array | x |  |  |
| Mirnov coils – toroidal array | x |  |  |
| MSE | x |  |  |
| NPA – ExB scanning |  |  |  |
| NPA – solid state |  | x |  |
| Neutron measurements | x |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer – 65GHz |  |  |  |
| Reflectometer – correlation |  |  |  |
| Reflectometer – FM/CW |  |  |  |
| Reflectometer – fixed f |  | x |  |
| Reflectometer – SOL |  |  |  |
| RF edge probes |  |  |  |
| Spectrometer – SPRED | x |  |  |
| Spectrometer – VIPS |  |  |  |
| SWIFT – 2D flow |  |  |  |
| Thomson scattering | x |  |  |
| Ultrasoft X-ray arrays | x |  |  |
| Ultrasoft X-rays – bicolor | x |  |  |
| Ultrasoft X-rays – TG spectr. |  |  |  |
| Visible bremsstrahlung det. |  | x |  |
| X-ray crystal spectrom’r - H |  |  |  |
| X-ray crystal spectrom’r - V |  |  |  |
| X-ray fast pinhole camera |  |  |  |
| X-ray spectrometer - XEUS |  | x |  |