| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Flux savings from inductive drive of a Transient CHI started plasma |  |  |  |
| OP-XP-8?? | Revision: 0 | Effective Date: 2/11/08 (Ref. OP-AD-97) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible author: R. Raman et al. |  |  | Date |
| ATI – ET Group Leader: R. Raman |  |  | Date |
| RLM – Run Coordinator: M. Bell |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

Title: Flux savings from inductive drive of a Transient CHI started plasma	OP-XP-8??

Authors: R. Raman, B.A. Nelson, D. Mueller, M. Bell, T.R. Jarboe

# 1.	Overview of planned experiment

The run plan from X726 will be improved for coupling a transient CHI discharge to induction by using improved lower divertor surface conditions and relying on well established current ramp-up conditions from a pre-charged central solenoid discharge.

# 2.	Theoretical/ empirical justification

Experimental results on the HIT-II Experiment have shown Transient CHI produced discharges to couple to and improve the performance of inductive only discharges. Recently NSTX has produced high levels of closed flux current using Transient CHI. The present XP will optimize these discharges by reducing the extent of absorber arcing using the staged capacitor bank system.. This XP will increase the current in the CHI produced discharge using induction from the central solenoid to produce a long-pulse discharge and measure flux savings.

This XP has minor modifications to part 3.1 of XP726 and an improved run plan (part 3.2) that uses well established discharges from a pre-charged central solenoid discharge.

# 3.	Experimental run plan

After the vessel boronization described in Section 4, first do parts 3.2-2 and 3.2-3. Then do 3.1 and switch to 3.2 if progress is not rapid enough with procedure 3.1.

3.1 Hand-off to inductive operation using zero pre-charge in the central solenoid

- Restore a reference CHI produced discharge from XP726 (No. 124271).

- After divertor surface conditioning, as described under Section 4, reproduce the CHI only portion of the discharge by using 3 capacitor banks. Using the staged bank capability discharge a second capacitor bank (with one or two modules) and third module (one or two modules) as required with about 0.5ms time delays between them to extend the decay portion of the CHI produced toroidal current.

- Close the OH circuit line switches and enable the OH power supply. Starting from the time of peak toroidal current, apply the pre-programmed OH wave form from shot 124271.

The increase in plasma current may be accompanied by vertical motion of the plasma. Initially, sdjust the currents in PF2/3 coils to keep the plasma positioned at z=0. Using the ratios:

IPF3/Ip = –2.4kA/100kA and

IPF2/Ip = +1.2kA/100kA.

Program the current in PF5 approximately in the ratio,

IPF5/Ip = –0.3kA/100kA, to provide radial equilibrium.

Later, Use between-shot EFIT reconstructions to adjust the currents in PF3 and PF2 coils.

- Using Injector 2 or the SGI inject at a rate of about 100 Torr.L/s from 10 to 20ms, to ensure discharge has sufficient electron density. Based on the resulting current ramp rates and the Thomson electron density profile adjust the gas injection conditions to maintain sufficient electron density in the discharge.

- Use pre-programmed current ramp-up in the OH coils for 30-50ms.

- During the pre-programmed phase increase or decrease the applied loop voltage to maintain a positive current increase during the 30-50ms current ramp.

- After plasma reproducibly ramps up during the preprogrammed phase, turn on the normal feed-back control of the equilibrium and plasma current using the PCS after Ip reaches 100-150kA and after imposing a time delay of at least 25ms from start time of current in the OH coil.

- ramp to take over at a suitable current level, which may vary from 50 to 150 kA.

- During this phase ramp the current to the level allowed by the flux in the central solenoid.

- After the plasma current exceeds 200kA, turn on NBI SRC A at 90kV for the duration needed for a MSE measurement.

- Add additional beam sources to maximize the total current that can be produced using this condition.

- If induction is not able to sustain the CHI-induced current, either:

  - Apply a 40ms, 1 – 2 MW pulse of HHFW power phased for plasma heating (0-π), or

  - If the CHI-induced plasma current is above 100kA and the edge neutral pressure is below 5E-5 Torr (\ENGINEERING::GS_IG_TORR) during the current decay, apply a 50 ms pulse of NBI at 40-50 keV, initially from one source and adding sources if needed.

No-CHI Comparison

After reliable, reproducible discharges are obtained conduct a no-CHI comparison by:

1) Turning of the CHI Power Supply voltage (this will most likely not allow an inductive breakdown because of a poor field null)

2) Turn off the PF1B, PF2L and PF1AL coil current used for injector flux programming (i.e. from 0 to 20ms).

3) If step (2) still does not produce an inductive breakdown, turn off all of the other coil current during the break-down phase and begin ramping these coils 5-10ms after breakdown is achieved.

NOTE: The EC-Pi needs to be configured for mid-plane injection instead of Lower Divertor injection.

3.2 Hand-off to inductive operation using full pre-charge in the central solenoid

- The advantage of this procedure (which may have easier success) is that the conditions needed for inductive ramp are part of routine NSTX operations. The required change is to add a CHI initial condition so that current generation is not dependent on good quality field null. Additionally, CHI will be able to produce an initial toroidal current that the subsequent inductive ramp would use.

- Restore inductive discharges 120404 (OH at 4kV, No PF2, with PF1B – LSN, single NBI source) and 120406 (OH at 4kV, No PF2, No PF1B – DN, 3 NBI sources) to see which performs better.

- Select a shot and increase TF to 65kA and repeat.

- Add the PF2L, 2U, 1B, 1AL, 1AU waveforms from CHI shot 124271 and repeat once or twice without CHI to assess the sensitivity of the field null. Move all the CHI waveforms back by 5ms so that CHI discharge can be started at 0ms instead of the usual +5ms.

- No add the PF1B waveform from 124271, change gas injection from mid-plane to Branch-5 LDGIS and repeat with a single module (2 caps) charged to 1.5kV. Note that PF1B and Branch-5 timings and the cap bank trigger time and dwell times need to be moved back by 5ms. Note that the CHI voltage phase would be over by about 6 or 7ms (during the preprogrammed inductive phase).

- Initially turn off the mid-plane gas injection that begins at 14ms. After the initial results, turn it back on. Similarly, turn off the NBI injection and turn it back on after sufficient current is generated.

- If no current is generated (fast camera does not show plasma pulling into vessel), increase the voltage up to the maximum of about 1.9kV (in steps of 100V) and repeat.

- If current is still not generated, add additional capacitor bank modules (2+1, 2+3, 2+3+1) and try at voltage of 1.6, 1.7, 1.8 and 1.9kV.

- If current is still not generated or there are indications of impurity injection, reduce the magnitude of the CHI injector flux to 75% and if necessary to 50% of the value used in 124271 and repeat steps 5 to 8.

- After successful discharges are obtained, move the start of the inductive loop voltage application time as forward in time as possible, and extend the subsequent discharge to a long-pulse CHI started reference discharge.

3.3	Assessment of Inductive Flux Saving

- After reliable, reproducible discharges are obtained, repeat 5 shots using CHI start-up assist.

- For procedure 3.1, develop an inductive startup discharge which reaches the same current and configuration in the same time as the CHI-assisted discharges in 3.1. Note that since the OH power supply voltage will be limited to 4kV for CHI experiments, some discharge development will be needed. Run 5 shots with inductive-only start-up to measure the poloidal flux consumption under identical current ramp-up conditions. Determine differences in density profile, current profile and temperature profile evolution.

- For procedure 3.2 suitable comparison shots will have been produced during the initial start of step 3.2

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

NOTE: Do NOT conduct “hi-pot” tests of any of the NSTX vacuum vessel components above 2kV with either the CHI supply capacitors or the CHI snubber capacitors connected to NSTX, as these capacitors have a rating of only 2kV.

NOTE: Ensure LDGIS interlocks are in the “green” state prior to conducting any “hi-pot” tests.

NOTE: The trained personnel identified in the procedure for changing the capacitor bank configuration should be available during the initial phase of CHI operations when the capacitor bank size is being changed.

The Phantom fast camera is required for this experiment.

Configure one of the Soft X-ray arrays that views the central plasma to be in bolometer mode.

Good machine wall conditions are required.

The vessel should be boronized after the end of Day-1 CHI operations as described below.

Capacitor Bank Configurations:

1. For Lower divertor plate conditioning (Day 1 of a run sequence)

Note: All MOVs to be connected during this operation

Bank-1: 3 capacitors, Bank 2: 1capacitors, Bank3: 6 capacitors

2. For Normal Operations (Day 2 and later of a run sequence)

Note: Use the MOV conditions required for 6 capacitor modules

Bank-1: 3 capacitors, Bank 2: 1capacitors, Bank3: 2 capacitors

On the first day of CHI operations, the lower divertor plates should be conditioned as follows.

1. Load shot 124271 (but with the inductive portion turned off) and repeat the discharge with 4 capacitors and diagnose using Thomson and Spectroscopy (few shots).

2. Now convert this to a stuffed injector flux condition case by increasing the injector flux with respect to the toroidal field (increase PF1B, 2L, 3L coil currents) by 50% and repeat with Bank 1 operated at 5ms, Bank2 at 5.5ms and Bank 3 at 6ms. During this conditioning phase, on different shots, vary the divertor coil currents in either direction by about 25% to sweep the current carrying  divertor flux footprints over a larger area of the lower divertor plates. Typically operate at a charging voltage of 1.75kV.

3. After 15 discharges, repeat step 1 to assess machine conditions.

4 Repeat steps 2 and 1, at-least twice more to remove as much surface deposits as possible from the lower divertor plates.

Configuration

- Connect the CHI capacitor to the CHI bus at the machine and connect the snubber capacitor and the MOV protection devices.

- Reduce the maximum voltage capability on the OH circuit to 4kV (from the normal 6kV).

- Configure PF1B for 2kV operation to drive current in the anti-clockwise direction as seen from above (normal for CHI). Change PF1B RIS and software limits to provide 10kA capability (from the present 5kA).

- Configure PF2L and PF2U for 2kV operation, with coil current in the clockwise direction (opposite of normal operation).

- Configure PF4 to have the same polarity as PF5

- Enable the EC-PI klystron #2 to provide the divertor chamber preionization and disable klystron #1 connected to the midplane EC-PI launcher. Apply the EC-PI power from t = -3 to +8ms.

- Configure HHFW for 0-π phasing during heating, if required.

- Operate the NB sources at >80kV acceleration voltage during the current ramp-up phase.

LDGIS system checks

- Ensure LDGIS is in the “Puff” mode and has been tested prior to the experiment.

- Ensure LDGIS gas pressure interlocks (PE107, PE104) are functional.

- Set LDGIS operating line pressure to 100Psig helium.

- Ensure 2000 Torr D2 filling gas pressure in LDGIS system

Pre-operational checklist

- Ensure MIG and Penning gauges are selected for operation.

- Remove CHI jumpers.

- Check TF interlock is set to 10kA.

- Check for the appearance of ionization light on the fast camera during the EC-PI pulse to ensure correct triggering.

# 5.	Planned analysis

The magnetic analysis codes LRDFIT and EFIT will be used to analyze the plasma equilibrium. It is hoped to be able to use data from the MPTS and other kinetic diagnostics to supplement the magnetic data for the assessment of flux closure.

# 6.	Planned publication of results

These results will be published at meetings and in journal articles.

PHYSICS OPERATIONS REQUEST

Title: Flux savings from inductive drive of a Transient CHI started plasma	OP-XP-8??

Machine conditions (specify ranges as appropriate)

ITF (kA): -41 to -65	Flattop start/stop (s):  -0.02 / 0.7

IP (MA): 1.0	Flattop start/stop (s):  n/a

Configuration: CHI transitioning to CS driven with NBI & HHFW

Outer gap (m):	>8cm	Inner gap (m):	>2cm

Elongation :	>2	Triangularity :	>0.4

Z position (m):	0

Gas Species:  D,	Injector:  LDGIS + LFS midplane

NBI-	80kV to 90kV D0 injection, Nominal Pulse during inductive drive

ICRF-	Up to 2MW in the heating phasing (0-π).

CHI-	Capacitor bank operation, 3 Bank modules

Shot numbers for setup: 124271, Other useful shot numbers: 120870, 120888, 120890

DIAGNOSTIC CHECKLIST

Coupling a Transient CHI Discharge to OH	OP-XP-726

| #### Diagnostic | #### Need | #### Desire | #### Instructions |
| --- | --- | --- | --- |
| Bolometer – tangential array | X |  | * Use when reproducible conditions are achieved |
| Bolometer array - divertor |  | X |  |
| CHERS |  |  |  |
| Divertor fast camera |  | X |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge pressure gauges | X |  |  |
| Edge rotation spectroscopy |  | X | * |
| Fast lost ion probes - IFLIP |  |  |  |
| Fast lost ion probes - SFLIP |  |  |  |
| Filterscopes | X |  |  |
| FIReTIP |  | X | * |
| Gas puff imaging |  |  |  |
| Halpha camera - 1D |  | X | * |
| Infrared cameras |  | X |  |
| Interferometer - 1 mm |  |  |  |
| Langmuir probe array |  | X |  |
| Magnetics – Diamagnetism | X |  |  |
| Magnetics - Flux loops | X |  |  |
| Magnetics - Locked modes |  |  |  |
| Magnetics - Pickup coils | X |  |  |
| Magnetics - Rogowski coils | X |  |  |
| Magnetics - RWM sensors |  |  |  |
| Mirnov coils – high frequency | X |  |  |
| Mirnov coils – poloidal array | X |  |  |
| Mirnov coils – toroidal array | X |  |  |
| MSE |  | X |  |
| Neutral particle analyzer |  |  |  |
| Neutron measurements |  |  |  |
| Plasma TV | X |  |  |
| Reciprocating probe |  | X |  |
| Reflectometer – core |  |  |  |
| Reflectometer - SOL |  |  |  |
| RF antenna camera |  |  |  |
| RF antenna probe |  |  |  |
| SPRED | X |  |  |
| Thomson scattering |  | X | * |
| Ultrasoft X-ray arrays |  | X | * |
| Ultrasoft X-ray arrays – bicolor |  | X | * |
| Visible bremsstrahlung det. |  | X | * |
| Visible spectrometer (VIPS) |  | X |  |
| X-ray crystal spectrometer - H |  |  |  |
| X-ray crystal spectrometer - V |  |  |  |
| X-ray fast pinhole camera |  | X | * |