| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Fast ion transport induced by Alfvén avalanches |  |  |  |
| OP-XP-819 | Revision: | Effective Date: Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: E. D. Fredrickson, M Podesta, W Heidbrink, D Darrow |  |  | Date |
| ATI – ET Group Leader: G Taylor, E Fredrickson, N Gorelenkov |  |  | Date |
| RLM - Run Coordinator: M Bell |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Fast ion transport induced by Alfvén avalanches | No. OP-XP-80? |
| --- | --- |
| AUTHORS: E Fredrickson | DATE: |

# 1.	Overview of planned experiment

This experiment combines ideas from five proposals and provides target plasmas to address several goals.  The highest priority is to investigate the scaling with density, toroidal field, beam voltage, and current profile of the TAE and Avalanche onset thresholds.  A second goal is to document the fast ion loss due to fishbone-like energetic particle modes as seen with sFLIP.   The third goal is to expand the quiescent operational space to higher beam power and H-mode plasmas.  A final goal is to validate FIDA diagnostic with both quiescent plasmas and plasmas with strong energetic particle driven instabilities.

The target plasma (e.g., 124781) developed for TAE and TAE-avalanche studies in the 2007 campaign includes a quiescent period, followed by a period with TAE or TAE avalanches and ends, typically, with a period of multiple, fishbone-like modes.  This target plasma will be systematically varied to provide longer quiescent periods, quiescent periods with beam blips (if required), quiescent periods at higher beam power, as well as scans of beam voltage and power to determine onset threshold scaling of TAE and TAE avalanches.  An attempt will be made to extend the quiescent regime to H-mode plasmas.  Attempts will be made to optimize the fishbone phase for fast ion transport studies.  High-k scattering studies of TAE interactions with the continuum will also be included.

# 2.	Theoretical/ empirical justification

Fast-ion redistribution or losses due to fast ion modes in ITER or other fusion reactors is expected to be through multi-mode transport, e.g., Alfvén avalanches.  NSTX is well situated to the study of Alfvén avalanches.  Avalanches have been observed [Fredrickson, NF 06], but in plasmas prior to routine availability of MSE.  To simulate the effect of avalanches on fast ion confinement, we need to measure the amplitude (and confirm the radial eigenmode structure) of each mode involved in the avalanche.  We need the q-profile and other equilibrium data for NOVA calculations of mode structure.  The new FIDA diagnostic, together with NPA, ssNPA and sFLIP may also help confirm the details of the predicted affect on fast ion transport.

# 3.	Experimental run plan

Goal is to reproduce target plasma with quiescent period similar to 124764, then extend quiescent regime to parameters.  Introduce source A to 150 ms to provide early measurement of q-profile and after 300 ms.

Three activities will proceed in parallel with main experiment.  The first will to look for TAE coupling to Kinetic Alfvén waves in the continuum with the high-k scattering diagnostic.  Without the capability for remote steering, this will have to be done at some fixed radius.  The second activity will be to study HHFW suppression of Angelfish in the early period of these discharges.  Assuming Angelfish are present, a 20 ms blip of HHFW will be applied approximately from 150 to 170 ms.  The third will be to accumulate data during EPM activity expected during the second pulse of source A.

Main experiment

1.	Reproduce density, current, beams, neutron rate evolution for shot 124764.  Source A from 85 to 150ms, 300ms to 500ms, source C from 150ms to 300ms.

IF (plasma quiescent) THEN GOTO 2.

2.	Extend source C-to-A swap from 300 ms to 400 ms to determine length of quiescent period, for subsequent plasmas, set source C/A swap accordingly for subsequent shots.

c…Optimize length of quiescent period

3.	Skip at discretion of Heidbrink/Podesta

DO I=1,3

Start FIDA beam-blip experiments.

c…Make fast ion distribution measurements, optimize FIDA

ENDDO

4.	Switch to deuterium operation

c…Can we get quiescent D shot for FIDA (improves neutron accounting)

c…Will D shot go into H-mode?  (avalanche threshold in H-mode, different gap structure)

IF (plasma quiescent) THEN GOTO 5

ELSE

Raise density

IF (plasma quiescent) THEN GOTO 5

ELSE

increase toroidal field

IF (plasma quiescent) THEN GOTO 5

ELSE

lower beam power

IF (plasma quiescent) THEN GOTO 5

ELSE

Skip quiescent deuterium operation (5)

GOTO 6

ENDIF

5.	Skip at discretion of Podesta/Heidbrink

DO I=1,3

Repeat beam blip experiments in Deuterium.

c…Make fast ion distribution measurements, deuterium for (absolute) neutron measurements.

ENDDO

6.	IF (quiescent D plasmas H-mode) THEN go back to Helium L-mode plasmas

Adjust density to optimize reflectometer coverage, if req’d

Sources B & C should be nominally at 65 kV.

Use B at nom. 65 kV for quiescent period (150ms to ≈240ms), then source C (≈240ms to 300+ms).

Start voltage/power scan  for C to avalanche threshold

DO I=1,4

IF (no avalanche) THEN

Increase beam voltage by 10 kV increment

c…maintain target density

c…Scan of bfast to determine TAE and TAE avalanche threshold.

c…Similar to XP705, but using beam voltage/power instead of power

ELSE IF (avalanche) THEN

Decrease beam voltage by 5 kV

GOTO 7

ENDIF

ENDDO

IF (no avalanches) THEN, revert to 124781 (B at 64kV, C at 70 kV)

7.	q-dependence of avalanche threshold. change  source B150ms to 220 ms, source C from 220ms to 300+ms.  Moves high b_fast time earlier to higher q_min time.

c…is avalanche threshold dependent on qmin?

DO WHILE(MHD)

Delay swap in 10 ms steps

ENDDO

IF(no MHD AND avalanches)THEN

Reduce C voltage in 5 kV steps

ELSE IF (no avalanche) THEN

Increase C voltage in 5 kV steps

ENDIF

c…NPA/ssNPA/FIDA documentation of fast ion transport

c…high-k scattering gets radial scan here

8.	Start documentation of TAE avalanche condition (from either part 6 or 7)

6-shot NPA scan (tangency radius nominally 50cm, pitch ≈0.6)

9.	Increase toroidal field to 5.5 kG

c…low-Vfast/Valfven scaling of avalanche threshold

c…try for lower density?  Find threshold with ≈ 65 kV beams?

IF(no avalanches)THEN

DO WHILE(no avalanches; max steps 3)

Increase source C voltage by 5 kV, repeat

ENDDO

ELSE

DO WHILE(avalanches; max steps 3)

Decrease source C voltage by 5 kV, repeat

ENDDO

ENDIF

10.	Reduce toroidal field to 3.5 kG, source C at avalanche threshold for 7.

c…high-Vfast/Valfven (CTF) scaling of avalanche threshold

c…add higher density point?

IF(no avalanches)THEN

DO WHILE(no avalanches; max steps 3)

Increase source C voltage by 5 kV, repeat

ENDDO

ELSE

DO WHILE(avalanches; max steps 3)

Decrease source C voltage by 5 kV, repeat

ENDDO

ENDIF

11.	Switch to D prefill/puffing, attempt H-mode

c…Look for threshold in H-mode plasma

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

MSE is a must.

5 channel reflectometer diagnostic.

FIDA/NPA/ssNPA/sFLIP(w/PMT)

Capability for vertical scan of NPA is highly desired.  Alternatively, radial scan might be done.

High-k scattering desired.

# 5.	Planned analysis

Benchmarking TRANSP beam, bootstrap and fast ion deposition calculations will require extensive TRANSP analysis/simulations.

Study of TAE onset, TAE avalanche threshold will require extensive NOVA-k and M3D-K simulations.

# 6.	Planned publication of results

What will be the final disposition of the results; where will results be published and when?

PHYSICS OPERATIONS REQUEST

| TITLE: Fast ion transport induced by Alfvén avalanches | No. OP-XP-80? |
| --- | --- |
| AUTHORS: E Fredrickson | DATE: |

Machine conditions (specify ranges as appropriate)

ITF (kA): 34-65	Flattop start/stop (s): 0.0/1.0

IP (MA): 800	Flattop start/stop (s): 0.2/0.9

Configuration: 124764

Outer gap (m): 		Inner gap (m):

Elongation :		Upper/lower triangularity :

Z position (m):

Gas Species: He,Ar,D		Injector(s):  Midplane/Inner wall

NBI Species: D	Sources: 	ABC	Voltage (kV): 90/60-90/60-90  Duration (s): 0.9s

ICRF Power (MW): 2-3 MW		Phasing: 	180º	Duration (s): 0.02

CHI:	On / Off	Bank capacitance (mF):

LITER:  On / Off

Either:	List previous shot numbers for setup:

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

| TITLE: Fast ion transport induced by Alfvén avalanches | No. OP-XP- |
| --- | --- |
| AUTHORS: E Fredrickson | DATE: |

| Diagnostic | Need | Want | Conditions |
| --- | --- | --- | --- |
| Bolometer – tangential array |  |  |  |
| Bolometer – divertor |  |  |  |
| CHERS – toroidal |  |  |  |
| CHERS – poloidal |  |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitors |  |  |  |
| Edge neutral density diag. |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation diagnostic |  |  |  |
| Fast ion D_alpha - FIDA |  |  |  |
| Fast lost ion probes - IFLIP |  |  |  |
| Fast lost ion probes - SFLIP |  |  |  |
| Filterscopes |  |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  |  |
| Ha camera - 1D |  |  |  |
| High-k scattering |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer - 1 mm |  |  |  |
| Langmuir probes - divertor |  |  |  |
| Langmuir probes – RF ant. |  |  |  |
| Magnetics – Diamagnetism |  |  |  |
| Magnetics - Flux loops |  |  |  |
| Magnetics - Locked modes |  |  |  |
| Magnetics - Pickup coils |  |  |  |
| Magnetics - Rogowski coils |  |  |  |
| Magnetics - RWM sensors |  |  |  |

| Diagnostic | Need | Want | Conditions |
| --- | --- | --- | --- |
| Mirnov coils – high f. |  |  |  |
| Mirnov coils – poloidal array |  |  |  |
| Mirnov coils – toroidal array |  |  |  |
| MSE |  |  |  |
| NPA – ExB scanning |  |  |  |
| NPA – solid state |  |  |  |
| Neutron measurements |  |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer – 65GHz |  |  |  |
| Reflectometer – correlation |  |  |  |
| Reflectometer – FM/CW |  |  |  |
| Reflectometer – fixed f |  |  |  |
| Reflectometer – SOL |  |  |  |
| RF edge probes |  |  |  |
| Spectrometer – SPRED |  |  |  |
| Spectrometer – VIPS |  |  |  |
| SWIFT – 2D flow |  |  |  |
| Thomson scattering |  |  |  |
| Ultrasoft X-ray arrays |  |  |  |
| Ultrasoft X-rays – bicolor |  |  |  |
| Ultrasoft X-rays – TG spectr. |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| X-ray crystal spectrom’r - H |  |  |  |
| X-ray crystal spectrom’r - V |  |  |  |
| X-ray fast pinhole camera |  |  |  |
| X-ray spectrometer - XEUS |  |  |  |