| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: High-k turbulent fluctuations in NSTX |  |  |  |
| OP-XP-821 | Revision: | Effective Date: 02/20/08 Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: E. Mazzucato |  |  | Date 02/20/08 |
| ATI – ET Group Leader: S. Kaye |  |  | Date |
| RLM - Run Coordinator: M. Bell |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: High-k turbulent fluctuations in NSTX | No. OP-XP-821 |
| --- | --- |
| AUTHOR: E. Mazzucato | DATE: 2/20/08 |

# 1.	Overview of planned experiment

Previous measurements of high-k fluctuations in Helium plasmas with HHFW heating (XP-735) have revealed the existence of a turbulence with large values of krs.

# The goal of this experiment is to study the plasma response to changes in the critical gradient for excitation of this type of turbulence.

# 2.	Theoretical/ empirical justification

In XP-735, plasmas were found near marginal stability, as defined by Jenko’s empirical formula for the critical gradient of the electron temperature gradient mode. Since the latter is very sensitive to Zeff and the temperature ratio Te / Ti, the goal of this experiment is to measure high-k fluctuations in plasmas with different values of these two parameters. This could be done by employing both Helium and Deuterium plasmas (Zeff) using a variable RF power and plasma densities (variable Te / Ti).

# Experimental run plan

XP should be scheduled after completion of XP-HHFW in D-plasmas.

First ½ day-run: repeat measurements of XP-735 in D-plasmas with a variable HHFW power (12.5 MW) and a longer (> 0.35 s) RF pulse (~12 shots).

Second ½ day-run: same in He-plasma (~12 shots).

Remaining of run plan (1 day) to be decided after first day operation (outboard measurements, density scan, RF+NBI)

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Shot development needed to attain MHD quiescent conditions in D plasmas

# 5.	Planned analysis

LRDFIT, TRANSP,  Turbulence Codes

# 6.	Planned publication of results

IAEA, PRL

PHYSICS OPERATIONS REQUEST

| TITLE: High-k turbulent fluctuations in NSTX | No. OP-XP-821 |
| --- | --- |
| AUTHORS: E. Mazzucato | DATE: 02/20/08 |

Machine conditions (specify ranges as appropriate)

BTF (kG): 5.5	Flattop start/stop (s):  0.15 /0.6

IP (MA): 0.7	Flattop start/stop (s):  0.15 /0.6

Configuration: As for 124901

Outer gap (m):			Inner gap (m):

Elongation :			Triangularity :

Z position (m):

Gas Species:  He / D        Injector:

NBI - Species: D	    Sources: 2 sources 	Voltage (kV): 90/70	Duration (s): 0.05 s blips

ICRF – Power (MW):  2 MW  	Duration (s): 0.15-0.55

CHI: Off

LITER: Off

Either:	List previous shot numbers for setup: 124901 with 0.15-0.60 flattop

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

| TITLE: High-k turbulent fluctuations in NSTX | No. OP-XP-821 |
| --- | --- |
| AUTHORS: E. Mazzucato | DATE: 02/20/08 |

| Diagnostic | Need | Want | Conditions |
| --- | --- | --- | --- |
| Bolometer – tangential array | x |  |  |
| Bolometer – divertor |  |  |  |
| CHERS – toroidal | x |  |  |
| CHERS – poloidal | x |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitors |  |  |  |
| Edge neutral density diag. |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation diagnostic | x |  |  |
| Fast ion D_alpha - FIDA |  |  |  |
| Fast lost ion probes - IFLIP |  |  |  |
| Fast lost ion probes - SFLIP |  |  |  |
| Filterscopes | x |  |  |
| FIReTIP | x |  |  |
| Gas puff imaging |  |  |  |
| H camera - 1D |  |  |  |
| High-k scattering | x |  |  |
| Infrared cameras |  |  |  |
| Interferometer - 1 mm |  |  |  |
| Langmuir probes - divertor |  |  |  |
| Langmuir probes – RF ant. |  |  |  |
| Magnetics – Diamagnetism | x |  |  |
| Magnetics - Flux loops | x |  |  |
| Magnetics - Locked modes | x |  |  |
| Magnetics - Pickup coils | x |  |  |
| Magnetics - Rogowski coils | x |  |  |
| Magnetics - RWM sensors | x |  |  |

| Diagnostic | Need | Want | Conditions |
| --- | --- | --- | --- |
| Mirnov coils – high f. | x |  |  |
| Mirnov coils – poloidal array | x |  |  |
| Mirnov coils – toroidal array | x |  |  |
| MSE | x |  |  |
| NPA – ExB scanning |  |  |  |
| NPA – solid state |  |  |  |
| Neutron measurements |  |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer – 65GHz |  |  |  |
| Reflectometer – correlation |  |  |  |
| Reflectometer – FM/CW |  |  |  |
| Reflectometer – fixed f |  |  |  |
| Reflectometer – SOL |  |  |  |
| RF edge probes |  |  |  |
| Spectrometer – SPRED | x |  |  |
| Spectrometer – VIPS |  |  |  |
| SWIFT – 2D flow |  |  |  |
| Thomson scattering | x |  |  |
| Ultrasoft X-ray arrays | x |  |  |
| Ultrasoft X-rays – bicolor | x |  |  |
| Ultrasoft X-rays – TG spectr. |  |  |  |
| Visible bremsstrahlung det. | x |  |  |
| X-ray crystal spectrom’r - H |  |  |  |
| X-ray crystal spectrom’r - V |  |  |  |
| X-ray fast pinhole camera |  |  |  |
| X-ray spectrometer - XEUS |  |  |  |