| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Halo Current Dependencies on IP/q95, Vertical Velocityand Halo Resistance |  |  |  |
| OP-XP-833 | Revision: 1 | Effective Date: Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: Stefan Gerhardt |  |  | Date |
| ATI – ET Group Leader: Steve Sabbagh |  |  | Date |
| RLM - Run Coordinator: Mike Bell |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Halo Current Dependencies on IP/q95, Vertical Velocityand Halo Resistance | No. OP-XP-833 |
| --- | --- |
| AUTHORS: Stefan Gerhardt | DATE: |

# 1.	Overview of planned experiment

Halo currents are electrical currents which flow partially in the plasma, and partially in plasma facing  components. They generally occur when control of the plasma vertical position is lost, either before or during a disruption (a scenario known as a vertical displacement event, or VDE). The goal of this experiment is the asses the scaling of Halo currents with certain important plasma parameters. Results from many tokamaks (Alcator C-mod, MAST, COMPASS-D, and JET) have seen an Ip/q95 scaling of the halo current magnitude, which this experiment will attempt to study with devoted scans for two very different halo current paths. In a separate scan, the dependence of the halo current magnitude on the VDE velocity will be studied. The next step will study if He and D2 have different halo currents for otherwise similar conditions. Finally, the dependence of halo current magnitude on halo electrical resistance will be studied, by cooling the halo with Neon injection. Part of these experiments will be conducted in a special low-triangularity shape, developed to steer the Halo currents into a well-diagnosed path, linking the outboard divertor and the secondary passive plates.

# 2.	Theoretical/ empirical justification

Halo current are known to provide a serious constraint on mechanical and electrical design of tokamak in-vessel components.  For instance, they are thought to be the dominant source of electromagnetic loading during slow VDE current quenches in ITER. From the perspective of NSTX, an improved understanding of halo currents will provide important information for the mechanical design of the liquid lithium divertor (LLD), as well as future larger STs. New halo current detection hardware was installed in NSTX during the CY07 outage period, allowing improved measurement of these currents. For instance, the data in the adjacent figure provides information on the scaling of halo currents with the parameter Ip/q95, for a large database of shots, and for four different halo current path measurements. One goal of the present experiment is to systematically vary this parameter for two different plasma shapes while generating large halo currents.

# 3.	Experimental run plan

Step 1: Restore Target Shot: 127070, 600 kA He shot, B=0.45 Tesla. Reached flat-top at ~0.1 sec. Stored energy of 30kJ.

Take Shot Without Freeze: ______

Take Shot with Freeze @250msec, lasting 100msec, and downward bias on PF3 of 10 V and 5msec duration at the start of the freeze:______

All He shots run on a 10 minute shot cycle with no GDC between shots.

Step 2: Complete a scan of Halo Currents vs IP/q95, in the high triangularity reference shape.

The first 8 shots in this table to be completed as part of initial scan, the final 4 at the end of the day, time permitting.

| Step # | Shot Number | IP | BT | Ip2/BT |
| --- | --- | --- | --- | --- |
| 2.1 |  | 500 | .45 | .55 |
| 2.2 |  | 600 | .45 | 0.8 |
| 2.3 |  | 650 | .45 | 0.94 |
| 2.4 |  | 700 | .45 | 1.08 |
| 2.5 |  | 700 | .4 | 1.225 |
| 2.6 |  | 700 | .35 | 1.4 |
| 2.7 |  | 500 | .55 | 0.45 |
| 2.8 |  | 500 | .5 | 0.5 |
| 2.9 |  | 700 | .35 | 1.4 |
| 2.10 |  | 500 | .55 | 0.45 |
| 2.11 |  | 550 | .45 | .672 |
| 2.12 |  | 650 | .45 | 0.94 |

Step 3: Develop Low Triangularity target:

Throughout this process, the timing and magnitude of the freeze and offset should be left alone, so that the halo currents can be studied as the triangularity is reduced. This allows a diagnosis of the halo current path during the VDE. The fully developed shot will have the halo current linking the secondary passive plates and outboard divertor.

|  | PF1A | PF2 (kA) | PF2 (kA/MA) | PF3 (kA) | PF3 (kA/MA) | PF5 |
| --- | --- | --- | --- | --- | --- | --- |
| Reference | 7.38 | 4.2 | 7 | -4.95 | -8.25 |  |
| 3.1 | 0 | 6.225 | 10.375 | -4.95 | -8.25 | -6.06 |
| 3.2 | 0 | 8.225 | 13.7 | -4.95 | -8.25 | -6.223 |
| 3.3 | 0 | 10.225 | 17.04 | -4.95 | -8.25 | -6.554 |
| 3.4 | 0 | 12.225 | 20.83 | -4.95 | -8.25 | -6.94 |
| 3.5 | 0 | 12.225 | 20.82 | -3.95 | -6.58 | -7.45 |
| 3.6 | 0 | 12.225 | 20.83 | -2.95 | -4.92 | -7.9 |

In order to bias down at any point in the scan: Add 600 A to PF3L (change of 1kA/MA)

Figure 1: Approximate final target for low triangularity halo current measurement, after completion of step 3.6 above

Step 4: Complete a scan of Halo Currents vs IP/q95 at low d

As in step 2, the first 8 shots are to be completed as part of the initial scan, and the last 4 later in the day, time permitting.

| Step # | Shot Number | IP | BT | Ip2/BT |
| --- | --- | --- | --- | --- |
| 4.1 |  | 500 | .45 | .55 |
| 4.2 |  | 600 | .45 | 0.8 |
| 4.3 |  | 650 | .45 | 0.94 |
| 4.4 |  | 700 | .45 | 1.08 |
| 4.5 |  | 700 | .4 | 1.225 |
| 4.6 |  | 700 | .35 | 1.4 |
| 4.7 |  | 500 | .55 | 0.45 |
| 4.8 |  | 500 | .5 | 0.5 |
| 4.9 |  | 700 | .35 | 1.4 |
| 4.10 |  | 500 | .55 | 0.45 |
| 4.11 |  | 550 | .45 | .672 |
| 4.12 |  | 650 | .45 | 0.94 |

Step 5: Complete a scan of the plasma vertical velocity.

Pick a shot from steps 2-4 that had reliably large and easily measurable halo currents, most likely the 700 kA case at either .35 or .4 Tesla. Also, note that there are two possible means to adjust the plasma vertical velocity: change the magnitude of the offset voltage, or the duration of the “kick”. Fill in either of the following matrices, depending on which seems most plausible from the results of steps 2-4.

| Step # | # of Shots | Shot Number(s) | IP | BT | Duration of Kick |
| --- | --- | --- | --- | --- | --- |
| 5.1 | 1 |  |  |  | 5 msec |
| 5.2 | 1 |  |  |  | 7 msec |
| 5.3 | 1 |  |  |  | 9 msec |
| 5.4 | 1 |  |  |  | 11 msec |
| 5.5 | 1 |  |  |  | 13 msec |

| Step # | # of Shots | Shot Number(s) | IP | BT | Voltage Offset |
| --- | --- | --- | --- | --- | --- |
| 5.6 | 1 |  |  |  | 10 V |
| 5.7 | 1 |  |  |  | 20 V |
| 5.8 | 1 |  |  |  | 30 V |
| 5.9 | 1 |  |  |  | 40 V |
| 5.10 | 1 |  |  |  | 50 V |

Step 6: Comparison of Halo currents in D2 and He.

Taking the reference shot from step 6, i.e. a shot with large halo currents, switch to D2 operations. The switch should start with:

- 2 minutes D2 glow

- 6.5 minutes He glow

A 10 minute shots cycle, with 5 minutes of He  GDC between shots, should then begin. Fill in the following table in a scan over Ip/q95.

| Step # | # of Shots | Shot Number(s) | IP | BT | Ip2/BT |
| --- | --- | --- | --- | --- | --- |
| 6.1 | 1 |  | 500 | .45 | .55 |
| 6.3 | 1 |  | 600 | .45 | 0.8 |
| 6.5 | 1 |  | 700 | .45 | 1.08 |
| 6.6 | 1 |  | 700 | .4 | 1.225 |
| 6.7 | 1 |  | 700 | .35 | 1.4 |
| 6.8 | 1 |  | 500 | .55 | 0.45 |
| 6.9 | 1 |  | 500 | .5 | 0.5 |

Step 7: Complete a scan of Neon Injection into the lower divertor during the VDE

Use the reference configuration developed in sections 2-4 and utilized in sections 5 & 6. Puff neon into the lower divertor, timed to enter just before the plasma begins its downward motion. Start with step 7.2, and then increase or decrease the Ne injection based upon result of scan.

| Step # | # of Shots | Shot Number(s) | IP | BT | Torr-Liters |
| --- | --- | --- | --- | --- | --- |
| 7.1 | 1 |  |  |  | 0.025 |
| 7.2 | 1 |  |  |  | 0.05 |
| 7.3 | 1 |  |  |  | 0.1 |
| 7.4 | 1 |  |  |  | 0.2 |
| 7.5 | 1 |  |  |  | 0.3 |
| 7.6 | 1 |  |  |  | 0.4 |

The successful implementation of this step will require that the Bay E lower piezo valve be calibrated for Neon gas flow. This exercise will be completed before the run day.

Note: Typical gas injection parameters for Neon seeding experiments are 1.5 TL/s for ~.006 sec., for a total injection of ~.01 TL.

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

# It is anticipated that a 10 minute clock cycle will be used for the entire day, including the cases in D2. It will not be necessary to “sync-up” with the neutral beam cycle.

# The reference shot and shot development will be done using the pcc (“gap control”) algorithm. Isoflux control will not be used, as that algorithm is not presently equipped with a voltage freeze command.

NBI, RF, and CHI are not required for the completion of this XP.

Between shot equilibrium reconstructions with 1msec time resolution are highly desirable.

A fast camera with ~1msec time resolution in fish-eye view is highly desirable

The injector 1 system should be configured for Neon injection into Bay E bottom.

Injector 3 should have Helium, and will additionally provide the glow-discharge gas during the D2 shots.

Injector 2 should have D2, for a prefill of about 1.6x10-5 T (this number take from 127070). This will also provide the gas for the D2 glow to precede the switch to D2 plasmas.

Extreme care must be take to not prefill or GDC in Neon

# 5.	Planned analysis

These specialized scans will be used in conjunction with database analysis of the CY08 campaign results to determine the scaling of halo currents with various plasma parameters. High time-resolution equilibrium analysis with the EFIT (01) and LRDFIT (01) codes will be utilized.

# 6.	Planned publication of results

It is expected that the Halo current data from this year, including the devoted scans in this XP, will be published in a Nuclear Fusion paper at the conclusion of the run campaign. The data will also be contributed to the ITPA disruption database activity.

PHYSICS OPERATIONS REQUEST

| TITLE: Halo Current Dependencies on IP/q95, Vertical Velocityand Halo Resistance | No. OP-XP-833 |
| --- | --- |
| AUTHORS: Stefan Gerhardt | DATE: |

Machine conditions (specify ranges as appropriate)

ITF (kA): .3-.55 Tesla	Flattop start/stop (s): 0/0.5

IP (MA): 500-700 kA	Flattop start/stop (s): .1/.4

Configuration: LSN

Outer gap (m): 	target 127070		Inner gap (m): target 127070

Elongation : target 127070	          Upper/lower triangularity : varied, see step 3.

Z position (m):	0, or slightly biased down

Gas Species: He, D2, Ne		Injector(s): See section 4

NBI Species: D	Sources: 	None	Voltage (kV): 		Duration (s):

ICRF Power (MW):	0	Phasing: 		Duration (s):

CHI:	Off	Bank capacitance (mF):

LITER: Off

Either:	List previous shot numbers for setup:	127070

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

| TITLE: Halo Current Dependencies on IP/q95, Vertical Velocityand Halo Resistance | No. OP-XP-833 |
| --- | --- |
| AUTHORS: Stefan Gerhardt | DATE: |

| Diagnostic | Need | Want | Conditions |
| --- | --- | --- | --- |
| Bolometer – tangential array |  | X |  |
| Bolometer – divertor | X |  |  |
| CHERS – toroidal |  |  |  |
| CHERS – poloidal |  |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitors |  |  |  |
| Edge neutral density diag. |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation diagnostic |  |  |  |
| Fast ion D_alpha - FIDA |  |  |  |
| Fast lost ion probes - IFLIP |  |  |  |
| Fast lost ion probes - SFLIP |  |  |  |
| Filterscopes | X |  |  |
| FIReTIP |  |  |  |
| Gas puff imaging |  |  |  |
| Ha camera - 1D |  |  |  |
| High-k scattering |  |  |  |
| Infrared cameras |  |  |  |
| Interferometer - 1 mm |  |  |  |
| Langmuir probes - divertor |  | X |  |
| Langmuir probes – RF ant. |  |  |  |
| Magnetics – Diamagnetism | X |  |  |
| Magnetics - Flux loops | X |  |  |
| Magnetics - Locked modes | X |  |  |
| Magnetics - Pickup coils | X |  |  |
| Magnetics - Rogowski coils | X |  |  |
| Magnetics - RWM sensors | X |  |  |

| Diagnostic | Need | Want | Conditions |
| --- | --- | --- | --- |
| Mirnov coils – high f. | X |  |  |
| Mirnov coils – poloidal array | X |  |  |
| Mirnov coils – toroidal array | X |  |  |
| MSE |  |  |  |
| NPA – ExB scanning |  |  |  |
| NPA – solid state |  |  |  |
| Neutron measurements |  |  |  |
| Plasma TV | X |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer – 65GHz |  |  |  |
| Reflectometer – correlation |  |  |  |
| Reflectometer – FM/CW |  |  |  |
| Reflectometer – fixed f |  |  |  |
| Reflectometer – SOL |  |  |  |
| RF edge probes |  |  |  |
| Spectrometer – SPRED |  |  |  |
| Spectrometer – VIPS |  |  |  |
| SWIFT – 2D flow |  |  |  |
| Thomson scattering |  | X |  |
| Ultrasoft X-ray arrays |  |  |  |
| Ultrasoft X-rays – bicolor |  |  |  |
| Ultrasoft X-rays – TG spectr. |  |  |  |
| Visible bremsstrahlung det. |  |  |  |
| X-ray crystal spectrom’r - H |  |  |  |
| X-ray crystal spectrom’r - V |  |  |  |
| X-ray fast pinhole camera |  |  |  |
| X-ray spectrometer - XEUS |  |  |  |