| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Effect of Rotation on the L-H Threshold |  |  |  |
| OP-XP-8xx | Revision: | Effective Date: 6/19/08 Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: S. Kaye |  |  | Date |
| ATI – ET Group Leader: S. Kaye |  |  | Date |
| RLM - Run Coordinator: M. Bell |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Effect of rotation on L-H threshold | No. OP-XP-8xx |
| --- | --- |
| AUTHORS: Kaye | DATE: 6/19/08 |

# 1.	Overview of planned experiment

# The goal of this experiment is to study the effect of rotation on the L-H threshold power. n=3 braking will be used to vary the rotation.

# 2.	Theoretical/ empirical justification

This XP is an element of the study of the 2008 Joule milestone, and it is to study the effect of rotation, and by extension rotational shear at the edge, on the L-H threshold. This has direct bearing on developing an understanding of the effect of this edge rotational shear and how it might impact thresholds in future devices at both low rotation (ITER) and at higher rotation (NHTX< ST-CTF).

# 3.	Experimental run plan

- Establish L-H threshold power in low-discharge (129019 basline) using one source; vary voltage in source to achieve threshold. If necessary modulate source to lower power even farther

  - Use optimal n=3 error field correction (-300 A in SPA1) and n=1 mode control

  - Source C will allow lower power (voltage) if necessary

- Vary n=3 SPA coil current to 0 A, 400 A, 800 A

  - Establish threshold in same manner at each level.

TABULAR SHOT LIST

| # Condition | # Ip (MA) | # BT (T) | # SPA 1 Current (steady) |
| --- | --- | --- | --- |
| # 1 | # 0.9 | # 0.45 | # -300 A (optimum) |
| # 2 | # 0.9 | # 0.45 | # 0 A |
| # 3 | # 0.9 | # 0.45 | # 400 A |
| # 4 | # 0.9 | # 0.45 | # 800 A |

# Total: 24 shots

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Discharge reproducibility, ability to achieve H-mode with one source.

Important feature is to use NBI pre-heat (one source) from 80-140 ms, then turn source back on after Ip flattop, at 220 ms.

# 5.	Planned analysis

LRDFIT, TRANSP, specialized codes

# 6.	Planned publication of results

Joule milestone, TTF, ITPA, IAEA

PHYSICS OPERATIONS REQUEST

| TITLE: Effect of rotation on L-H threshold | No. OP-XP-8xx |
| --- | --- |
| AUTHORS: Kaye | DATE: 6/19/08 |

Machine conditions (specify ranges as appropriate)

ITF (kA): 52 kA (4.5 kG)	Flattop start/stop (s):

IP (MA): 0.9 MA	Flattop start/stop (s):

Configuration: LSN

Outer gap (m): 		Inner gap (m):

Elongation : 1.8		Upper/lower triangularity : 0.45

Z position (m):	0

Gas Species: D		Injector(s):

NBI Species: D	Sources: 	3	Voltage (kV): 	80, 90	Duration (s): full shot

ICRF Power (MW):	0	Phasing: 		Duration (s):

CHI:	Off	Bank capacitance (mF):

LITER:  Off (during initial attempt at XP)

Either:	List previous shot numbers for setup:	129019

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

| TITLE: Effect of rotation on L-H threshold | No. OP-XP-8xx |
| --- | --- |
| AUTHORS: Kaye | DATE: 6/19/08 |

| Diagnostic | Need | Want | Conditions |
| --- | --- | --- | --- |
| Bolometer – tangential array | x |  |  |
| Bolometer – divertor |  |  |  |
| CHERS – toroidal | x |  |  |
| CHERS – poloidal | x |  |  |
| Divertor fast camera |  |  |  |
| Dust detector |  |  |  |
| EBW radiometers |  |  |  |
| Edge deposition monitors |  |  |  |
| Edge neutral density diag. |  |  |  |
| Edge pressure gauges |  |  |  |
| Edge rotation diagnostic |  | x |  |
| Fast ion D_alpha - FIDA | x |  |  |
| Fast lost ion probes - IFLIP |  |  |  |
| Fast lost ion probes - SFLIP |  | x |  |
| Filterscopes | x |  |  |
| FIReTIP |  | x |  |
| Gas puff imaging |  | x |  |
| H camera - 1D |  |  |  |
| High-k scattering |  | x |  |
| Infrared cameras |  |  |  |
| Interferometer - 1 mm |  |  |  |
| Langmuir probes - divertor |  |  |  |
| Langmuir probes – RF ant. |  |  |  |
| Magnetics – Diamagnetism | x |  |  |
| Magnetics - Flux loops | x |  |  |
| Magnetics - Locked modes | x |  |  |
| Magnetics - Pickup coils | x |  |  |
| Magnetics - Rogowski coils | x |  |  |
| Magnetics - RWM sensors | x |  |  |

| Diagnostic | Need | Want | Conditions |
| --- | --- | --- | --- |
| Mirnov coils – high f. | x |  |  |
| Mirnov coils – poloidal array | x |  |  |
| Mirnov coils – toroidal array | x |  |  |
| MSE |  | x |  |
| NPA – ExB scanning |  |  |  |
| NPA – solid state |  | x |  |
| Neutron measurements | x |  |  |
| Plasma TV |  |  |  |
| Reciprocating probe |  |  |  |
| Reflectometer – 65GHz |  |  |  |
| Reflectometer – correlation |  |  |  |
| Reflectometer – FM/CW |  |  |  |
| Reflectometer – fixed f |  | x |  |
| Reflectometer – SOL |  |  |  |
| RF edge probes |  |  |  |
| Spectrometer – SPRED | x |  |  |
| Spectrometer – VIPS |  |  |  |
| SWIFT – 2D flow |  |  |  |
| Thomson scattering | x |  |  |
| Ultrasoft X-ray arrays | x |  |  |
| Ultrasoft X-rays – bicolor |  | x |  |
| Ultrasoft X-rays – TG spectr. |  |  |  |
| Visible bremsstrahlung det. |  | x |  |
| X-ray crystal spectrom’r - H |  |  |  |
| X-ray crystal spectrom’r - V |  |  |  |
| X-ray fast pinhole camera |  |  |  |
| X-ray spectrometer - XEUS |  | x |  |