| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Use of Resonant Field Amplification Measurements To Establish MHD Stability Boundaries |  |  |  |
| OP-XP-930 | Revision: 0 | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: S.P.Gerhardt, L. Delgado-Aparicio, D.A. Gates, J.E. Menard, J. K. Park, H. Reimerdes, S. Sabbagh |  |  | Date |
| ATI – ET Group Leader: S. Sabbagh |  |  | Date |
| RLM - Run Coordinator: R. Raman |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: RFA For Stability Boundaries | No. OP-XP-930 |
| --- | --- |
| AUTHORS: S.P. Gerhardt, et al | DATE: |

# 1.	Overview of planned experiment

The ultimate goal of this line of research is to implement realtime stability limit assessment via measurements of resonant field application (RFA). The present XP (XP-930) has two goals which contribute to this larger effort: i) make more detailed measurements of RFA as a function of bN, and ii) assess how RFA changes as a function of parameters like dl and li. If time permits, studies near the low-q limit will also be attempted. In this XP, n=1 traveling waves will be applied to the plasma, and the n=1 plasma response will be measured. This response will be measured as a function of plasma internal inductance, boundary triangularity, q*, and traveling wave frequency.

# 2.	Theoretical/ empirical justification

It is likely that the reliable operation of a tokamak power plant will require realtime assessment of the plasma stability. Ideally, this would be done by stability calculations on highly converged equilibria, constrained by pitch angle and kinetic profile data, all done in real time. The ability to do such calculation does not exist at the moment, and is unlikely to exist anytime in the near future.

An potential alternative method for assessing the stability of the plasma to the deadly n=1 resistive wall mode has been suggested, for instance in Reimerdes, et al. NF 45. When the plasma b value is near or exceeds the stability limit in the absence of a conducting wall (the no-wall limit, or bN,no-wall), the plasma response to an applied n=1 field is to amplify that field, possible with a phase shift. Hence, by monitoring this plasma response, it is possible to assess the proximity of the plasma to this important stability limit.

For this scheme to work for stability-limit detection, it is important to verify that the variables that are known to impact the ideal stability of the plasma will also change the RFA level. For instance, it is known from numerical and discharge optimization studies that increasing the plasma triangularity or internal inductance will improve the plasma’s stability to n=1 modes. There is also a current limit at q*~2. It is the purpose of this XP to assess how changes in these two quantities will impact the measured RFA in NSTX.

# 3.	Experimental run plan

Lithium Specification: 20 mg/min, no D2 glow discharge, 10 minute shot cycle.

SPA/ EFC Coil Specification: Standard anti-series connection.

Regarding SPA waveforms: For each condition, apply traveling waves at 4 frequencies: -60, -30, 30, 60.

PCS waveform inputs for these cases can be derived from the following table.

Information for generating SPA waveforms.

Step 3.0: Develop a low-d, high-k shot.

- This to be done before the XP is run.  Shot #_________

- Call this “Shape #1”

- Shape #1 k:_____

- Shape #1 Outer Gap:_____

Step 3.1: Demonstrate RFA vs. bN trend in a single low-li, high-d discharge.

- Reference shot is a recent fiducial:_133025_______

- Adjust kappa and outer-gap to match those of Shape #1. Call the new shape “Shape #2”.

- Modify NBI timing to have smooth bN ramp-up and ramp-down. Likely source timings for this case, in msec: A: 40-End of shot, B: 80-1000, C: 80-130, 400-600.

- Add 30 Hz co-going n=1 perturbation, along with slow n=1 feedback (tSPA=100msec). Verify that there is no n=1 feedback request perturbation at the frequency of the traveling wave.

- Repeat this target discharge with the 4 different TW frequencies.

| Case | Frequency | Shots |
| --- | --- | --- |
| 1 | 30 |  |
| 2 | -30 |  |
| 3 | 60 |  |
| 4 | -60 |  |

Step 3.2: Test of RFA at low-li, low-d.

- Reload Shape #1 (low-d)

- Keep NBI ramps from previous step, though be aware that this shape may require a reduction in input power to maintain stability.

- Repeat this discharge with the 4 different TW frequencies.

| Case | Frequency | Shots |
| --- | --- | --- |
| 1 | 30 |  |
| 2 | -30 |  |
| 3 | 60 |  |
| 4 | -60 |  |

Step 3.3: Test RFA at higher-li, high-d.

- Reload Shape #2 (high-d)

- Reduce the early beam power to delay the H-mode transition until the start of the flat-top. Likely source timings for this case, in msec: A: 40-End of shot, B: 180-1000, C: 400-600.

- Repeat this discharge with the 4 different TW frequencies.

| Case | Frequency | Shots |
| --- | --- | --- |
| 1 | 30 |  |
| 2 | -30 |  |
| 3 | 60 |  |
| 4 | -60 |  |

Based on the results of steps 3.1-3.3, go to either step 3.4 or 3.5

Step 3.4: Test RFA at high-d, near the current limit.

- Reload Shape #2 (high-d). Increase IP to 1.3 MA. This should achieve q* values in the range of 2.3.

- Revert to beam timing for early H-mode, likely similar to that used in 3.1.

- If shot survives, use as target. Otherwise, reduce IP to 1.2 MA.

- Repeat this discharge with the 4 different TW frequencies.

| Case | Frequency | Shots |
| --- | --- | --- |
| 1 | 30 |  |
| 2 | -30 |  |
| 3 | 60 |  |
| 4 | -60 |  |

Step 3.5: Test RFA at higher-li, low-d

- Reload Shape #1 (low-d)

- Reduce the early beam power to delay the H-mode transition until the start of the flat-top. Likely source timings for this case, in msec: A: 40-End of shot, B: 180-1000, C: 400-600.

- Repeat this discharge with the 4 different TW frequencies.

| Case | Frequency | Shots |
| --- | --- | --- |
| 1 | 30 |  |
| 2 | -30 |  |
| 3 | 60 |  |
| 4 | -60 |  |

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

It is highly desirable to operate in a Li-conditioned scenario on a 10-minute shot cycle.

It is highly desirable that the development of shape #1 be done before this XP is executed.

# 5.	Planned analysis

Equilibrium reconstruction (EFIT & LRDFIT) are essential tools. The stable plasma response to the applied n=1 field will be studied with the RWM sensors and the optical USXR array. The plasma’s ideal stability will be tested with the DCON code. It may be necessary to use the CHEASE code coupled to DCON in order to compute the no-wall and with-wall stability limit.

# 6.	Planned publication of results

If successful, these results will make for excellent presentations at, for instance, the IAEA and EPS meetings. Publication in a journal such as Nuclear Fusion or PPCF would be appropriate. Equally important, these results would contribute to the design of realtime stability calculation software in NSTX.

PHYSICS OPERATIONS REQUEST

| TITLE: RFA For Stability Boundaries | No. OP-XP-930 |
| --- | --- |
| AUTHORS: S.P. Gerhardt, et al. | DATE: |

(use additional sheets and attach waveform diagrams if necessary)

Describe briefly the most important plasma conditions required for the experiment: This XP will need two different shapes: Shape #1, high-k, low-dIt appears that k=2.25 and dl=0.38, shifted down ~2cm, is a reasonable equilibrium, diverting with the PF2 coils only. Ideally, this shape should be developed before this XP is run. Shape #2, high-k, high-d This shot should be a PF1A shot with k and outer gap matched to Shape #1. This can likely be achieved with very minor modifications to the fiducial.

Previous shot(s) which can be repeated: Previous shot(s) which can be modified: 2009 fiducial such as 133025

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): 0.45 T Flattop start/stop (s): Standard Waveform IP (MA): 900 kA, 1300 kA Flattop start/stop (s): 0/1.3 sec. Configuration: See above Equilibrium Control: Isoflux (rtEFIT) Outer gap (m): ~10cm Inner gap (m): Z position (m): -0.02 Elongation : 2.25 Upper/lower triangularity : varies, see above Gas Species: D2 Injector(s): Standard D2 prefill and HFS gas. NBI Species: D Voltage (kV) A: 90 B: 90 C: 70-90 Duration (s): up to 1.3 sec ICRF Power (MW): 0 Phase between straps (°): Duration (s): CHI: Off Bank capacitance (mF): LITERs: On Total deposition rate (mg/min): 20 (or whatever is sufficient for a 10 minute, no-glow scenario) EFC coils: On Configuration: Odd

DIAGNOSTIC CHECKLIST

| TITLE: RFA For Stability Boundaries | No. OP-XP-930 |
| --- | --- |
| AUTHORS: S.P. Gerhardt, et al. | DATE: |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array | √ |  |
| Bolometer – divertor |  |  |
| CHERS – toroidal | √ |  |
| CHERS – poloidal | √ |  |
| Divertor fast camera |  |  |
| Dust detector |  |  |
| EBW radiometers |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  |  |
| Edge pressure gauges |  |  |
| Edge rotation diagnostic |  |  |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes |  |  |
| FIReTIP |  |  |
| Gas puff imaging |  |  |
| H camera - 1D |  |  |
| High-k scattering |  |  |
| Infrared cameras |  |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  |  |
| Langmuir probes – BEaP |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – Diamagnetism |  |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes | √ |  |
| Magnetics – Pickup coils | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents | √ |  |
| Magnetics – RWM sensors | √ |  |
| Mirnov coils – high f. | √ |  |
| Mirnov coils – poloidal array |  |  |
| Mirnov coils – toroidal array | √ |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE | √ |  |
| NPA – E¦¦B scanning |  |  |
| NPA – solid state |  |  |
| Neutron measurements | √ |  |
| Plasma TV | √ |  |
| Reciprocating probe |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – SPRED | √ |  |
| Spectrometer – VIPS |  |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray arrays | √ |  |
| Ultrasoft X-rays – bicolor | √ |  |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. | √ |  |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer - XEUS |  |  |