| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Comparison of H-mode fueling with supersonic gas injector and conventional gas injector |  |  |  |
| OP-XP-912 | Revision: | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: V. A. Soukhanovskii |  |  | Date 02/04/2009 |
| ATI – ET Group Leader: |  |  | Date |
| RLM - Run Coordinator: |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Comparison of H-mode fueling with supersonic gas injector and conventional gas injector | No. OP-XP-912 |
| --- | --- |
| AUTHORS: V. A. Soukhanovskii | DATE: 02/04/2009 |

# 1.	Overview of planned experiment

The goal of the experiment is to study H-mode fueling with the supersonic gas injector (SGI). The following points are to be addressed through a comparative analysis of fueling by the SGI and a conventional low field side (LFS) gas injector (CGI):

- Fueling efficiency

- Pedestal characteristics and pedestal fueling

- Response of wall and divertor recycling source to gas injection

- Formation of X-point MARFE

# 2.	Theoretical/ empirical justification

Previous SGI experiments in NSTX demonstrated the possibility of high-efficiency H-mode fueling with the SGI. We are seeking to compare it to a LFS CGI at identical plasma and machine conditions, to quantify the fueling efficiency estimates, particle balance, and to study wall and divertor response to the gas injection. Another objective is to study the H-mode pedestal fueling. Substantial experimental and theoretical work performed in large aspect ratio tokamaks implied that the H-mode density pedestal width and height are determined by neutral penetration length. We would like to document SGI and CGI fueled pedestal conditions so that analytical models of neutral penetration can be applied.

# 3.	Experimental run plan

## Comparison of SGI and CGI in discharge front-end (up to 5 shots)

- Run a fiducial 0.8 MA, 6 MW discharge with high field side fueling. Note the Injector # 2 time history and rate in the front-end of the discharge (first 150 ms).

- Replace the LFS injector with an identical or as similar as possible SGI pulse.

- The SGI will be operated at R=1.56 - 1.58 m.

- SGI setup: plenum pressure P0=2000-5000 Torr.

- Adjust SGI pulse duration or pressure as needed

## H-mode fueling with SGI and CGI (up to 10 shots)

- Obtain a reference H-mode discharge with reduced HFS fueling (up to 3 shots)

- Use the fiducial discharge with HFS fueling, keep outer gap 10 cm

- Start with HFS plenum pressure 300-500 Torr and SGI

  - SGI setup: R=1.56 - 1.58 m, Plenum pressure 5000 Torr, Timing to match the HFS fueling profile

- Adjust SGI timing to optimize H-mode discharge

- Repeat with CGI (Injector # 2) at identical rate and timing

- Use MPTS relative laser timing to get the “gas on – gas off” conditions

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

- SGI XMP  results available - need to know SGI injection rate calibration

- SGI plenum pressure up to 5000 Torr

- Wall / impurity conditions to allow reliable H-mode access

# 5.	Planned analysis

What analysis of the data will be required: LRDFIT, BFIT, possibly TRANSP.

# 6.	Planned publication of results

Results will be used to complete a manuscript summarizing SGI fueling on NSTX. It is planned to submit the manuscript to a major fusion journal in FY 2008.

PHYSICS OPERATIONS REQUEST

| TITLE: Comparison of H-mode fueling with supersonic gas injector and conventional gas injector | No. OP-XP-912 |
| --- | --- |
| AUTHORS: V. A. Soukhanovskii | DATE: 02/04/2009 |

(use additional sheets and attach waveform diagrams if necessary)

Describe briefly the most important plasma conditions required for the experiment: Standard fiducial discharge - unbalanced double null configuration, = 2.15-2.25, =0.7 -0.75, Outer gap ~ 10 cm, Drsep=-6-10 mm - 0.8 MA, 6 MW - HFS, Injector 2, SGI gas systems with D2

Previous shot(s) which can be repeated: 123931 (2007) Previous shot(s) which can be modified: 128797 (1 MA)

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): -53 kA Flattop start/stop (s): IP (MA): 0.80 MA Flattop start/stop (s): Configuration: LSN Equilibrium Control: Outer gap / Isoflux (rtEFIT) Outer gap (m): 0.09-0.10 Inner gap (m): 0.06-0.07 Z position (m): 0.0 Elongation : 2.15-2.25 Upper/lower triangularity : 0.70-0.75 Gas Species: D2 Injector(s): SGI, HFS, Inj. 2 NBI Species: D Voltage (kV) A: 90 B: 90 C: 90 Duration (s): 1 s ICRF Power (MW): 0 Phase between straps (°): Duration (s): CHI: Off Bank capacitance (mF): LITERs: Off Total deposition rate (mg/min): EFC coils: Off Configuration: Odd / Even / Other (attach detailed sheet

DIAGNOSTIC CHECKLIST

| TITLE: Comparison of H-mode fueling with supersonic gas injector and conventional gas injector | No. OP-XP-912 |
| --- | --- |
| AUTHORS: V. A. Soukhanovskii | DATE: 02/04/2009 |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array |  | √ |
| Bolometer – divertor |  | √ |
| CHERS – toroidal | √ |  |
| CHERS – poloidal | √ |  |
| Divertor fast camera |  | √ |
| Dust detector |  |  |
| EBW radiometers |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  |  |
| Edge pressure gauges | √ |  |
| Edge rotation diagnostic |  |  |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes | √ |  |
| FIReTIP | √ |  |
| Gas puff imaging |  |  |
| H camera - 1D | √ |  |
| High-k scattering |  |  |
| Infrared cameras | √ |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor | √ |  |
| Langmuir probes – BEaP |  | √ |
| Langmuir probes – RF ant. |  | √ |
| Magnetics – Diamagnetism |  |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes |  |  |
| Magnetics – Pickup coils | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents |  |  |
| Magnetics – RWM sensors |  |  |
| Mirnov coils – high f. |  |  |
| Mirnov coils – poloidal array |  |  |
| Mirnov coils – toroidal array |  |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE |  |  |
| NPA – E¦¦B scanning |  |  |
| NPA – solid state |  |  |
| Neutron measurements |  |  |
| Plasma TV |  |  |
| Reciprocating probe |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL | √ |  |
| RF edge probes |  |  |
| Spectrometer – SPRED | √ |  |
| Spectrometer – VIPS | √ |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray arrays |  | √ |
| Ultrasoft X-rays – bicolor |  | √ |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. |  | √ |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer - XEUS |  |  |