| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: FIDA Red/Blue Doppler Shift |  |  |  |
| OP-XP-917 | Revision: | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: W. Heidbrink |  |  | Date |
| ATI – ET Group Leader: |  |  | Date |
| RLM - Run Coordinator: |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: FIDA Red/Blue Doppler Shift | No. OP-XP-917 |
| --- | --- |
| AUTHORS: W.W. Heidbrink, M. Podesta | DATE: 1/20/09 |

# 1.	Overview of planned experiment

Use modulated neutral-beam injection at low voltage in plasmas with different values of toroidal and poloidal field to confirm a predicted asymmetry between the short and long-wavelength sides of the FIDA spectra.

# 2.	Theoretical/ empirical justification

Calculations indicate that there should be marked differences in the blue and red-shifted FIDA spectra caused by two special features of the ST: large poloidal field and large gyroradius.  The large poloidal field causes a large field-line pitch that, for a downward vertical view and co-passing ions in standard NSTX geometry, makes the blue-shifted signals larger than the red-shifted signals outside the magnetic axis (and vice versa inside).  In the presence of a gradient in the spatial profile of guiding centers, the large gyroradius causes the blue-shifted profile to shift inward, while the red-shifted spectra shift outward (for the standard clockwise toroidal field). To date, the predicted asymmetries are not observed in the data, casting doubt on the validity of the data (especially the red-shifted spectra).  MHD-quiescent plasmas (where the fast-ion distribution function is reasonably well known) with special neutral beam injection patterns are needed to understand this discrepancy.  Note that measurements with either reversed plasma current or reversed toroidal field are also highly desirable.

# 3.	Experimental run plan

Baseline condition: 0.8 MA, 5.5 kG, 3-4e13 cm-3, deuterium gas (~128742 –exact conditions not crucial).  Sources B and C at 65 keV; Source A at 90 keV.

1) Baseline Source B at 50% duty cycle (10 ms on/off).  Establish baseline.  Add 40 ms Source A pulse at beginning and  end of time of interest for MSE data.  (2-3 shots)

2) Toroidal Field Scan.  Lower toroidal field to 4.5 kG, then 3.5 kG.  (2 shots)

3) Plasma Current Scan. At 5.5 kG, lower current to 0.5 MA, then raise current to 1.2 MA (2 shots).

4) Source C.  In most interesting condition, substitute Source C for Source B (1 shot).

5) Filter Scan Repeat favorite case several times while scanning the angle of the f-FIDA bandpass filter (3 shots).

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Machine: nothing special.  Need fairly clean machine to avoid impurity contamination of FIDA spectra.

Beam:  Lower voltage operation than usual & beam modulation.

ICRF:  not needed.

Diagnostics:  Magnetics to verify quiet plasma; fast-ion (neutrons, NPA, SSNPA, sFLIP) to corroborate FIDA; plasma (Thomson scattering, CHERS) for theoretical fast-ion distribution function, MSE for field-line pitch.

# 5.	Planned analysis

TRANSP analysis to compute expected fast-ion distribution function.  This distribution function is input to the FIDA simulation code.

# 6.	Planned publication of results

If we get results that we understand, we will publish a short paper on these novel features of the fast-ion distribution function in an ST.

PHYSICS OPERATIONS REQUEST

| TITLE: FIDA Red/Blue Doppler Shift | No. OP-XP-917 |
| --- | --- |
| AUTHORS: W.W. Heidbrink, M. Podesta | DATE: 1/20/09 |

(use additional sheets and attach waveform diagrams if necessary)

Can be early in run but cannot tolerate unusually high impurity levels.

Previous shot(s) which can be repeated: 128742 Previous shot(s) which can be modified:

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): 42-66 Flattop start/stop (s): 0/0.75 IP (MA): 0.5-1.1 Flattop start/stop (s): 0.25/0.5 Configuration: Inner Wall Limiter (shape not crucial) Equilibrium Control: Operator’s preference Outer gap (m): 0.04 Inner gap (m): 0 Z position (m): 0 Elongation : 1.9 Upper/lower triangularity : 0.4 Gas Species: D Injector(s): Fuel from location that avoids H-mode NBI Species: D Voltage (kV) A: 90 B: 65 C: 65 Duration (s): Modulate ICRF Power (MW): 0 Phase between straps (°): Duration (s): CHI: Off Bank capacitance (mF): LITERs: Off Total deposition rate (mg/min): EFC coils: Off Configuration: Odd / Even / Other (attach detailed sheet

DIAGNOSTIC CHECKLIST

| TITLE: FIDA Red/Blue Doppler Shift | No. OP-XP- |
| --- | --- |
| AUTHORS: Heidbrink, Podesta | DATE:1/20/09 |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array |  |  |
| Bolometer – divertor |  |  |
| CHERS – toroidal |  | x |
| CHERS – poloidal |  | x |
| Divertor fast camera |  |  |
| Dust detector |  |  |
| EBW radiometers |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  | x |
| Edge pressure gauges |  |  |
| Edge rotation diagnostic |  |  |
| Fast ion D_alpha - FIDA | X |  |
| Fast lost ion probes - IFLIP |  | X |
| Fast lost ion probes - SFLIP |  | X |
| Filterscopes |  |  |
| FIReTIP |  | X |
| Gas puff imaging |  |  |
| H camera - 1D |  |  |
| High-k scattering |  |  |
| Infrared cameras |  |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  |  |
| Langmuir probes – BEaP |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – Diamagnetism |  |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes |  |  |
| Magnetics – Pickup coils | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents |  |  |
| Magnetics – RWM sensors |  |  |
| Mirnov coils – high f. | X |  |
| Mirnov coils – poloidal array | X |  |
| Mirnov coils – toroidal array | X |  |
| Mirnov coils – 3-axis proto. | X |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE |  | X |
| NPA – E¦¦B scanning | X |  |
| NPA – solid state | X |  |
| Neutron measurements | X |  |
| Plasma TV |  |  |
| Reciprocating probe |  |  |
| Reflectometer – 65GHz |  | X |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  | X |
| Reflectometer – fixed f |  | X |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – SPRED |  |  |
| Spectrometer – VIPS |  |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering | X |  |
| Ultrasoft X-ray arrays |  | X |
| Ultrasoft X-rays – bicolor |  |  |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. |  | X |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer - XEUS |  |  |