Proposed Shot plan for XP 918:

"Effect of Impurities and Wall Conditioning on NTMs" (F. Volpe), 0.5 day

a) Establish role of Lithization and, more generally, of good wall conditioning and of impurity content, in suppressing/preventing NTMs

- Repeat #133025 with 4MW NBI, n=3 EFC, n=1 DEFC, NO Lithium. Requires pre-emptive 1min D2 + 8min He glow discharge cleaning (GDC) . n=1 NTM expected. If not, tweak parameters (1 good shot).

- Repeat after Lithization (same evaporation rate as for 129125?). NTM suppression expected. If not, increase Li amount (1 good shot).

- Repeat for different amounts of Li. To avoid cumulative effects, 1min D2-GDC + 8min He-GDC between shots (3-4 shots).

- Repeat best shot over and over, but without re-lithizing every time, until first NTM appears. This is to assess duration of benefits of a single Lithization. He-GDC might be required between shots, to avoid disrupting during Ip ramp. (2-4 shots).

- If time, repeat "marginal shot" (i.e., NTM-stabilized shot that required the smallest amount of Li) with reduced n=3 EFC and/or reduced n=1 DEFC gains, to isolate their effects on NTMs (4-5 shots).

b) Deliberately seed impurities. Compared to a), scan of impurity content will be broader and yet partly decoupled from wall conditioning. Also, impurity is different, which is interesting to compare.

- Repeat best lithized shot with Argon puffed at the edge. Vary Ar puffing rate (2-3 shots).

- Repeat in non-lithized reference shot (2-3 shots).