| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Development and characterization of an intermediate δ discharge with lithium PFC coatings |  |  |  |
| OP-XP-919 | Revision: | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: |  |  | Date |
| ATI – ET Group Leader: |  |  | Date |
| RLM - Run Coordinator: |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: | No. OP-XP-919 |
| --- | --- |
| AUTHORS: Josh Kallman | DATE: |

# 1.	Overview of planned experiment

In order to study the effect of the LLD on particle pumping in NSTX and to utilize advanced control capabilities such as ELM triggering, an intermediate triangularity discharge will be developed and characterized. The discharge will run the strike point on the bull-nose tiles just outboard of the CHI gap and inboard of the LLD. The discharge will be studied under baseline pre-Li, LITER, and then LLD conditions. The development and study of such a discharge will allow a controlled characterization of how lithium PFCs affect the plasma and what benefits they can provide.

# 2.	Theoretical/ empirical justification

Solid lithium coatings have shown many performance benefits in NSTX, but the rises in hydrogenic and impurity density remain problematic. New diagnostics will provide the capability to better diagnose these phenomena and the LLD will later provide a more powerful tool for using lithium in NSTX. Characterization of heat fluxes will show how lithium modifies the SOL properties in this plasma shape.

# 3.	Experimental run plan

1. Pre-Li baseline - 3 discharges

2. Lithium evaporation at sufficient rate to establish ELM-suppressed discharge with fueling to prevent discharge collapse due to Li pumping.Characterization of discharge parameters under LITER operation once robust equilibrium established – 10 discharges

3. Scan in input power to find H-mode threshold and observe changes in heat flux (3@3MW,  3@4MW, 3@5MW)– 9 discharges

4. Scan in fueling to search for effects on density and radiation; start at all CS gas @ 3 or 4 MW of power, depending on results from above, then add SGI while reducing CS– 8 discharges

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Key prerequisite is development of intermediate triangularity discharge with controlled strike point location in XP by Koleman. NBI heating desired in order to reach H-mode. Diagnostics desired shown on attached.

# 5.	Planned analysis

EFIT for equilibrium reconstruction, TRANSP for transport analysis, and edge modeling codes to determine effect of LLD on discharge parameters (R, T_e, etc)

# 6.	Planned publication of results

Results will be published as thesis research, with incidental publications occurring as needed.

PHYSICS OPERATIONS REQUEST

| TITLE: | No. OP-XP-919 |
| --- | --- |
| AUTHORS: | DATE: |

Describe briefly the most important plasma conditions required for the XP:

Desire is to establish an ELM-free discharge with lithium present and with the strike point remaining at R=~.63 m (intermediate triangularity). This is the baseline for LLD operation and the goal is to characterize the impurity, density, and heat flux properties of this discharge:

A) without lithium

B) with evaporated lithium

C) with the LLD

This XP will fulfill parts A+B and will attempt to establish a stable equilibrium under the new shape parameters.

List any pre-existing shots:

132164

Equilibrium Control: Gap Control / rtEFIT(isoflux control):

Machine conditions (specify ranges as appropriate, use more than one sheet if necessary)

ITF (kA): 50	Flattop start/stop (s): 0/1

IP (MA): .8	Flattop start/stop (s): .2/.8

Configuration: Limiter / DN / LSN / USN  (strike out inapplicable cases)

Outer gap (m): 	.08	Inner gap (m): 	.13	Z position (m):

Elongation :	2.25	Upper/lower triangularity :	.55

Gas Species: 	D	Injector(s): CS, SGI

NBI Species: D	Voltages (kV or off)	A:	80	B: 80	C: 80	Duration (s):

ICRF Power (MW):		Phasing: 		Duration (s):

CHI:	Off / On	Bank capacitance (mF):

LITERs:	Off / On	Total deposition rate (mg/min): variable, but as necessary for ELM suppression, up to maximum of 80 mg/min.

EFC coils:	Off/On	Configuration: Odd / Even / Other (attach detailed sheet)

DIAGNOSTIC CHECKLIST

| TITLE: | No. OP-XP- |
| --- | --- |
| AUTHORS: | DATE: |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array | X |  |
| Bolometer – divertor | X |  |
| CHERS – toroidal | X |  |
| CHERS – poloidal | X |  |
| Divertor fast camera | X |  |
| Dust detector | X |  |
| EBW radiometers |  |  |
| Edge deposition monitors | X |  |
| Edge neutral density diag. | X |  |
| Edge pressure gauges | X |  |
| Edge rotation diagnostic |  | X |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes | X |  |
| FIReTIP | X |  |
| Gas puff imaging |  |  |
| H camera - 1D | X |  |
| High-k scattering |  |  |
| Infrared cameras | X |  |
| Interferometer - 1 mm | X |  |
| Langmuir probes – divertor | X |  |
| Langmuir probes – BEaP |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – Diamagnetism | X |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes | X |  |
| Magnetics – Pickup coils | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents |  |  |
| Magnetics – RWM sensors | X |  |
| Mirnov coils – high f. |  |  |
| Mirnov coils – poloidal array | X |  |
| Mirnov coils – toroidal array | X |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE |  | X |
| NPA – E¦¦B scanning |  |  |
| NPA – solid state |  |  |
| Neutron measurements |  |  |
| Plasma TV | X |  |
| Reciprocating probe |  | X |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL | X |  |
| RF edge probes |  |  |
| Spectrometer – SPRED | X |  |
| Spectrometer – VIPS | X |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering | X |  |
| Ultrasoft X-ray arrays | X |  |
| Ultrasoft X-rays – bicolor | X |  |
| Ultrasoft X-rays – TG spectr. | X |  |
| Visible bremsstrahlung det. | X |  |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer - XEUS | X |  |
| LoWEUS | X |  |