| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Ip Dependence of L-H threshold, Hysteresis and Confinement Quality |  |  |  |
| OP-XP-922 | Revision: 0 | Effective Date: Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: Stanley Kaye |  |  | Date 2/9/09 |
| ATI – ET Group Leader: Kevin Tritz |  |  | Date |
| RLM - Run Coordinator: Roger Raman |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Ip Dependence of L-H threshold, Hysteresis and Confinement Quality | No. OP-XP-922 |
| --- | --- |
| AUTHORS: S. Kaye, R. Maingi, W. Solomon | DATE: Feb. 9, 2009 |

# 1.	Overview of planned experiment

# The goal of this experiment is to the dependence of the L-H threshold power on plasma current, confinement quality for powers just above the threshold, and H-L hysteresis.

# 2.	Theoretical/ empirical justification

This XP studies the effect plasma current on the L-H threshold. This has direct bearing on developing an understanding of the thresholds in future devices at both low rotation (ITER) and at higher rotation (NHTX, ST-CTF). This study is an element of an ITPA JEX. It further assesses confinement quality for powers just above the threshold power (can H~1 be attained at this power level?) as well as the H-L hysteresis. Both these issues are the foci of ITPA JEXs.

# 3.	Experimental run plan

- Establish L-H threshold power in high-LSNdischarge (127267 baseline, but with Ip=700 kA). This shape was chosen in order to avoid OH H-modes. The power will be varied by either lowering the beam voltage, or modulating the source if necessary.

  - Use optimal n=3 error field correction (-300 A in SPA1) and n=1 mode control

- Maintain plasma discharge at P~PLH (once in H-mode) for as long as possible

  - Assess confinement quality

  - Determine H-L hysteresis as density increases (does Pheat drop below “new” PLH?)

- Vary Ip in two additional steps (150 kA each step)

  - Establish threshold, H, H-L hysteresis in same manner at each level (may not be possible at highest current)

- Use 10-15 mg/min Li evaporation.

- No HeGDC between shots.

- Start with Source A on 10 ms after Ip flattop reached

  - Do not use preheat unless necessary to achieve good discharge conditions. If necessary, preheat with Source A from 80 to 140 ms.

TABULAR SHOT LIST

| # Condition | # Ip (MA) | # BT (T) | # SPA 1 Current (+low-n feedback) |
| --- | --- | --- | --- |
| # 1 | # 0.7 | # 0.45 | # -300 A (optimum) |
| # 2 | # 0.850 | # 0.45 | # -300 A (optimum) |
| # 3 | # 1.0 | # 0.45 | # -300 A (optimum) |

# Total: ~12 shots

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Discharge reproducibility, ability to achieve H-mode with one source.

# 5.	Planned analysis

EFIT, TRANSP, specialized codes

# 6.	Planned publication of results

Joule milestone, TTF, ITPA, IAEA

PHYSICS OPERATIONS REQUEST

| TITLE: Ip Dependence of L-H threshold, Hysteresis and Confinement Quality | No. OP-XP-922 |
| --- | --- |
| AUTHORS: S. Kaye, R. Maingi, W. Solomon | DATE: Feb. 9, 2009 |

Machine conditions (specify ranges as appropriate)

ITF (kA): 52 (4.5 kG)	Flattop start/stop (s):

IP (MA): 0.7, 0.85, 1.0	Flattop start/stop (s):

Configuration: LSN

Outer gap (m): 		Inner gap (m):

Elongation : ~1.9	Upper/lower triangularity : ~0.65

Z position (m):	0

Gas Species: D		Injector(s):

NBI Species: D	Sources: 	3	Voltage (kV): 	55-80 (B, C), 90 (A)	Duration (s): <0.6 s

ICRF Power (MW):	0	Phasing: 		Duration (s):

CHI:	Off	Bank capacitance (mF):

LITER:  On

Either:	List previous shot numbers for setup:	127267

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc.

A

A (+B) or C

200 ms

Start without Source A preheat; use only if necessary.

DIAGNOSTIC CHECKLIST

| TITLE: Ip Dependence of L-H threshold, Hysteresis and Confinement Quality | No. OP-XP-922 |
| --- | --- |
| AUTHORS: S. Kaye, R. Maingi, W. Solomon | DATE: Feb. 9, 2009 |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array | √ |  |
| Bolometer – divertor |  |  |
| CHERS – toroidal | √ |  |
| CHERS – poloidal | √ |  |
| Divertor fast camera |  |  |
| Dust detector |  |  |
| EBW radiometers |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  | √ |
| Edge pressure gauges |  | √ |
| Edge rotation diagnostic | √ |  |
| Fast ion D_alpha - FIDA | √ |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  | √ |
| Filterscopes | √ |  |
| FIReTIP |  | √ |
| Gas puff imaging |  | √ |
| H camera - 1D |  | √ |
| High-k scattering |  | √ |
| Infrared cameras |  |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  | √ |
| Langmuir probes – BEaP |  |  |
| Langmuir probes – RF ant. | √ |  |
| Magnetics – Diamagnetism | √ |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes | √ |  |
| Magnetics – Pickup coils | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents | √ |  |
| Magnetics – RWM sensors | √ |  |
| Mirnov coils – high f. | √ |  |
| Mirnov coils – poloidal array | √ |  |
| Mirnov coils – toroidal array | √ |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE |  | √ |
| NPA – ExB scanning |  |  |
| NPA – solid state |  | √ |
| Neutron measurements | √ |  |
| Plasma TV | √ |  |
| Reciprocating probe |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  | √ |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – SPRED | √ |  |
| Spectrometer – VIPS | √ |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray arrays | √ |  |
| Ultrasoft X-rays – bicolor |  | √ |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. | √ |  |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer - XEUS |  | √ |