| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Effect of Rotation on the L-H Threshold |  |  |  |
| OP-XP-936 | Revision: | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: Stanley Kaye |  |  | Date |
| ATI – ET Group Leader: Kevin Tritz |  |  | Date |
| RLM - Run Coordinator: Roger Raman |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Effect of Rotation on the L-H Threshold | No. OP-XP- |
| --- | --- |
| AUTHORS: S. Kaye | DATE: 4/16/09 |

# 1.	Overview of planned experiment

# The goal of this experiment is to study the effect of rotation on the L-H threshold power. n=3 braking will be used to vary the rotation.

# 2.	Theoretical/ empirical justification

This XP studies the effect of rotation, and by extension rotational shear at the edge, on the L-H threshold. This has direct bearing on developing an understanding of the effect of this edge rotational shear and how it might impact thresholds in future devices at both low rotation (ITER) and at higher rotation (NHTX, ST-CTF). Results on MAST indicate that the L-H transition is delayed with increasing applied n=3 field, although the real reason for this is not known. Applied n=1 fields on MAST showed no change. This study is an element of an ITPA JEX.

# 3.  Experimental run plan

- Establish L-H threshold power in high-LSNdischarge (132717 baseline) using one source (use B or B+C as needed, initially); vary voltage in source to achieve threshold. If necessary modulate source to lower power even farther

  - Use optimal n=3 error field correction (-300 A in SPA1) and n=1 mode control

  - Source C will allow lower power (voltage) if necessary

- Vary n=3 SPA coil current to 0 A, 400 A, 800 A (adjust levels as necessary to avoid disruptions)

  - Establish threshold in same manner at each level.

- Use 10-15 mg/min Li evaporation

- No HeGDC between shots.

- Shorten shot to 500-600 ms to reduce shot cycle to 10 minutes if possible

TABULAR SHOT LIST

| # Condition | # Ip (MA) | # BT (T) | # SPA 1 Current (steady) |
| --- | --- | --- | --- |
| # 1 | # 0.8 | # 0.45 | # -300 A (optimum) |
| # 2 | # 0.8 | # 0.45 | # 0 A |
| # 3 | # 0.8 | # 0.45 | # 400 A |
| # 4 | # 0.8 | # 0.45 | # 800 A |

# Total: 12 good shots

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Discharge reproducibility, including ability to achieve H-mode.

.

# 5.	Planned analysis

EFIT, TRANSP, specialized codes

# Planned publication of results

TTF, ITPA, IAEA

TTF, ITPA, IAEA PHYSICS OPERATIONS REQUEST

| TITLE: Effect of Rotation on the L-H Threshold | No. OP-XP- |
| --- | --- |
| AUTHORS: S. Kaye | DATE: 4/16/09 |

Describe briefly the most important plasma conditions required for the XP:

Discharge reproducibility achieving the L-H transition.

List any pre-existing shots: 132717

Equilibrium Control: Gap Control / rtEFIT(isoflux control):

Machine conditions (specify ranges as appropriate, use more than one sheet if necessary)

ITF (kA): 52 (4.5 kG)	Flattop start/stop (s):

IP (MA): 0.8 MA	Flattop start/stop (s):

Configuration: LSN

Outer gap (m): 	0.09	Inner gap (m): 	0.38	Z position (m): 	0.00

Elongation :	~2	lower triangularity : ~

Gas Species: D		Injector(s):	as in 132717

NBI Species: D	Voltages (kV)   A:	90	  B:  var	C:  var	    Duration (s): on @ 190 ms

ICRF Power (MW):	0	Phasing: 		Duration (s):

CHI:	Off	Bank capacitance (mF):

LITERs:	On	Total deposition rate (mg/min): as in 132717

EFC coils:	On	Configuration:	optimum EF correction to start, n=1 f/b

Coils on at 150 ms.

DIAGNOSTIC CHECKLIST

| TITLE: Effect of Rotation on the L-H Threshold | No. OP-XP- |
| --- | --- |
| AUTHORS: S. Kaye | DATE: 4/16/09 |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array | √ |  |
| Bolometer – divertor |  |  |
| CHERS – toroidal | √ |  |
| CHERS – poloidal | √ |  |
| Divertor fast camera |  |  |
| Dust detector |  |  |
| EBW radiometers |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  |  |
| Edge pressure gauges |  |  |
| Edge rotation diagnostic |  |  |
| Fast ion D_alpha - FIDA |  | √ |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes | √ |  |
| FIReTIP |  |  |
| Gas puff imaging |  |  |
| H camera - 1D |  |  |
| High-k scattering |  | √ |
| Infrared cameras |  |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  |  |
| Langmuir probes – BEaP |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – Diamagnetism |  | √ |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes |  | √ |
| Magnetics – Pickup coils | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents |  |  |
| Magnetics – RWM sensors | √ |  |
| Mirnov coils – high f. | √ |  |
| Mirnov coils – poloidal array | √ |  |
| Mirnov coils – toroidal array | √ |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE | √ |  |
| NPA – ExB scanning |  |  |
| NPA – solid state |  |  |
| Neutron measurements | √ |  |
| Plasma TV |  |  |
| Reciprocating probe |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  | √ |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – SPRED |  | √ |
| Spectrometer – VIPS |  |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray arrays | √ |  |
| Ultrasoft X-rays – bicolor |  | √ |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. |  | √ |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer - XEUS |  | √ |