| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: L-H Threshold Power: D vs He |  |  |  |
| OP-XP-941 | Revision: | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: Stanley Kaye |  |  | Date |
| ATI – ET Group Leader: Kevin Tritz |  |  | Date |
| RLM - Run Coordinator: Roger Raman |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: L-H Threshold Power: D vs He | No. OP-XP-941 |
| --- | --- |
| AUTHORS: S. Kaye | DATE: 7/21/09 |

# 1.	Overview of planned experiment

# The goal of this experiment is to study the species dependence of the L-H transition using HHFW in D vs He plasmas.

# 2.	Theoretical/ empirical justification

This study is related to a high priority ITPA issue, as the first operation phase of ITER will be in non-Deuterium plasmas. The hope is to perform target scenario development in He plasmas, but L-H access in He needs to be demonstrated on present day devices. Asdex-Upgrade has shown the L-H threshold power for He to be the same as that for D, but that is the only experiment so far that has performed this study. Results from other devices are needed.

# 3.  Experimental run plan

- Re-establish L-H transition in HHFW-heated only plasma, discharge=130632 (PLH~0.85 MW)

  - Start in Helium (~ 2.0e15 cm-2); inject 1.5 MW of HHFW to establish transition condition

  - Perform binary search for transition, increasing or decreasing power as necessary

    - Might need to increase or decrease outer gap and/or density

  - Redo in Deuterium

- Use optimal n=3 error field correction (-300 A in SPA1) and n=1 mode control

  - Source C will allow lower power (voltage) if necessary

- Use 15-20 mg/min Li evaporation

- Inject source A shortly after transition (after transition time is established) for CHERs, etc.

TABULAR SHOT LIST

| # Condition | # Species | # HHFW Power |
| --- | --- | --- |
| # 1 | # He | # Start at 1.5 MW |
| # 2 | # D | # Start at PLH from above |

# Total: 12 good shots (anticipate 6 to 10 shots in each species)

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

Discharge reproducibility, including ability to achieve H-mode.

.

# 5.	Planned analysis

EFIT, TRANSP, specialized codes

# Planned publication of results

TTF, ITPA, IAEA

NSTX PHYSICS OPERATIONS REQUEST

| TITLE: L-H Threshold Power: D vs He | No. OP-XP-941 |
| --- | --- |
| AUTHORS: S. Kaye | DATE: 7/21/09 |

Describe briefly the most important plasma conditions required for the XP:

Discharge reproducibility achieving the L-H transition.

List any pre-existing shots: 130632

Equilibrium Control: Gap Control / rtEFIT(isoflux control):

Machine conditions (specify ranges as appropriate, use more than one sheet if necessary)

ITF (kA): 63 (5.5 kG) ?	Flattop start/stop (s):

IP (MA): 0.8 MA	Flattop start/stop (s):

Configuration: LSN

Outer gap (m): 		Inner gap (m): 		Z position (m): 	0.00

Elongation :	~2	lower triangularity : ~

Gas Species: D, then He	Injector(s):

NBI Species: D	Voltages (kV)   A:	90	  B:  var	C:  var	    Duration (s):

ICRF Power (MW):	<=3 MW	Phasing:  -150 deg    Duration (s): 220 ms

CHI:	Off	Bank capacitance (mF):

LITERs:	On	Total deposition rate (mg/min): 15-20 mg/min (as in 130632?)

EFC coils:	On	Configuration:	optimum EF correction to start, n=1 f/b

as in 130632

DIAGNOSTIC CHECKLIST

| TITLE: L-H Threshold Power: D vs He | No. OP-XP-941 |
| --- | --- |
| AUTHORS: S. Kaye | DATE: 7/21/09 |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array | √ |  |
| Bolometer – divertor |  |  |
| CHERS – toroidal |  |  |
| CHERS – poloidal |  |  |
| Divertor fast camera |  |  |
| Dust detector |  |  |
| EBW radiometers |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  |  |
| Edge pressure gauges |  |  |
| Edge rotation diagnostic |  |  |
| Fast ion D_alpha - FIDA |  | √ |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes | √ |  |
| FIReTIP |  |  |
| Gas puff imaging |  |  |
| H camera - 1D |  |  |
| High-k scattering |  | √ |
| Infrared cameras |  |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  |  |
| Langmuir probes – BEaP |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – Diamagnetism |  | √ |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes |  | √ |
| Magnetics – Pickup coils | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents |  |  |
| Magnetics – RWM sensors | √ |  |
| Mirnov coils – high f. | √ |  |
| Mirnov coils – poloidal array | √ |  |
| Mirnov coils – toroidal array | √ |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE | √ |  |
| NPA – ExB scanning |  |  |
| NPA – solid state |  |  |
| Neutron measurements | √ |  |
| Plasma TV |  |  |
| Reciprocating probe |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  | √ |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – SPRED |  | √ |
| Spectrometer – VIPS |  |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray arrays | √ |  |
| Ultrasoft X-rays – bicolor |  | √ |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. |  | √ |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer - XEUS |  | √ |