| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Early error-field correction in long-pulse plasmas |  |  |  |
| OP-XP-954 | Revision: 0 | Effective Date: July 27, 2009 (Approval date unless otherwise stipulated) Expiration Date: July 27, 2011 (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: Jon Menard |  |  | Date July 27, 2009 |
| ATI – ET Group Leader: David Gates |  |  | Date |
| RLM - Run Coordinator: Roger Raman |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Early error-field correction in long-pulse plasmas | No. OP-XP-9xx |
| --- | --- |
| AUTHORS: J. Menard, S. Gerhardt, D. Gates | DATE: 07/27/2000 |

# 1.	Overview of planned experiment

The combination of active suppression of n=1 RFA, n=3 pre-programmed error field correction (EFC), and LITER have produced record plasma pulse-durations in NSTX.  High elongation + EFC + LITER have produced record poloidal beta and record low flux-consumption in NSTX.   Further decreases in flux consumption are possible with increased non-inductive fraction and/or increased conductivity.  Reduced density could increase the NBI CD efficiency, and if Te increases at reduced density, density reduction could increase the conductivity and further increase NBI-CD.  This experiment will attempt to systematically lower the plasma density while avoiding disruptive MHD activity by optimizing early error-field correction to reduce mode locking.  This experiment will focus on discharges with little or no late MHD activity to simplify NBICD analysis, and with sufficient flat-top that the inductive and non-inductive profiles become equilibrated.

# 2.	Theoretical/ empirical justification

Reduced density is predicted to increase beam current drive efficiency, and higher Te at reduced density could further increase NBI-CD efficiency and conductivity – all resulting in increased pulse duration and higher non-inductive fraction plasmas.  Higher non-inductive fraction is important for improved operation of NSTX and NSTX-Upgrade and is essential to future ST devices such as NHTX and ST-CTF. Variations in plasma density should modify the beam current drive fraction at fixed heating power, thereby providing data for the ITPA IOS group for validating beam current drive models.  Finally, this experiment is important for the anticipated operation of the liquid lithium divertor (LLD) which could result in reduced early density and locked-modes if the LLD acts as an effective pump.

# Experimental run plan

- Reproduce a long-pulse and late-MHD-free discharge at 750-800kA		          	(3 shots)

  - Start with evaporation rate = 10mg/min

    - Use LITER shutter to fix evaporation duration at 10min with 12min shot cycle

    - Start from 129125, or recent discharge with 1.2-1.4s period without low-n MHD

    - Use 16cm outer gaps to reduce impurity density and Zeff

  - Increase evaporation rate to 20mg/min

    - Increase fueling as needed to achieve similar early density and long pulse duration

- Modify early fueling to trigger early MHD event/mode locking

  - Reduce early NBI power if beta-limit is reached, but maintain early H-mode

  - Start n=3 EF correction at t=20ms with gain = 200A/10kA (IRWM/IPF5)		(3 shots)

    - Assess locking behavior and rotation modification from n=3 EFC

  - With n=3 EFC off, turn on n=1 OHxTF EFC from XP 614 (120335) 		(3 shots)

    - Ramp-down OHxTF EFC by t=400ms

    - Assess locking behavior and rotation modification from n=3 EFC

  - Combine early n=3 and n=1 EFC, and assess mode-locking and rotation		(3 shots)

- Further reduce early fueling until mode locking/MHD occurs again				(4 shots)

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

The usual diagnostic capabilities are required, NBI voltage on A, B, C = 90, 90, 80kV.

# 5.	Planned analysis

EFIT/LRDFIT, TRANSP, MPTS, CHERS, and RWM/EF sensor analysis will be performed.

# 6.	Planned publication of results

Results will be published in conference proceedings and/or journal such as Nuclear Fusion or Physics of Plasmas within one year of experiment.

PHYSICS OPERATIONS REQUEST

| TITLE: Early error-field correction in long-pulse plasmas | No. OP-XP-9xx |
| --- | --- |
| AUTHORS: J. Menard, S. Gerhardt, D. Gates | DATE: 07/27/2009 |

Machine conditions (specify ranges as appropriate)

ITF (kA):       up to 63kA	     		Flattop start/stop (s):    0.0, ~2s

IP (MA):       0.75 or 0.8MA	     	Flattop start/stop (s):    0.2, ~2s

Configuration:      DN / LSN

Outer gap (m): 	  15cm	Inner gap (m): 	                   2-8cm

Elongation :	  2.4-2.7	Upper/lower triangularity :	0.75 / 0.5

Z position (m):	   0cm

Gas Species: 	   D	Injector(s):     CS midplane, outer midplane, SGI

NBI Species: D	Sources:    A, B, C       Voltage (kV):    90, 90, 80kV	Duration (s):  2s

ICRF Power (MW):	   0	Phasing: 	0	Duration (s):      0

CHI:	         Off	Bank capacitance (mF):

LITER:      On – 20 mg/min

Either:	List previous shot numbers for setup:	129125 or more recent discharge

Or:	Sketch the desired time profiles, including inner and outer gaps, , , heating, fuelling, etc. as appropriate. Accurately label the sketch with times and values.

DIAGNOSTIC CHECKLIST

| TITLE: Early error-field correction in long-pulse plasmas | No. OP-XP-9xx |
| --- | --- |
| AUTHORS: J. Menard, S. Gerhardt, D. Gates | DATE: 07/27/2009 |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array |  |  |
| Bolometer – divertor |  |  |
| CHERS – toroidal | X |  |
| CHERS – poloidal | X |  |
| Divertor fast camera |  |  |
| Dust detector |  |  |
| EBW radiometers |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  |  |
| Edge pressure gauges |  |  |
| Edge rotation diagnostic |  |  |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes |  |  |
| FIReTIP |  |  |
| Gas puff imaging |  |  |
| H camera - 1D |  |  |
| High-k scattering |  |  |
| Infrared cameras |  |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  |  |
| Langmuir probes – BEaP |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – Diamagnetism | X |  |
| Magnetics – Flux loops | X |  |
| Magnetics – Locked modes | X |  |
| Magnetics – Pickup coils | X |  |
| Magnetics – Rogowski coils | X |  |
| Magnetics – Halo currents |  |  |
| Magnetics – RWM sensors | X |  |
| Mirnov coils – high f. | X |  |
| Mirnov coils – poloidal array | X |  |
| Mirnov coils – toroidal array | X |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE | X |  |
| NPA – ExB scanning |  |  |
| NPA – solid state |  |  |
| Neutron measurements | X |  |
| Plasma TV |  |  |
| Reciprocating probe |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – SPRED |  |  |
| Spectrometer – VIPS |  |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering | X |  |
| Ultrasoft X-ray arrays | X |  |
| Ultrasoft X-rays – bicolor |  |  |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. |  |  |
| X-ray crystal spectrom. – H |  |  |
| X-ray crystal spectrom. – V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer – XEUS |  |  |