| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Basic Operation with Reversed TF |  |  |  |
| OP-XP-955 | Revision: 0 | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: D. Mueller |  |  | Date |
| ATI – ET Group Leader: |  |  | Date |
| RLM - Run Coordinator: R. Raman |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Basic Operation with Reversed TF | No. OP-XP-955 |
| --- | --- |
| AUTHORS: D. Mueller, M. Bell, R. Maingi | DATE: 7/29/2009 |

# 1.	Overview of planned experiment

The toroidal field direction on NSTX will be changed from its normal, i.e. clockwise viewed from above, to the reversed, i.e. counter-clockwise viewed from above, direction.  The goals of this experiment are to:

- Establish standard plasma operation with reversed TF;

- Compare the NB-heated LSN fiducial plasma with reversed TF to that with normal TF;

- Compare this fiducial plasma with the sign of the EFIT control parameter DRSEP changed to move the dominant divertor from the bottom to the top of NSTX;

- Perform USN, LSN and DN discharges and assess their performance, including differences in H-Mode power threshold, ELMs and MHD activity.

# Theoretical/empirical justification

This work is a prerequisite for other physics investigations planned to exploit reversed-TF operation.

# 3.	Experimental run plan

Shots	Total

1)	Run recent LSN fiducial shot with reversed TF with both 4 and 6 MW NBI,	4	4
repeating as necessary to optimize the startup and gas puffing, outer gap = 10cm.

2)	Change sign of DRSEP after start-up phase and run shots with 4 and 6 MW NBI 	4	8

3)	Run LSN shot 117747 repeating as necessary to optimize the startup and gas puffing	2	10

4)	Perform power scan 4.5, 3.5, 3, 2 MW to get H-Mode threshold for LSN	6	16

5)	Run USN shot 117750 repeating as necessary to optimize the startup and gas puffing	2	18

6)	Perform power scan 4.5, 3.5, 3, 2 MW to get H-Mode threshold for USN	6	24

7)	Run DN shot 117745 repeating as necessary to optimize the startup and gas puffing	2	26

8)	Perform power scan 4.5, 3.5, 3, 2 MW to get H-Mode threshold for DN	6	32

9)	If N >4.5 in steps 1) or 2), using the plasma with highest N, perform an n=3 error field 	6	38

correction scan using Table 1 below.

10) Using the “best” value of n=3 correction from 9), perform a scan of the phase of the 	6	44

n=1 feedback at 0, 90, 180, and 270 deg. using the standard gains from 134808.

11) Fine tune scans in 4), 6) and 8) to get threshold power more accurately as time allows	6	50

12)	Add lithium powder starting before t=0 and continuing through the shot for  the	4	54
best conditions established for the USN discharges

13)	Add lithium powder starting before t=0 and continuing through the shot for  the	2	56
best conditions established for a complementary LSN discharges

14) Using the best performing shot from 10), perform an outer gap scan 	7	63

(10 , 7, 13, 16, 19 cm), taking care to increase the early outer gap to match the

flattop value.

TABLE 1.

| # SPA1 (A) | # SPA2 (A) | # SPA3 (A) | # Shot No. |
| --- | --- | --- | --- |
| # 0 | # 0 | # 0 |  |
| # -250 | # 250 | # 250 |  |
| # 250 | # 250 | # -250 |  |
| # -500 | # -500 | # 500 |  |
| # 500 | # 500 | # -500 |  |
| # -750 | # -750 | # 750 |  |

# 4. Required machine, NBI, RF, CHI and diagnostic capabilities

TF reversed and ISTP for operating with it completed.

NBI A @2 MW, B @1.5 MW,  C @1 MW with possible changes in voltage, including 6 MW total power

Apply lithium evaporation at a rate of 10 mg/min and run on a 12.5 min shot cycle.

One lithium powder dropper, preferably both, is required for steps 10) and 11).

# 5.	Planned analysis

EFIT and TRANSP

# 6.	Planned publication of results

These results will intrinsically contribute to NSTX publications and inform other experiments.

PHYSICS OPERATIONS REQUEST

| TITLE: Basic Operation with Reversed TF | No. OP-XP-955 |
| --- | --- |
| AUTHORS: D. Mueller, M. Bell, R. Maingi | DATE: 7/29/2009 |

(use additional sheets and attach waveform diagrams if necessary)

Describe briefly the most important plasma conditions required for the experiment: Reversed TF

Previous shot(s) which can be repeated: 117745 (DN), 117747 (LSN), 117750 (USN) Previous shot(s) which can be modified: 133808 for DRSEP scan

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): 53 Flattop start/stop (s): -0.02/1.4 IP (MA): 0.6–0.8 Flattop start/stop (s): Configuration: USN, LSN and DN Equilibrium Control: Outer gap / Isoflux (rtEFIT) Outer gap (m): 0.1 Inner gap (m): 0.01 Z position (m): variable Elongation : 2.0-2.4 Upper/lower triangularity : 0.4 Gas Species: D Injector(s): all NBI Species: D Voltage (kV) A: 90 B: 90 C: 90 Duration (s): 1.4 ICRF Power (MW): 0 Phase between straps (°): Duration (s): CHI: Off Bank capacitance (mF): LITERs: On Total deposition rate (mg/min): 10 EFC coils: On Configuration: Odd

DIAGNOSTIC CHECKLIST

| TITLE: Basic Operation with Reversed TF | No. OP-XP-955 |
| --- | --- |
| AUTHORS: D. Mueller, M. Bell, R. Maingi | DATE: 7/29/2009 |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array |  | √ |
| Bolometer – divertor | √ |  |
| CHERS – toroidal | √ |  |
| CHERS – poloidal | √ |  |
| Divertor fast camera | √ |  |
| Dust detector |  |  |
| EBW radiometers |  |  |
| Edge deposition monitors |  | √ |
| Edge neutral density diag. |  |  |
| Edge pressure gauges | √ |  |
| Edge rotation diagnostic |  | √ |
| Fast ion D_alpha - FIDA |  | √ |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes | √ |  |
| FIReTIP |  | √ |
| Gas puff imaging |  |  |
| H camera - 1D |  | √ |
| High-k scattering |  | √ |
| Infrared cameras |  | √ |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  |  |
| Langmuir probes – BEaP |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – Diamagnetism | √ |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes | √ |  |
| Magnetics – Pickup coils | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents |  |  |
| Magnetics – RWM sensors | √ |  |
| Mirnov coils – high f. |  | √ |
| Mirnov coils – poloidal array |  | √ |
| Mirnov coils – toroidal array |  | √ |
| Mirnov coils – 3-axis proto. |  | √ |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE |  | √ |
| NPA – E¦¦B scanning |  | √ |
| NPA – solid state |  | √ |
| Neutron measurements |  | √ |
| Plasma TV | √ |  |
| Reciprocating probe |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – SPRED |  | √ |
| Spectrometer – VIPS |  | √ |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray arrays |  | √ |
| Ultrasoft X-rays – bicolor |  | √ |
| Ultrasoft X-rays – TG spectr. |  | √ |
| Visible bremsstrahlung det. | √ |  |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer - XEUS |  | √ |