| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: L-H threshold and edge transport and turbulence in NSTX reversed Bt discharges |  |  |  |
| OP-XP-956 | Revision: | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: S. Kaye, R. Maingi, V. Soukhanovskii, K. Tritz |  |  | Date |
| ATI – ET Group Leader: |  |  | Date |
| RLM - Run Coordinator: |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: L-H threshold and edge transport and turbulence in NSTX reversed Bt discharges | No. OP-XP-956 |
| --- | --- |
| AUTHORS: S. Kaye, R. Maingi, V. Soukhanovskii, K. Tritz | DATE: |

# 1.	Overview of planned experiment

The goals of the experiment is to conduct initial physics scoping studies of the reversed Bt effects on edge, scrape-off layer and divertor heat and particle transport and turbulence: 1) L-H power thresholds in NBI-heated discharges, 2) divertor and SOL transport and turbulence in L- and H-mode discharges in low and high triangularity configurations.

# 2.	Theoretical/ empirical justification

Brief justification of activity, including supporting calculations as appropriate. Describe briefly any previous or related experiments.

# 3.	Experimental run plan

### SOL, divertor, and pedestal characterization in low-δ discharges – 1 day

- Setup: Use discharge 135169 (or long pulse version of 132721 with a larger drsep ~ -2-3 cm) as template (0.8 MA, 5 MW, δL=0.4, drsep =-1 cm, GAPOUT=10 cm) and results from Part 1.

- Scan PNBI between 1 MW and 6 MW in steps of 1 MW to map out L- and H-mode operational space boundaries. Optimize measurements of edge profiles by small radial jogs, as needed.

- In an H-mode discharge (at high PNBI ), scan Ip between 0.6 and 1.0 MA, 0.2 MA increments

- In an 0.8 MA H-mode discharge, attempt to obtain radiative divertor

- Use Bay E divertor gas injector at pressures 2000-5000 Torr, pulse duration 100-200 ms

- (Optional, time permitting) Scan LITER rate between 10 and 200 mg/min to study pumping characteristics

- (Optional-time permitting) Use SGI to study ion tp* at several LITER rates

### L-H threshold studies – 1 day

- Reproduce low dL discharge with ohmic ramp-up (132721), with 50-100mg Lithium between discharges, i.e. just enough to avoid HeGDC

- Measure PLH through NBI voltage scans, using src C at lowest voltage, and B at intermediate

- Repeat with heavy Lithium 200-300 mg between discharges

- Create USN version of this discharge, and measure PLH with low lithium, and heavy lithium

### SOL, divertor, and pedestal characterization in high-δ discharges – 1 day

- Use most recent fiducial discharge as template (0.8 MA, 4-6 MW, δL=0.8, drsep =-1 cm, GAPOUT=10 cm)

- Scan PNBI between 1 MW and 6 MW in steps of 1 MW.

- In an H-mode discharge (at high PNBI ), scan Ip between 0.8 and 1.2 MA

- In an 1.0 MA H-mode discharge, attempt to obtain radiative divertor

- Use Bay E divertor gas injector at pressures 2000-5000 Torr, pulse duration 100-200 ms

- (optional, time permitting) repeat for a 1.2 MA discharge

- (Optional, time permitting) Scan LITER rate between 10 and 200 mg/min to study pumping characteristics

- (Optional-time permitting) Use SGI to study ion tp* at several LITER rates

## Optional, time permitting studies

- Perform PNBI and IP scans in high-d and low-d double-null configuration.

- Use RMP in several discharges to study pumping / ELM behaviour

- Obtain “snowflake” divertor, use discharge template from XP 924

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

The experiment assumes that 1) plasma facing component conditioning has been performed to enable plasma operations, and 2) reliable reversed Bt plasma operations have been established under a separate XP.

Both LITERs, SGI, and Bay E divertor gas injector will be required.

# 5.	Planned analysis

What analysis of the data will be required: EFIT, LRDFIT, TRANSP, UEDGE, SOLPS, DEGAS 2, SOLT, BOUT

# 6.	Planned publication of results

Results will be presented at the upcoming PSI and ITPA meetings if appropriate.

PHYSICS OPERATIONS REQUEST

| TITLE: L-H threshold and edge transport and turbulence in NSTX reversed Bt discharges | No. OP-XP-956 |
| --- | --- |
| AUTHORS: S. Kaye, R. Maingi, V. Soukhanovskii, K. Tritz | DATE: |

(use additional sheets and attach waveform diagrams if necessary)

Describe briefly the most important plasma conditions required for the experiment:

Previous shot(s) which can be repeated: Previous shot(s) which can be modified:

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): Flattop start/stop (s): IP (MA): Flattop start/stop (s): Configuration: Limiter / DN / LSN / USN Equilibrium Control: Outer gap / Isoflux (rtEFIT) Outer gap (m): Inner gap (m): Z position (m): Elongation : Upper/lower triangularity : Gas Species: Injector(s): NBI Species: D Voltage (kV) A: B: C: Duration (s): ICRF Power (MW): Phase between straps (°): Duration (s): CHI: Off / On Bank capacitance (mF): LITERs: Off / On Total deposition rate (mg/min): EFC coils: Off/On Configuration: Odd / Even / Other (attach detailed sheet

DIAGNOSTIC CHECKLIST

| TITLE: … reversed Bt discharges … | No. OP-XP-956 |
| --- | --- |
| AUTHORS: S. Kaye, R. Maingi, V. Soukhanovskii, K. Tritz | DATE: |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array |  |  |
| Bolometer – divertor |  |  |
| CHERS – toroidal |  |  |
| CHERS – poloidal |  |  |
| Divertor fast camera |  |  |
| Dust detector |  |  |
| EBW radiometers |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  |  |
| Edge pressure gauges |  |  |
| Edge rotation diagnostic |  |  |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes |  |  |
| FIReTIP |  |  |
| Gas puff imaging |  |  |
| H camera - 1D |  |  |
| High-k scattering |  |  |
| Infrared cameras |  |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  |  |
| Langmuir probes – BEaP |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – Diamagnetism |  |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes |  |  |
| Magnetics – Pickup coils | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents |  |  |
| Magnetics – RWM sensors |  |  |
| Mirnov coils – high f. |  |  |
| Mirnov coils – poloidal array |  |  |
| Mirnov coils – toroidal array |  |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE |  |  |
| NPA – E¦¦B scanning |  |  |
| NPA – solid state |  |  |
| Neutron measurements |  |  |
| Plasma TV |  |  |
| Reciprocating probe |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – SPRED |  |  |
| Spectrometer – VIPS |  |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering |  |  |
| Ultrasoft X-ray arrays |  |  |
| Ultrasoft X-rays – bicolor |  |  |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. |  |  |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer - XEUS |  |  |