| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Optimization of ELM pace-making with 3D fields |  |  |  |
| OP-XP-943 | Revision: | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: |  |  | Date |
| ATI – ET Group Leader: |  |  | Date |
| RLM - Run Coordinator: |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Optimization of ELM pace-making with 3D fields | No. OP-XP-943 |
| --- | --- |
| AUTHORS: J.M. Canik, A.C. Sontag | DATE: |

# 1.	Overview of planned experiment

The goal of the experiment is to optimize fuelling and the use of 3D fields to trigger ELMs for discharge control during lithium-enhanced ELM free H-modes.  The plasma shape, the waveform of the n=3 field application, and the particle fuelling will be optimized with a goal of producing a discharge with stationary density and radiated power, with small triggered ELMs.  In this addendum, the plasma current will be reduced to reproduce the smallest ELM case from XP 809, and the improved waveform of XP 943 will be applied.

# 2.	Theoretical/ empirical justification

The waveform and shape optimization parts of XP 943 have been performed.  The waveform has been improved so that the ELM frequency has been increased to over 60 Hz with 100% triggering efficiency, with data taken at elongations up to ~2.55.  However, initial analysis of these experiments show that the triggered ELM size remains large at ΔW/W ~ 10%.  This is actually somewhat larger than the best case from XP 809 in the last campaign.   Several differences exist in the recent experiments compared to those of XP 809, and this Addendum will explore the impact of these changes on ELM size.  First, it is hypothesized that the larger ELMs may be partially due to the higher plasma currents of these experiments (1 MA compare to the 800 kA of last years XP), since peeling-ballooning modes tend to have larger radial extent at low q, and so the improved waveform from the initial run XP 943 will be applied to the same low-current target discharge used in XP 809.  The waveform used in XP 943 differs primarily in the amplitude of the n=3 pulses (3 kA in 943 compared to 1.2 kA last year).  In the present experiments, the amplitude will be scanned to test if the pulses themselves are contributing to the size of the triggered ELMs.  From the first run of XP 943, a dependence of the triggered ELM size was clear at lower frequency (20 Hz vs 40 Hz), but the dependence at high frequency was weak (50 Hz vs. 62.5 Hz).  Since these  frequency scans were performed over a variety of shapes and heating powers, a controlled scan of frequency with all else fixed is needed to clarify the dependence of ELM size on the period of the triggering.

# 3.	Experimental run plan

1. Produce reference discharge (3 shots)

  - Reload 133720: Bt=0.45 T, k=2.3, d=0.8; change Ip to 800 kA, drsep to ~0, PNBI to 4 MW

  - Start with LITER at 100 mg/shot, increase in increments of 100 mg/shot until ELM-free conditions are reached

  - If satisfactory conditions can’t be reached at drsep~0, change drsep to -5 mm

2. Frequency scan (8 shots)

  - Use high amplitude, short duration n=3 pulses (3 kA, 4 ms)

  - Trigger ELMs using pulse frequencies of 60, 10, 20, 40, 77 Hz

  - Fast current reversal of each pulse may be necessary at the highest frequency to reduce rotation braking

3. Pulse amplitude scan (5 shots)

  - Frequency fixed at highest achieved in section 2

  - Trigger ELMs using pulse amplitudes of 2, 1.5, 1.0 kA

  - Longer pulses may be required at low SPA currents

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

This XP requires a fully operational NBI system.  We desire LITER operating at an evaporation rate of 20 mg/min with a 10 minute shot cycle.

# 5.	Planned analysis

EFIT/LRFDIT; pedestal profile analysis to use in kinetic BFIT; stability calculations with PEST and ELITE; spectral analysis using vacuum approximation and IPEC

# 6.	Planned publication of results

The results of this experiment will be presented during an invited talk at the APS 09 meeting if the nomination is accepted, and will be published in the corresponding Physics of Plasmas issue.

PHYSICS OPERATIONS REQUEST

| TITLE: Characterization of magnetically triggered ELMs in lithium conditioned discharges | No. OP-XP-943 |
| --- | --- |
| AUTHORS: J.M. Canik, A.C. Sontag | DATE: |

(use additional sheets and attach waveform diagrams if necessary)

Describe briefly the most important plasma conditions required for the experiment: Reliable ELM-free operation with lithium conditioning is required.

Previous shot(s) which can be repeated: 130669,130652 Previous shot(s) which can be modified:

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): 53 Flattop start/stop (s): 0/1 IP (MA): 0.8 Flattop start/stop (s): .15/1 Configuration: Limiter / DN / LSN / USN Equilibrium Control: Outer gap / Isoflux (rtEFIT) Outer gap (m): 0.1 Inner gap (m): 0.05 Z position (m): Elongation : 2.4 Upper/lower triangularity : 0.7 Gas Species: D Injector(s): NBI Species: D Voltage (kV) A: 90 B: 60 C: 90 Duration (s): 1 ICRF Power (MW): Phase between straps (°): Duration (s): CHI: Off / On Bank capacitance (mF): LITERs: Off / On Total deposition rate (mg/min): 40 EFC coils: Off/On Configuration: Odd / Even / Other (attach detailed sheet

DIAGNOSTIC CHECKLIST

| TITLE: Characterization of magnetically triggered ELMs in lithium conditioned discharges | No. OP-XP-943 |
| --- | --- |
| AUTHORS: J.M. Canik, A.C. Sontag | DATE: |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array | √ |  |
| Bolometer – divertor |  | √ |
| CHERS – toroidal | √ |  |
| CHERS – poloidal |  | √ |
| Divertor fast camera |  | √ |
| Dust detector |  |  |
| EBW radiometers |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  | √ |
| Edge pressure gauges |  | √ |
| Edge rotation diagnostic |  | √ |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes | √ |  |
| FIReTIP |  | √ |
| Gas puff imaging |  | √ |
| H camera - 1D | √ |  |
| High-k scattering |  |  |
| Infrared cameras | √ |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor | √ |  |
| Langmuir probes – BEaP |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – Diamagnetism | √ |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes | √ |  |
| Magnetics – Pickup coils | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents | √ |  |
| Magnetics – RWM sensors | √ |  |
| Mirnov coils – high f. |  | √ |
| Mirnov coils – poloidal array |  | √ |
| Mirnov coils – toroidal array |  | √ |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE |  | √ |
| NPA – E¦¦B scanning |  |  |
| NPA – solid state |  |  |
| Neutron measurements |  | √ |
| Plasma TV |  | √ |
| Reciprocating probe |  |  |
| Reflectometer – 65GHz |  | √ |
| Reflectometer – correlation |  | √ |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL |  | √ |
| RF edge probes |  |  |
| Spectrometer – SPRED |  | √ |
| Spectrometer – VIPS |  | √ |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray arrays | √ |  |
| Ultrasoft X-rays – bicolor |  | √ |
| Ultrasoft X-rays – TG spectr. |  | √ |
| Visible bremsstrahlung det. |  | √ |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer - XEUS |  |  |