| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Optimization of long-pulse high-bT discharges |  |  |  |
| OP-XP-938 | Revision: | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: S.P. Gerhardt, D.A. Gates |  |  | Date |
| ATI – ET Group Leader: D. A. Gates |  |  | Date |
| RLM - Run Coordinator: R. Raman |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| Chit Review Board (designated by Run Coordinator) |  |  |  |
| MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Optimization of long-pulse high-bT discharges | No. OP-XP-948 |
| --- | --- |
| AUTHORS: S.P. Gerhardt, D.A. Gates | DATE: |

# 1.	Overview of planned experiment

The goal of this experiment is to expand the high-bT operational space achieved in NSTX. This experiment will follow the traditional method for achieving high-b: the plasma current will be raised as the toroidal field is reduced, and high elongation will be used to keep q*>~2.2. More specifically, the recent high-bP shot 133964, which operated at 700 kA and 0.47T, will be used as an initial target, though at 900 kA and 0.45 T. The toroidal field will then be dropped to 0.4 T and discharges made with IP values of 700, 900,1000, 1100, and 1200 kA, all at the maximum NB power. This should result in a series of discharges at increasing values of bT, but decreasing pulse duration; the scaling of confinement and NI fraction with plasma current could will be determined. If time permits one or two of these IP values will be repeated at both lower (0.375 or 0.35 T) and higher (0.475 or 0.55 T) toroidal fields, in order to study the scaling of transport and NI fraction with BT in these high-k, low-li plasmas.

# 2.	Theoretical/ empirical justification

Most designs that make use of the ST concept to make something useful (say, a CTF or a reactor) operate at substantially higher toroidal b than is common in NSTX: CTFs may operate at bT in the rage of 18-28, while an ST reactor may need bT=50%! Furthermore, the elongation in these studies typically is 3 or higher. It is the purpose of the present XP to push the present NSTX operating space to the longest available pulses at high-bT and low-q*, using a high-k discharge that should be optimal for increasing the NI fraction.

Fig 1: The NSTX database of flat-top average bT values, sorted by year. Also shown in red circles are the target points for this XP at BT=0.4T, assuming good confinement.

A further important topic for these equilibria is to understand how the transport and non-inductive current fraction scale with IP and BT in these high-k, low-li discharges. Though these studies are not the primary goal of the XP, the subject will be addressed in the scans to be conducted.

In preparation for this XP, a series of target equilibrium were studied with the isolver code. The profiles and shape were taken from 133964, the value of bN was fixed at 5.5, and IP and BT were scanned. The resulting equilibrium characteristics are shown in table 1.  The lines in red indicate the equilibrium at BT=0.4 T that are of primary interest for this XP.

| Case | Ip (MA) | BT (T) | betaN | betaT (%) | q* | q95 | WMHD (kJ) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0.7 | 0.5 | 5.5 | 13.6 | 4.78 | 14 | 260 |
| 2 | 0.7 | 0.45 | 5.5 | 14.8 | 4.4 | 12.5 | 234 |
| 3 | 0.7 | 0.4 | 5.5 | 16.4 | 4 | 11.1 | 208 |
| 4 | 0.7 | 0.375 | 5.5 | 17.4 | 3.8 | 10.4 | 195 |
| 5 | 0.7 | 0.35 | 5.5 | 18.5 | 3.57 | 9.7 | 183 |
| 6 | 0.9 | 0.5 | 5.5 | 17 | 3.9 | 10.7 | 330 |
| 7 | 0.9 | 0.45 | 5.5 | 18.5 | 3.57 | 9.7 | 300 |
| 8 | 0.9 | 0.4 | 5.5 | 20.5 | 3.23 | 8.6 | 270 |
| 9 | 0.9 | 0.375 | 5.5 | 21.8 | 3.05 | 8.1 | 252 |
| 10 | 0.9 | 0.35 | 5.5 | 23 | 2.87 | 7.5 | 235 |
| 11 | 1 | 0.55 | 5.5 | 17 | 3.9 | 10.7 | 410 |
| 12 | 1 | 0.5 | 5.5 | 18.5 | 3.6 | 9.7 | 373 |
| 13 | 1 | 0.45 | 5.5 | 20.3 | 3.3 | 8.7 | 337 |
| 14 | 1 | 0.4 | 5.5 | 22.5 | 2.9 | 7.7 | 300 |
| 15 | 1 | 0.375 | 5.5 | 23.9 | 2.7 | 7.3 | 281 |
| 16 | 1 | 0.35 | 5.5 | 25.3 | 2.6 | 6.8 | 263 |
| 17 | 1.1 | 0.55 | 5.5 | 18.5 | 3.6 | 9.7 | 452 |
| 18 | 1.1 | 0.5 | 5.5 | 20 | 3.3 | 8.8 | 411 |
| 19 | 1.1 | 0.45 | 5.5 | 22 | 3 | 7.9 | 371 |
| 20 | 1.1 | 0.4 | 5.5 | 24.6 | 2.7 | 7.1 | 330 |
| 21 | 1.1 | 0.375 | 5.5 | 26.2 | 2.6 | 6.63 | 310 |
| 22 | 1.1 | 0.35 | 5.5 | 28 | 2.4 | 6.2 | 289 |
| 23 | 1.2 | 0.55 | 5.5 | 19.5 | 3.3 | 8.88 | 493 |
| 24 | 1.2 | 0.5 | 5.5 | 21 | 3.1 | 8.1 | 449 |
| 25 | 1.2 | 0.45 | 5.5 | 24 | 2.8 | 7.3 | 405 |
| 26 | 1.2 | 0.4 | 5.5 | 26.7 | 2.5 | 6.5 | 361 |
| 27 | 0.2 | 0.375 | 5.5 | 28.4 | 2.4 | 6.1 | 339 |
| 28 | 1.2 | 0.35 | 5.5 | 30 | 2.2 | 5.7 | 317 |
| 29 | 1.3 | 0.5 | 5.5 | 21.4 | -3.1 | 8.2 | 537 |
| 30 | 1.3 | 0.5 | 5.5 | 23.4 | 2.8 | 7.5 | 487 |
| 31 | 1.3 | 0.45 | 5.5 | 25.8 | 2.6 | 6.7 | 439 |
| 32 | 1.3 | 0.4 | 5.5 | 28.8 | 2.3 | 6 | 391 |
| 33 | 1.3 | 0.375 | 5.5 | 30.6 | 2.2 | 5.6 | 367 |
| 34 | 1.3 | 0.35 | 5.5 | 32 | 2.05 | 5.2 | 343 |

Table #1: Characteristics of the high-k discharge at different IP and BT.

# 3.	Experimental run plan

For reference, here is a table of possible BT waveforms and a figure showing their durations:

Figure #2: Example TF waveforms, along with their approach to the I2t limit of the coil.

| Shot | ITF (kA) | BT (T) | I2t (kA2s) | SOFT (msec) |
| --- | --- | --- | --- | --- |
| 125074 | -64.2 | -0.534 | 4437 | -92 |
| 124579 | -58.5 | -0.49 | 5684 | -116 |
| 133964 | -56.2 | -0.47 | 4657 | -39 |
| 134256 | -52.7 | -0.44 | 4639 | -29 |
| 129125 | -48.3 | -0.4 | 4513 | -71 |
| 123987 | -47.3 | -0.395 | 3548 | -81 |
| 129126 | -45.3 | -0.38 | 4331 | -98 |
| 123975 | -40.9 | -0.34 | 2683 | -138 |

Table #2: Selected TF waveforms to reload; some will require modification. The I2t limit on the OH coil is 4750 kA2s. SOFT is the start of flat top, and should be about -40 msec. In a few cases (125074,124579), the TF waveform should be delayed in order to maximize the flat-top after t=0.

Regarding Fueling: LITER rate appropriate for 300 mg/shot, using two evaporators. Use 12.5 minute shot cycle, no GDC. For the LFS gas, can use waveforms from 133964 (700kA), 132592 (1MA), or 132912 (1.3 MA). For the HFS gas, the approximate scaling of plenum fill pressure in torr, with IP (MA) is 1050+400*IP. However, this is for guidance only, and the actual required amount may vary depending on conditions at the time.

3.1) Reload shot 133964. Reduce the outer gap to ~13 cm, from 16 cm. Reduce BT to 0.45 Tesla and increase IP to 900 kA (can simply use the waveforms for the fiducial). May need a few shots to get the fuelling right.

3.2) Reduce BT to 0.4 Tesla (TF waveform from 129125) and extend the IP flat-top duration such that the current is ramped all the way down by 1.6 seconds. Fill in the following table of shots at this toroidal field.

The plasma current ramp rate is to be left unchanged as IP is raised; shots with higher IP will simply reach flat-top that much later.

To first order, the center-stack gas should be increased/decreased in a proportion to IP, as noted above. Care should be taken to track changing fuelling requirements through the day.

It may be desirable to increase the NB power to 95kV on sources B and C.

Once the desired condition is reached, the shot should be repeated at least once.

| IP | BT | Shots |
| --- | --- | --- |
| 900 | 0.4 |  |
| 1000 | 0.4 |  |
| 1100 | 0.4 |  |
| 1200 | 0.4 |  |
| 1300 | 0.4 |  |
| 700 | 0.4 |  |

3.3) Pick a value of plasma current from table #1 and scan the toroidal field.

| IP | BT | Shots |
| --- | --- | --- |
|  | 0.55 |  |
|  | 0.475 |  |
|  | 0.375 |  |
|  | 0.35 |  |

3.4) Pick a value of plasma current from table #1 and scan the toroidal field

| IP | BT | Shots |
| --- | --- | --- |
|  | 0.55 |  |
|  | 0.475 |  |
|  | 0.375 |  |
|  | 0.35 |  |

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

May request sources B & C to be run at 95 kV. Would use b-feedback if it is available, but it is not a requirement. However, all three NB sources are required. RWM feedback is a requirement. Dual LITER operations and 30mg/min total rate is a requirement.

# 5.	Planned analysis

EFIT/LRDFIT equilibrium analysis, TRANSP for non-inductive current and transport analysis, DCON for ideal stability limit calculations

# 6.	Planned publication of results

These results will be part of the report for the ASC milestone for 2009. They will be presented at the 2009 APS meeting, and likely go into a NF paper.

PHYSICS OPERATIONS REQUEST

| TITLE: Optimization of long-pulse high-bT discharges | No. OP-XP-948 |
| --- | --- |
| AUTHORS: S.P. Gerhardt, D.A. Gates | DATE: |

(use additional sheets and attach waveform diagrams if necessary)

Describe briefly the most important plasma conditions required for the experiment: This XP does not require any special conditions. Need 2 LITER units to be functional, all three NB sources working, and RWM feedback on-line.

Previous shot(s) which can be repeated: Previous shot(s) which can be modified: 133964

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): 40-65 Flattop start/stop (s): See table #2 IP (MA): 700-1.3 Flattop start/stop (s): Variable Configuration: LSN, though drsep is so small that it is almost DN Equilibrium Control: Isoflux (rtEFIT) Outer gap (m): 12 Inner gap (m): 6 Z position (m): -0.02 Elongation : 2.5 Upper/lower triangularity : PF1A shot Gas Species: D2 Injector(s): 1& 2 NBI Species: D Voltage (kV) A: 90 B: 90 (95?) C: 90 (95?) Duration (s): 1.6 ICRF Power (MW): 0 Phase between straps (°): Duration (s): CHI: Off Bank capacitance (mF): LITERs: On Total deposition rate (mg/min): 30 EFC coils: On Configuration: Odd

DIAGNOSTIC CHECKLIST

| TITLE: Optimization of long-pulse high-bT discharges | No. OP-XP-948 |
| --- | --- |
| AUTHORS: S.P. Gerhardt, D.A. Gates | DATE: |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Bolometer – tangential array | √ |  |
| Bolometer – divertor |  |  |
| CHERS – toroidal | √ |  |
| CHERS – poloidal |  |  |
| Divertor fast camera |  |  |
| Dust detector |  |  |
| EBW radiometers |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. | √ |  |
| Edge pressure gauges | √ |  |
| Edge rotation diagnostic |  |  |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes | √ |  |
| FIReTIP |  |  |
| Gas puff imaging |  |  |
| H camera - 1D | √ |  |
| High-k scattering |  |  |
| Infrared cameras | √ |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  |  |
| Langmuir probes – BEaP |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – Diamagnetism | √ |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes | √ |  |
| Magnetics – Pickup coils | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents | √ |  |
| Magnetics – RWM sensors | √ |  |
| Mirnov coils – high f. | √ |  |
| Mirnov coils – poloidal array |  |  |
| Mirnov coils – toroidal array | √ |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE | √ |  |
| NPA – E¦¦B scanning |  |  |
| NPA – solid state |  |  |
| Neutron measurements | √ |  |
| Plasma TV | √ |  |
| Reciprocating probe |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – SPRED | √ |  |
| Spectrometer – VIPS |  |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray arrays | √ |  |
| Ultrasoft X-rays – bicolor | √ |  |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. |  |  |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray fast pinhole camera |  |  |
| X-ray spectrometer - XEUS | √ |  |