| Princeton Plasma Physics Laboratory NSTX Machine Proposal |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Title: Optimization of bN Control Gains |  |  |  |  |  |  |
| OP-XMP-65 |  | Revision: 0 |  |  | Effective Date: Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| Procedure Approvals |  |  |  |  |  |  |
| Responsible author: Stefan Gerhardt |  |  |  |  |  | Date 3/11/2010 |
| ATI (NSTX Physics Ops): |  |  |  |  |  | Date |
| RLM (NSTX Expt. Research Ops): |  |  |  |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |  |  |  |
| Procedure Requirements designated by RLM |  |  |  |  |  |  |
|  | NSTX Work Permit |  |  | T-MOD (OP-AD-03) |  |  |
|  | Independent Review |  |  | ES&H Review |  |  |
| RESTRICTIONS AND MINOR MODIFICATIONS Approved by RLM |  |  |  |  |  |  |

| REVIEWERS (designated by RLM) |  |  |
| --- | --- | --- |
| Organization/Position | Name | Signature |
| ATI | D. Mueller |  |
| Test Director |  |  |
| Independent Reviewer |  |  |
| NB system |  |  |
| RF systems |  |  |
| FCPC systems |  |  |
| Diagnostics |  |  |

| TRAINING (designated by RLM) Training required: No  Yes  Instructor _____________________ |  |  |  |
| --- | --- | --- | --- |
| Personnel (group, job title or individual name) | Read Only | Instruction | Hands-On |
| RLM _______________________________ |  |  |  |

NSTX MACHINE PROPOSAL

| TITLE: Optimization of bN Control Gains | No. OP-XMP-65 |
| --- | --- |
| AUTHORS: Gerhardt, et al. | DATE: 3/11/2010 |

## 1.	Overview:

Beta control was first implemented in NSTX during the 2009 run, for a limited number of discharges. This system compares the present value of a request for the normalized b, to the value computed by rtEFIT, and then adjusts the NB power to reduce this difference to zero. The system performed well in the single instance of its use, but was not optimized.  During the 2009 outage, some small changes woo the algorithm were made.

The “Beta Normal Feedback” (or bnf) algorithm for 2010 is based on the following PID equations.

e=βN,reqeust−LPFβN,RTEFIT;tLPF

Pinj=PβNCβNe+IβNCβNedt+DβNCβNdedtCβN=1000⋅t⋅IPVBT200m0a

Shots from 2008 with large steps in the beam power have been used to select reasonable value of P & I for the controller, using the Ziegler-Nichols “Process Reaction Method”. The purpose of the present XMP is to test that the values of P and I so determined are reasonable, and make any necessary further adjustments. It is not clear that derivative gain is actually useful, given the noise on the bN signal (the subscript bN on the gains is dropped for the remainder of this document).

The PID scheme noted above produces an analog power request, with an additional block of code required to convert this power request to source modulations. There is also a direct analog power request capability built into the algorithm: the PIC specifies the requested power as a function of time as an input waveform, and PCS determines the beam modulations to match this power (on average). This block of code will be tested as part of the XMP.

If this XMP is successfully completed, the bN control will be available for use as a general tool.

Note that the algorithm also has contingency for bT and WMHD control. However, these controls will not be tested as part of the present XMP.

## 2.	Justification:

NSTX is known to have large fluctuations in the normalized beta and confinement transients that can cause stability limits to be exceeded (followed by disruption). Both of these problems can hopefully be mitigated by using beta control. If this is indeed successful, a large number of experiments in the MHD and ASC groups will benefit. Use of this system is also important for the FY-10 MHD milestone on techniques for reducing disruptivity

## 3.	Plan:

3.0: Off-line testing of the code against idl equivalent code.

The entire algorithm has been duplicated in idl, in order to provide a direct check of the outputs. The  bnf algorithm will be run on the 2nd control computer during plasma operations, using the input data from the actual shot, and the outputs tested against those computed by the idl code.

3.1 Gain optimization

3.1.1 Load 800 kA, BT=0.45, with the shape from the 2009 high-delta morning fiducial. Establish discharge to reasonable long pulse.

Note 1: Might be desirable to further lower IP in order to stave off late rotating MHD. However, it is also desirable to maintain good absorption of source C, motivating the choice of 800 kA.

Note 2: If there remain mhd problems, raising the TF to 0.48 T may be beneficial. A reasonable waveform to reload is from 133964. Be sure to ramp down IP earlier in this case.

3.1.2: Notify the NBOS that beta feedback will be used, and that they must plug in the cable on the back of the chassis.

3.1.3: Turn on beta-control, with a waveform similar to that in Fig 3.1. Use gains P=2 and I=0, with 20 msec on/off times. Check that:

____ Gains are correct (P=0.75, D=0, I=0)

____ The batting order matrix is correct.

Batting Order = 123132231

____ The “Max Blocks Default” correct: [1,1,1]

____ The source power waveforms have the same values as the power of the actual sources.

____ That feedback is turned on at the correct time.

____ That source A is NOT allowed the modulate, but that B & C are specified as used for feedback.

____ That source A is asked to be on the entire time.

____ That the feedback window has the correct timing.

____ Beams enabled waveform set high

____ Deadband for bN feedback set to zero.

____ Maximum number of blocks per source set to 18.

____Target filter time constant ramping between 0.2 and 0.25 seconds, from t=0 to t=0.01 sec.

Shots:________ __________ __________

Fig 1: Proposed waveform for the bN request during gain optimization.

3.1.4: Increase the proportional and integral gains as in the table below. Only one shot per condition is required, and the three shots per row are contingency for failures.

| P | I | Shot #1 | Shot #2 | Shot #3 |
| --- | --- | --- | --- | --- |
| 1 | 10 |  |  |  |
| 1 | 20 |  |  |  |
| 1 | 30 |  |  |  |
| 2 | 20 |  |  |  |

3.2 Test of more rapid modulations

3.2.1 Repeat optimal case with 15 msec on/off modulations. May need to modify batting order.

Shots:________ __________ __________

3.3 Test of pre-programmed power ramps

3.2.1 Turn off feedback and replace it with an injected power request of (approximately) the following shape. PCS should calculate modulations to achieve, on average, this power level.

Shots:________ __________ __________

Fig. 2: Proposed waveform for the Pinj request test.

4.	Required machine, beam, ICRF and diagnostic capabilities:

Standard profile diagnostics and operations magnetics. RWM control would be helpful in order to increase the discharge reliability. LITER operation at ~150-200 mg/shot is also requested.

## 5.	Sign off at run time:

5.1	Permission to Proceed:

Physics Operations Head

5.2	Documentation of results:

Documentation of the results completed, attached to proposal and sent to Ops. Center with copies to Cognizant Physicist and Head of Physics Operations.

Cognizant Physicist/Test Director

PHYSICS OPERATIONS REQUEST

| TITLE: Optimization of bN Control Gains | No. OP-XMP-65 |
| --- | --- |
| AUTHORS: Gerhardt, et al. | DATE: 3/11/2010 |

Brief description of the most important operational plasma conditions required: 1: IP flat-top of >800 msec without mhd. This allows a long period for testing the step response of the system. 2: RWM control available. This should improve the shot-to-shot reliability at higher values of bN.

Previous shot(s) which can be repeated: Previous shot(s) which can be modified: Any 2009 fiducial-like shot, say 135643

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): 0.45-0.48 kG Flattop start/stop (s): Standard front end, go to max. I2t IP (MA): 700-800 Flattop start/stop (s): standard ramp rate, and use full OH and TF. Configuration: 2009 high-k, high-d shape, which is slightly biased down DN. rtEFIT controls: Isoflux, no need for S.P. control at high-d. Outer gap (m): 10-15cm Inner gap (m): few cm Z position (m): 0<Z<-2 cm Elongation: ~2.3 Triangularity (U/L): 0.5/0.7 OSP radius (m): high-d Gas Species: D2 Injector(s): Standard prefill + LFS + HFS (unless SGI becomes the working standard) NBI Species: D Voltage (kV) A: 90 B: 90 C: 90 Duration (s): ~1-1.1 sec. ICRF Power (MW): 0 Inter-strap phase (°): Duration (s): CHI: Off Bank capacitance (mF): LITERs: On Total deposition rate (mg/min): rate to get ~200 mg/shot LLD: No Temperature (°C): Room Temperature EFC/RWM coils: n=1 Feedback + n=3 correction Configuration: Odd

DIAGNOSTIC CHECKLIST

| TITLE: Optimization of bN Control Gains | No. OP-XMP-65 |
| --- | --- |
| AUTHORS: Gerhardt, et al. | DATE: 3/11/2010 |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Beam Emission Spectroscopy |  |  |
| Bolometer – divertor |  |  |
| Bolometer – midplane array | √ |  |
| CHERS – poloidal |  |  |
| CHERS – toroidal | √ |  |
| Dust detector |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  |  |
| Edge pressure gauges |  |  |
| Edge rotation diagnostic |  | √ |
| Fast cameras – divertor/LLD | √ |  |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes | √ |  |
| FIReTIP |  |  |
| Gas puff imaging – divertor |  |  |
| Gas puff imaging – midplane |  |  |
| H camera - 1D |  |  |
| High-k scattering |  |  |
| Infrared cameras |  |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  |  |
| Langmuir probes – LLD |  |  |
| Langmuir probes – bias tile |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – B coils | √ |  |
| Magnetics – Diamagnetism | √ |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents |  | √ |
| Magnetics – RWM sensors | √ |  |
| Mirnov coils – high f. | √ |  |
| Mirnov coils – poloidal array |  |  |
| Mirnov coils – toroidal array | √ |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE | √ |  |
| NPA – E¦¦B scanning |  |  |
| NPA – solid state |  |  |
| Neutron detectors | √ |  |
| Plasma TV | √ |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – divertor |  |  |
| Spectrometer – SPRED |  |  |
| Spectrometer – VIPS |  |  |
| Spectrometer – LOWEUS |  |  |
| Spectrometer – XEUS |  |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray – pol. arrays |  | √ |
| Ultrasoft X-rays – bicolor |  |  |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. | √ |  |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray tang. pinhole camera |  |  |