| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Test of bN-Control for Disruptivity Reduction |  |  |  |
| OP-XP-1019 | Revision: 0 | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: Stefan Gerhard. J. M. Canik, D. Gates, R. Maingi, J. Menard, S. Sabbagh |  |  | Date |
| ATI – ET Group Leader: S. Sabbagh |  |  | Date |
| RLM - Run Coordinator: E. Fredrickson |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| RESTRICTIONS or MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: b-Control for Disruptivity Reduction | No. OP-XP-1019 |
| --- | --- |
| AUTHORS: Gerhardt, et al. | DATE: |

# 1.	Overview of planned experiment

The purpose of this experiment is to test the efficacy of the bN-control system for 1) reducing disruptivity and 2) minimizing stored energy transients, both across confinement transitions and during otherwise steady-state conditions. Discharges that are unstable without bN control will be run with bN  control on, to determine the ability of the control system to interdict ideal instabilities. The required time response of the bN control system will be tested by scanning the time-constant of the causal low-pass filter on the rtEFIT bN value.

# 2.	Theoretical/ empirical justification

It is desirable to run an ST with the highest bN, in order to maximize the non-inductive current fraction (for constant q); there is typically an upper bound on the achievable bN due to instabilities such as the RWM. NSTX has historically run with feed-forward control of the injected power. This has been successful in achieving high-noninductive fractions and reliable scenarios. However, it has often been necessary to reduce the injected power, in order to ensure that confinement transients do not push bN above stability limits. This problem can be especially pernicious if those confinement transients are sustained as improved confinement regimes. It is hoped that feedback control of the injected power, in order to achieve a pre-programmed value of the requested bN,, will allow more reliability operation near bN limits and reduced disruptivity across confinement transients.

# 3.	Experimental run plan

3.1 Development of target.

3.1.1 Target is high-k, high-d shape used in the 2009 fiducial discharge. Likely use IP=800 kA and BT=0.42 T, Pinj=6MW. These parameters are historically prone to disruption.                                 (2 shots)

Shot Numbers ______ ______ ______ ______

3.1.2 Reduce power to 4 MW. Add square n=3 pulses at t=0.4, 3 kA amplitude, 3 msec long.  If the pulse does not trigger a transition to Enhanced Pedestal H-mode (EP H-mode), then i) move pulse to t=0.5 seconds or ii) increase current to 900 kA.  If improved confinement regime occurs, plasma will likely disrupt. If no transition occurs, then discharge should be stable.                                                     (2 shots)

Shot Numbers ______ ______ ______ ______

Decision Point #1: If step 3.1.2 generated a transition to an enhanced confinement regime, the go on the “EPH Path” steps below using the shot with the transition as a reference. Otherwise use these as 4MW and 6 MW pre-programmed power references cases for the “HM Path”.

3.2 EPH Path: Scan of bN.

IC 3.2.1: Turn on bN control, with a requested value equal to the value of bN before the IC transition. Test that the beam power is indeed reduced after the transition.                                                          (2 shots)

Shot Numbers ______ ______ ______ ______

IC 3.2.2: Because gains scale like (1/confinement)2 for a similar response of the bN control system, reduce proportional and integral gains during period after transition if necessary.                                 (0-2 shots)

Shot Numbers ______ ______ ______ ______

IC 3.2.3: Increase the requested bN in units of 0.5 during the IC phase until disruption occurs. Bracket the stability limit.                                                                                                                                    (5 shots)

In all cases, leave the early part of the shot alone in order to ensure the regime transition is repeatable.

| Shot # | bN,req | Comment |
| --- | --- | --- |

3.2 HM (=H-Mode) Path: Scan of bN.

HM 3.2.1: Note the bN  level of the disruptive case at 6 MW (bN,6MW) and the bN level of the non-disruptive case at 4MW (bN,4MW). Turn on bN control with a request of bN,req=(bN,6MW+bN,4MW)/2.                     (2 shots)

Shot Numbers ______ ______ ______ ______

HM 3.2.2: Use a bisection like approach to determine the bN request that the plasma can tolerate. (5 shots)

| Shot # | Average Power | bN,req | typicalbN | Disruption |
| --- | --- | --- | --- | --- |
|  | 6 MW | NA |  |  |
|  | 4MW | NA |  |  |

3.3 Scan of filter time constant.

3.3.1 Take a typical stable case from 3.2 and repeat it. Then repeat with a low-pass filter time constant on the bN request of 50 msec. Increase or decrease this value to find the stable limit.                         (5 shots)

| Shot | tLPF (msec) | bN,req | Comment |
| --- | --- | --- | --- |

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

XMP-65 should have been completed before this XP is run.

All profile diagnostics are required: MSE, CHERS, MPTS, Bolometry.

6 MW injection must be available, but RF & CHI are not required.

# 5.	Planned analysis

Equilibrium reconstruction with EFIT and LRDFIT. Likely TRANSP runs for interesting cases. Ideal stability calculations for low-n global MHD may also be useful. Devoted pedestal analysis my also be run if the “EPH Path” is followed, as these shots would be interesting beyond simply global-MHD.

# 6.	Planned publication of results

Successful results (i.e. bN control leading to improved performance) would make a nice Nuclear Fusion paper.  Would get shown by multiple people at IAEA and APS. Also contributes strongly to the FY-10 MHD milestone on disruptivity reduction.

PHYSICS OPERATIONS REQUEST

| TITLE: b-Control for Disruptivity Reduction | No. OP-XP-1019 |
| --- | --- |
| AUTHORS: Gerhardt, et al. | DATE: |

Brief description of the most important operational plasma conditions required: 1: The beta-control algorithm must have been successfully tested, as demonstrated by successful completion of XMP-65. 2: 6 MW of NBI power must be available. 3: Profile diagnostics and RWM control must all be available.

Previous shot(s) which can be repeated: This XP will likely use the 2009 high-kappa fiducial shape, though with different vales of IP and (maybe) BT.

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): 0.4-0.45 T Flattop start/stop (s): To the I2t limit of the coil. IP (MA): 700-800 kA Flattop start/stop (s): Longest consistent with getting fdia Configuration: DN (maybe slightly biased down) Equilibrium Control: Isoflux (rtEFIT) Outer gap (m): 10-15 cm Inner gap (m): Z position (m): Elongation: Triangularity (U/L): OSP radius (m): Gas Species: Injector(s): NBI Species: D Voltage (kV) A: 90 B: 90 C: 90 Duration (s): ~1.3 ICRF Power (MW): 0 Phase between straps (°): N. A. Duration (s): 0 CHI: Off Bank capacitance (mF): LITERs: On Total deposition rate (mg/min): TBD, likely 200 mg/shot LLD: Probably no. Temperature (°C): EFC coils: On Configuration: Odd

DIAGNOSTIC CHECKLIST

| TITLE: b-Control for Disruptivity Reduction | No. OP-XP-1019 |
| --- | --- |
| AUTHORS: Gerhardt, et al. | DATE: |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Beam Emission Spectroscopy |  |  |
| Bolometer – divertor |  |  |
| Bolometer – midplane array |  |  |
| CHERS – poloidal |  |  |
| CHERS – toroidal |  |  |
| Dust detector |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  |  |
| Edge pressure gauges |  |  |
| Edge rotation diagnostic |  |  |
| Fast cameras – divertor/LLD |  |  |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes |  |  |
| FIReTIP |  |  |
| Gas puff imaging – divertor |  |  |
| Gas puff imaging – midplane |  |  |
| H camera - 1D |  |  |
| High-k scattering |  |  |
| Infrared cameras |  |  |
| Interferometer - 1 mm |  |  |
| Langmuir probes – divertor |  |  |
| Langmuir probes – LLD |  |  |
| Langmuir probes – bias tile |  |  |
| Langmuir probes – RF ant. |  |  |
| Magnetics – B coils | √ |  |
| Magnetics – Diamagnetism |  |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes |  |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents |  |  |
| Magnetics – RWM sensors |  |  |
| Mirnov coils – high f. |  |  |
| Mirnov coils – poloidal array |  |  |
| Mirnov coils – toroidal array |  |  |
| Mirnov coils – 3-axis proto. |  |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE |  |  |
| NPA – E¦¦B scanning |  |  |
| NPA – solid state |  |  |
| Neutron detectors |  |  |
| Plasma TV |  |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL |  |  |
| RF edge probes |  |  |
| Spectrometer – divertor |  |  |
| Spectrometer – SPRED |  |  |
| Spectrometer – VIPS |  |  |
| Spectrometer – LOWEUS |  |  |
| Spectrometer – XEUS |  |  |
| SWIFT – 2D flow |  |  |
| Thomson scattering |  |  |
| Ultrasoft X-ray – pol. arrays |  |  |
| Ultrasoft X-rays – bicolor |  |  |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. |  |  |
| X-ray crystal spectrom. - H |  |  |
| X-ray crystal spectrom. - V |  |  |
| X-ray tang. pinhole camera |  |  |