| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: HHFW Heating of Low Te(0), Ip Plasmas |  |  |  |
| OP-XP-1009 | Revision: 1 | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: G. Taylor & D. Mueller |  |  | Date |
| ATI – ET Group Leader: G. Taylor/D. Mueller |  |  | Date |
| RLM - Run Coordinator: E. Fredrickson |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| RESTRICTIONS or MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: HHFW Heating of Low Te(0), Ip Plasmas | No. OP-XP-1009 |
| --- | --- |
| AUTHORS: G. Taylor, D. Mueller, J.C. Hosea, S. Gerhardt, C. Kessel, B.P. LeBlanc, C.K. Phillips, S. Zweben, R. Raman, P.M. Ryan, R. Maingi | DATE: January 28, 2010 |

# 1.	Overview of planned experiment

This experiment tests HHFW heating of low Ip plasmas as a precursor to studying HHFW-assisted Ip ramp-up in the Solenoid-Free Start-up Topical Science Group. The experiment will use the new HHFW double-feed antenna and the increased lithium conditioning coverage available in 2010 to improve on previous HHFW electron heating results obtained with low Ip and Te(0) plasmas. The goal of the experiment is to couple  ~ 3 MW of -14 + -18 m-1 (180o) antenna phasing) and kf = ±8 m-1 (±90o antenna phasing)  HHFW power into a deuterium plasma with Ip ≤ 200 kA. Although the primary goal is to study heating of low current plasmas, a secondary goal is to look for current drive. This is a revised version of XP-920, which was approved but not run in 2009. This experiment contributes to the NSTX research milestone R10-2.

# 2.	Theoretical/ empirical justification

A major goal of the 30 MHz HHFW program on NSTX is to generate bootstrap current overdrive during non-inductive Ip ramp-up. TSC Ip ramp-up simulations predict that coupling 6 MW of HHFW power into the Ip ramp-up can result in bootstrap current overdrive. 85% bootstrap current fraction was achieved in XP-521 by coupling 2.5 MW of -14 + -18 m-1 (180o) HHFW power into an Ip = 250 kA deuterium plasma. The goal of this experiment is to couple HHFW power into plasmas with Ip ≤ 200 kA. HHFW-assisted Ip ramp-up experiments have recently benefited from lithium wall conditioning, which reduces the edge density, improving HHFW heating efficiency. In 2008 XP-817 successfully coupled HHFW power, with kf = - 8 m-1 current drive phasing, into lithium-conditioned deuterium plasmas. 550 kW of RF power coupled after the initiation of a CHI start-up plasma increased Te(0) from 3 to 15 eV when 
ne(0) ~ 4 x 1018m-3 and Ip ~ 100 kA. Also, 1.1 MW was coupled into the early phase of an ohmically-heated Ip ramp-up, when Ip was ramping from 100 kA to 300 kA, increasing Te(0) from 140 to 700 eV, when ne(0) ~ 6-9 x1018m-3. HHFW low Ip experiments in 2010 will benefit from the recent HHFW antenna upgrade from single-end fed straps to double-end fed straps, which should provide higher power than was previously achieved, as well as allowing coupling of HHFW power during a wider range of plasma edge conditions.

# 3.	Experimental run plan

This experiment is expected to take about 1.5-2 run days to complete. Considerable effort may be needed to develop a discharge with Ip ≤ 200 kA.

Normally the rtEFIT isoflux control is set to operate when Ip ≥ 250 kA. For this experiment the rtEFIT isoflux control will be set for Ip ≥ 150 kA. The experiment requires clamping the OH coil current, instead of feeding back on Ip. Shots without HHFW power will therefore have decaying Ip and when RF power is applied Ip will be sustained or decay more slowly depending on the amount of RF current drive and bootstrap current resulting from RF heating. It may be necessary to run some ohmic discharges with Ip feedback on to gain an estimate of the OH current needed to minimize plasma motion at low Ip.

The run plan is as follows:

1. Setup a 600 ms Ip flattop plasma, similar to RF conditioning shot 135260 (Ip = 650 kA, BT = 5.5 kG, helium), but run in deuterium and initially operate at Ip = 500 kA. Add kf = 14 + 18 m-1 (180o) HHFW power, coupled from 150 – 500 ms, with a 50 ms ramp-up in power at the start of the RF pulse. Increase RF power to 3 MW, while adjusting lithium evaporation rate, gas injection rate and outer gap to optimize HHFW heating efficiency. (5-10 shots)

2. Run a sequence of shots, reducing Ip in 100 kA increments from 500 kA to 300 kA, while coupling 
3 MW of kf = 14 + 18 m-1 (180o) HHFW power. Adjust lithium evaporation rate, gas injection rate and outer gap to optimize HHFW heating efficiency. Take no RF shot at the lowest Ip achieved with good RF coupling (10 shots)

3. Repeat (1) and (2) with -8 m-1 (-90o) heating. (10 shots)

4. Reduce Ip from 300 kA to ≤ 200 kA in 25-50 kA steps, while maintaining plasma position control so that the outer gap is 5-10 cm. (5-10 shots)

5. At the lowest Ip achieved with good plasma control, add kf = 14 + 18 m-1 (180o) power, coupled from 150 – 500 ms, with a 50 ms ramp-up in power at the start of the RF pulse. Increase RF power to ~ 3 MW. (5 shots)

6. Repeat (5) with -8 m-1 (-90o) heating. (5 shots)

7. Repeat (5) with kf = +8 m-1 (+90o) heating. (5 shots)

8. Perform a density scan with kf = -8 m-1 (-90o) heating. (5-10 shots)

9.  Adjust RF pulse to start as soon as Ip reaches the flattop value.  Then use open loop OH programming to provide no ohmic drive after plasma current reaches the minimum value, (< 200 kA at approximately 25 ms).  (5 shots to get start time, 5-10 shots to perform CD with ohmic current flat)

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

This experiment should follow the first two days of the HHFW plasma conditioning XMP, and requires PRF ~ 3 MW at 14 + 18 m-1and ±8 m-1. This experiment needs rtEFIT isoflux control for the outer gap. LITERs are required, but the LLD should be maintained in “cold”, solid lithium state. An NBI blip from source A at 90 keV should be added for MSE and CHERS data acquisition from 480 to 550 ms. In addition a 20 ms NBI blip from source A at 90 keV will be scanned through the RF pulse once good, reproducible, ~ 3 MW RF heating has been established. Thomson scattering data are required for core and edge electron heating data. For analysis of edge power loss and coupling efficiency the experiment also requires SOL reflectometry and edge ion heating data from edge rotation diagnostic.

# 5.	Planned analysis

Planned analysis includes analysis of heating efficiency at kf = ±8 m-1 and 14 + 18 m-1, TRANSP and GENRAY/CQL3D modeling.

# 6.	Planned publication of results

The results will be submitted for publication in Nuclear Fusion or Physics of Plasmas, and may contribute to an HHFW IAEA paper.

PHYSICS OPERATIONS REQUEST

| TITLE: HHFW Heating of Low Te(0), Ip Plasmas | No. OP-XP-1009 |
| --- | --- |
| AUTHORS: G. Taylor, D. Mueller, J.C. Hosea, S. Gerhardt, C. Kessel, B.P. LeBlanc, C.K. Phillips, S. Zweben, R. Raman, P.M. Ryan, R. Maingi | DATE: January 28, 2010 |

Brief description of the most important operational plasma conditions required: Ip as low as possible, preferable Ip < 200 kA. Stable, reproducible plasma with outer gap = 0.05 – 0.1 m. Request D. Mueller as operator.

Previous shot(s) which can be repeated: Previous shot(s) which can be modified: RF plasma conditioning shot 135260

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): 66 Flattop start/stop (s): 0/0.7 IP (MA): 0.15-0.5 Flattop start/stop (s): 0.08/0.6 Configuration: LSN Equilibrium Control: Outer gap / Isoflux (rtEFIT) / Strike-point control (rtEFIT) Outer gap (m): 0.05-0.1 Inner gap (m): Z position (m): 0.0 Elongation: Triangularity (U/L): OSP radius (m): Gas Species: D Injector(s): NBI Species: D Voltage (kV) A: 90 B: C: Duration (s): 480-550 ms, and 20 ms blip stepped between 250 to 450 ms from shot to shot ICRF Power (MW): 3 Phase between straps (°): ±90, 180 Duration (s): 0.35 CHI: Off Bank capacitance (mF): LITERs: On Total deposition rate (mg/min): 20 mg/min to start, adjust as needed LLD: Cold Temperature (°C): EFC coils: Off Configuration: Odd / Even / Other

DIAGNOSTIC CHECKLIST

| TITLE: HHFW Heating of Low Te(0), Ip Plasmas | No. OP-XP-1009 |
| --- | --- |
| AUTHORS: G. Taylor, D. Mueller, J.C. Hosea, S. Gerhardt, C. Kessel, B.P. LeBlanc, C.K. Phillips, S. Zweben, R. Raman, P.M. Ryan, R. Maingi | DATE: January 28, 2010 |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Beam Emission Spectroscopy |  |  |
| Bolometer – divertor | √ |  |
| Bolometer – midplane array | √ |  |
| CHERS – poloidal | √ |  |
| CHERS – toroidal | √ |  |
| Dust detector |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  | √ |
| Edge pressure gauges |  | √ |
| Edge rotation diagnostic | √ |  |
| Fast cameras – divertor/LLD |  |  |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes | √ |  |
| FIReTIP |  | √ |
| Gas puff imaging – divertor |  |  |
| Gas puff imaging – midplane |  | √ |
| H camera - 1D |  | √ |
| High-k scattering |  |  |
| Infrared cameras | √ |  |
| Interferometer - 1 mm |  | √ |
| Langmuir probes – divertor |  | √ |
| Langmuir probes – LLD |  |  |
| Langmuir probes – bias tile |  |  |
| Langmuir probes – RF ant. |  | √ |
| Magnetics – B coils | √ |  |
| Magnetics – Diamagnetism | √ |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents | √ |  |
| Magnetics – RWM sensors | √ |  |
| Mirnov coils – high f. | √ |  |
| Mirnov coils – poloidal array | √ |  |
| Mirnov coils – toroidal array | √ |  |
| Mirnov coils – 3-axis proto. | √ |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE | √ |  |
| NPA – E¦¦B scanning |  | √ |
| NPA – solid state |  | √ |
| Neutron detectors |  | √ |
| Plasma TV | √ |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL | √ |  |
| RF edge probes | √ |  |
| Spectrometer – divertor | √ |  |
| Spectrometer – SPRED | √ |  |
| Spectrometer – VIPS | √ |  |
| Spectrometer – LOWEUS |  |  |
| Spectrometer – XEUS |  | √ |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray – pol. arrays | √ |  |
| Ultrasoft X-rays – bicolor | √ |  |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. | √ |  |
| X-ray crystal spectrom. - H |  | √ |
| X-ray crystal spectrom. - V |  | √ |
| X-ray tang. pinhole camera |  | √ |