| Princeton Plasma Physics Laboratory NSTX Experimental Proposal |  |  |  |
| --- | --- | --- | --- |
| Title: Sustainment of HHFW-Driven 100% Non-Inductive H-Mode Plasma |  |  |  |
| OP-XP-1010 | Revision: 1 | Effective Date: (Approval date unless otherwise stipulated) Expiration Date: (2 yrs. unless otherwise stipulated) |  |
| PROPOSAL APPROVALS |  |  |  |
| Responsible Author: G. Taylor |  |  | Date |
| ATI – ET Group Leader: G. Taylor |  |  | Date |
| RLM - Run Coordinator: E. Fredrickson |  |  | Date |
| Responsible Division: Experimental Research Operations |  |  |  |
| RESTRICTIONS or MINOR MODIFICATIONS (Approved by Experimental Research Operations) |  |  |  |

NSTX EXPERIMENTAL PROPOSAL

| TITLE: Sustainment of HHFW-Driven 100% Non-Inductive H-Mode Plasma | No. OP-XP-1010 |
| --- | --- |
| AUTHORS: G. Taylor, D. Mueller, J.C. Hosea, S. Gerhardt, C. Kessel, B.P. LeBlanc, C.K. Phillips, S. Zweben, R. Maingi, P.M. Ryan, R. Maingi | DATE: January 28, 2010 |

# 1.	Overview of planned experiment

Use solenoid to ramp Ip to 300-400 kA, and then couple ~ 5 MW of HHFW heating to drive plasma into 
H-mode, generating ~ 100% of Ip via boostrap current. This experiment should be run after XP-1009, and only when ~ 5 MW of HHFW has been successfully coupled into deuterium plasma. This experiment contributes to the NSTX research milestone R10-2.

# 2.	Theoretical/ empirical justification

A goal of the NSTX program is to demonstrate 100% non-inductive H-mode plasmas. 85% bootstrap current fraction was achieved in XP-521 by coupling 2.5 MW of -14 + -18 m-1 (180o) HHFW power into an Ip = 250 kA deuterium plasma. It should be possible to achieve 100% bootstrap current fraction with 
~ 5 MW of RF coupled into an Ip ~ 400 kA discharge.

# 3.	Experimental run plan

This experiment should take 1 day to complete.

1. Setup 600 ms Ip flattop plasma, similar to 123712 (BT = 5.5 kG, deuterium), but with 
Ip = 300-400 kA. (5 shots)

2. Add kf = 14 + 18 m-1 -8 m-1 (180o) HHFW power, coupled from 150 – 500 ms, with a 50 ms ramp-up in power at the start of the RF pulse. Increase RF power to 5 MW, while adjusting lithium evaporation rate, gas injection rate and outer gap to optimize HHFW heating efficiency. (10 shots)

3. Repeat with kf = (-90o) heating (10 shots)

# 4.	Required machine, NBI, RF, CHI and diagnostic capabilities

This experiment should follow XP-1009, and requires PRF ~ 5 MW at -14 + -18 m-1and -8 m-1. This experiment needs rtEFIT isoflux control for the outer gap. LITERs are required, but the LLD can be maintained in “cold”, solid lithium state. An NBI blip from source A at 90 keV should be added for MSE and CHERS data acquisition from 480 to 550 ms. In addition a 20 ms NBI blip from source A at 90 keV will be scanned through the RF pulse once good, reproducible, ~ 5 MW RF heating has been established. Thomson scattering data are required for core and edge electron heating data. For analysis of edge power loss and coupling efficiency the experiment also requires SOL reflectometry and edge ion heating data from edge rotation diagnostic.

# 5.	Planned analysis

Planned analysis includes analysis of heating efficiency at kf = -8 m-1 and -14 + -18 m-1, TRANSP and GENRAY/CQL3D modeling.

# 6.	Planned publication of results

The results will be submitted for publication in Nuclear Fusion or Physics of Plasmas, and may contribute to an HHFW IAEA paper.

PHYSICS OPERATIONS REQUEST

| TITLE: Sustainment of HHFW-Driven 100% Non-Inductive H-Mode Plasma | No. OP-XP-1010 |
| --- | --- |
| AUTHORS: G. Taylor, D. Mueller, J.C. Hosea, S. Gerhardt, C. Kessel, B.P. LeBlanc, C.K. Phillips, S. Zweben, R. Maingi, P.M. Ryan, R. Maingi | DATE: January 28, 2010 |

Brief description of the most important operational plasma conditions required: Stable, reproducible Ip = 300-400 kA deuterium plasma with outer gap = 0.05 – 0.1 m. Request D. Mueller as operator.

Previous shot(s) which can be repeated: Previous shot(s) which can be modified: 123712

Machine conditions (specify ranges as appropriate, strike out inapplicable cases) ITF (kA): 66 Flattop start/stop (s): 0/0.7 IP (MA): 0.4 Flattop start/stop (s): 0.08/0.6 Configuration: LSN Equilibrium Control: Outer gap / Isoflux (rtEFIT) / Strike-point control (rtEFIT) Outer gap (m): 0.05-0.1 Inner gap (m): Z position (m): 0.0 Elongation: Triangularity (U/L): OSP radius (m): Gas Species: D Injector(s): NBI Species: D Voltage (kV) A: 90 B: C: Duration (s): 480-550 ms, and 20 ms blip stepped between 250 to 450 ms from shot to shot ICRF Power (MW): 5 Phase between straps (°): ±90, 180 Duration (s): 0.35 CHI: Off Bank capacitance (mF): LITERs: On Total deposition rate (mg/min): 20 mg/min to start, adjust as needed LLD: Cold Temperature (°C): EFC coils: Off Configuration: Odd / Even / Other

DIAGNOSTIC CHECKLIST

| TITLE: Sustainment of HHFW-Driven 100% Non-Inductive H-Mode Plasma | No. OP-XP-1010 |
| --- | --- |
| AUTHORS: G. Taylor, D. Mueller, J.C. Hosea, S. Gerhardt, C. Kessel, B.P. LeBlanc, C.K. Phillips, S. Zweben, R. Maingi, P.M. Ryan, R. Maingi | DATE: January 28, 2010 |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| Beam Emission Spectroscopy |  |  |
| Bolometer – divertor | √ |  |
| Bolometer – midplane array | √ |  |
| CHERS – poloidal | √ |  |
| CHERS – toroidal | √ |  |
| Dust detector |  |  |
| Edge deposition monitors |  |  |
| Edge neutral density diag. |  | √ |
| Edge pressure gauges |  | √ |
| Edge rotation diagnostic | √ |  |
| Fast cameras – divertor/LLD |  |  |
| Fast ion D_alpha - FIDA |  |  |
| Fast lost ion probes - IFLIP |  |  |
| Fast lost ion probes - SFLIP |  |  |
| Filterscopes | √ |  |
| FIReTIP |  | √ |
| Gas puff imaging – divertor |  |  |
| Gas puff imaging – midplane |  | √ |
| H camera - 1D |  | √ |
| High-k scattering |  |  |
| Infrared cameras | √ |  |
| Interferometer - 1 mm |  | √ |
| Langmuir probes – divertor |  | √ |
| Langmuir probes – LLD |  |  |
| Langmuir probes – bias tile |  |  |
| Langmuir probes – RF ant. |  | √ |
| Magnetics – B coils | √ |  |
| Magnetics – Diamagnetism | √ |  |
| Magnetics – Flux loops | √ |  |
| Magnetics – Locked modes | √ |  |
| Magnetics – Rogowski coils | √ |  |
| Magnetics – Halo currents | √ |  |
| Magnetics – RWM sensors | √ |  |
| Mirnov coils – high f. | √ |  |
| Mirnov coils – poloidal array | √ |  |
| Mirnov coils – toroidal array | √ |  |
| Mirnov coils – 3-axis proto. | √ |  |

Note special diagnostic requirements in Sec. 4

| Diagnostic | Need | Want |
| --- | --- | --- |
| MSE | √ |  |
| NPA – E¦¦B scanning |  | √ |
| NPA – solid state |  | √ |
| Neutron detectors |  | √ |
| Plasma TV | √ |  |
| Reflectometer – 65GHz |  |  |
| Reflectometer – correlation |  |  |
| Reflectometer – FM/CW |  |  |
| Reflectometer – fixed f |  |  |
| Reflectometer – SOL | √ |  |
| RF edge probes | √ |  |
| Spectrometer – divertor | √ |  |
| Spectrometer – SPRED | √ |  |
| Spectrometer – VIPS | √ |  |
| Spectrometer – LOWEUS |  |  |
| Spectrometer – XEUS |  | √ |
| SWIFT – 2D flow |  |  |
| Thomson scattering | √ |  |
| Ultrasoft X-ray – pol. arrays | √ |  |
| Ultrasoft X-rays – bicolor | √ |  |
| Ultrasoft X-rays – TG spectr. |  |  |
| Visible bremsstrahlung det. | √ |  |
| X-ray crystal spectrom. - H |  | √ |
| X-ray crystal spectrom. - V |  | √ |
| X-ray tang. pinhole camera |  | √ |